3GPP TSG-RAN WG4 Meeting #102-e	R4-2204014

21st February – 3rd March 2022

Source:	Qualcomm Incorporated

Title:	Calculation of MSD for V2X_n1A-47A and accompanying TP

Agenda item:	9.25.2

Work Item:	          NR_LTE_V2X_PC5_combos-Core

Document for:	Approval

# 1.	Introduction

In RAN#94e a new inter-band V2X combination V2X_n1A-47A was introduced [1]. In this paper we present the MSDs required for band 47 for this V2X band combination.

# 2. 	Discussion

In [1] the inter-band combination V2X_n1A-47A was introduced. For this combination the 3rd harmonic of band 1 can fall into band 47 causing degradation in sensitivity. The 3rd harmonic of band 1 can couple into the 47 receiver through the PCB or can be conducted from the band 1 antenna to the 47 antenna. The below table details the MSD calculation for these V2X combinations.

Table 1: TX to RX H3 budget

|  | B1/n1 PA-->CV2x 47 Rx |
| --- | --- |
| Bandwidth (MHz) | 10 |
| SCS (kHz) | 15 |
| 3GPP Sensitivity(dBm) | -90.4 |
| PA out B1/n1 H3 (dBm) | -7 |
| B1 Tx Filter Rejection+ FE filtering | -71 |
| PCB coupling to B1/n1 PRX/DRX LNA | -70 |
| Tx-Rx Antenna Isolation | -10 |
| 47 front-end loss (dB) | 4 |
| PRX/DRX B1/n1 H3 conducted into 47 RX (dBm) | -88 |
| PRX/DRX B1/n1 H3 coupled into 47 RX (dBm) | -73 |
| PRX/DRX total H3 (dBm) | -72.9 |
| PRX/DRX noise (dBm) | -86.4 |
| Dual sensitivity (dBm) | -73.8 |
| Desensitization (dB) | 16.6 |

The MSD values for the other SCS and BW combinations were calculated in a similar manner and based on these results the following MSD recommendations are made for each V2X combination and BW.

Table 2: Band 47 MSD for V2X_n1A-47A

| BW(MHz) | 10 | 20 |
| --- | --- | --- |
| MSD (dB) | 16.6 | 13.8 |

Proposal 1: Adopt the MSD’s given in Table2 for band 47 for the following V2X combinations: V2X_n1A-47A

# Conclusion

We present the analysis for MSD for inter-band combination V2X_n1A-47A and make the following proposals.

Proposal 1: Adopt the MSD’s given in Table2 for band 47 for the following V2X combination: V2X_n1A-47A

Table 2: Band 47 MSD for V2X_n1A-47A

| BW(MHz) | 10 | 20 |
| --- | --- | --- |
| MSD (dB) | 16.6 | 13.8 |

# Annex - TP for TR 37.875 for V2X_n1A-n47A, V2X_1A_n47A and V2X_n1A_47A

## --Start of changes

***********************************Start of the TP*************************************

# 1	Scope

The present document is the Technical Report on TR on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X band.

The purpose of the present document is to study the extension of the band combinations for V2X service to grow the NR V2X ecosystem. Operators propose new band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X band. Whether to specify con-current operation of LTE/NR CA/DC band combinations and V2X band depends on requests in Rel-17. Specifically, the self-desensitization problem of con-current operation band combinations will be analysed including harmonics, IMD problem, etc.. Also the candidate solutions will be studied to solve the self-desensitization problem.

Table 1-1: Release 17 NR V2X band combinations

| V2X Band combination | REL independent from |
| --- | --- |
| V2X_n1-n47 | Rel-16 |
| V2X_n8-n47 | Rel-16 |
| V2X_n39-n47 | Rel-16 |
| V2X_n40-n47 | Rel-16 |
| V2X_n41-n47 | Rel-16 |
| V2X_n78-n47 | Rel-16 |
| V2X_n79-n47 | Rel-16 |
| V2X_n1-n47 | Rel-16 |
| V2X_n8-n47 | Rel-16 |
| V2X_n39_47 | Rel-16 |
| V2X_n40_47 | Rel-16 |
| V2X_n41_47 | Rel-16 |
| V2X_n78_47 | Rel-16 |
| V2X_n79_47 | Rel-16 |
| V2X_1_n47 | Rel-16 |
| V2X_8_n47 | Rel-16 |
| V2X_3_n47 | Rel-16 |
| V2X_39_n47 | Rel-16 |
| V2X_40_n47 | Rel-16 |
| V2X_41_n47 | Rel-16 |

Note: All band combinations in table 1-1 that are release independent from Rel-16 are optional

# 6	Con-current operation with one Uu band and one PC5 band

## 6.1	Con-current operation between one LTE Uu band and one NR PC5 band

<<Unchanged section omitted>>

### 6.1.6	V2X_1A_n47A

#### 6.1.6.1	Operating bands for V2X_1A_n47A

The operating bands for V2X_1A_n47A are specified in table 6.1.6.1-1.

Table 6.1.6.1-1: Inter-band con-current V2X operating bands for V2X_1A_n47A

| V2X con-current configuration | E-UTRA / NR Operating Band | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_1A_n47A | 1 | Uu | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

#### 6.1.6.2	Channel bandwidths per operating band for V2X_1A_n47A

The channel bandwidths per operating band for V2X_1A_n47A are specified in table 6.1.6.2-1.

Table 6.1.6.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_1A_n47A

| V2X inter-band Configuration | E-UTRA / NR operating Band | SCS kHz | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_1A_n47A | 1 | 15 |  |  | Yes | Yes | Yes | Yes |  |  |  |  | 60 | 0 |
|  | n47 | 15 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |
|  |  | 60 |  |  |  | Yes |  | Yes |  | Yes | Yes |  |  |  |

#### 6.1.6.3	UE co-existence studies

The UE co-existence studies specified for V2X_n1A-n47A in clause 6.2.6.3 are applicable to V2X_1A_n47A since band 1 and band n1 have the same frequency range.

#### 6.1.6.4	MSD, ∆TIB and ∆RIB values

#### 6.1.6.5	REFSENS requirements

## 6.2	Con-current operation between one NR Uu band and one NR PC5 band

<<Unchanged section omitted>>

### 6.2.7	V2X_n1A-n47A

#### 6.2.7.1	Operating bands for V2X_n1A-n47A

The operating bands for V2X_n1A-n47A are specified in table 6.2.7.1-1.

Table 6.2.7.1-1: Inter-band con-current V2X operating bands for V2X_n1A-n47A

| V2X con-current configuration | E-UTRA / NR Operating Band | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_n1A_n47A | n1 | Uu | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

#### 6.2.7.2	Channel bandwidths per operating band for V2X_n1A-n47A

The channel bandwidths per operating band for V2X_n1A-n47A are specified in table 6.2.7.2-1.

Table 6.2.7.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_n1A-n47A

| V2X inter-band Configuration | NR operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n1A-n47A | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 90 | 0 |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |

#### 6.2.7.3	Coexistence studies

The harmonics analysis for V2X_n1A-n47A is specified in table 6.2.7.3-1. Up to 4th harmonics of band n1 are provided since the frequency range of the 5th harmonics is much higher than 5.9GHz. The harmonics of band n47 are not listed as the harmonics distributed in the frequency range much higher than 5.9GHz have no impact on GNSS and ISM bands. Based on the harmonics analysis, it is observed that the 3rd harmonic of band n1 falls on band n47.

Table 6.2.7.3-1: Harmonics analysis for V2X_n1A-n47A

| Operating Band | Band n1 |  | Band n47 |  |
| --- | --- | --- | --- | --- |
| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| UL frequency (MHz) | 1920 | 1980 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | No effect |  |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | Has impact |  |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 |  |  |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | No effect |  |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 |  |  |

The IMD analysis for V2X_n1A-n47A is specified in table 6.2.7.3-2. Up to the 5th order IMDs of band n1 and band n47 are provided. Based on the IMD analysis, it is observed that no IMD products fall into the associated bands.

| Operating Band | Band n1 |  | Band n47 |  |
| --- | --- | --- | --- | --- |
| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| UL frequency (MHz) | 1920 | 1980 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3860 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 17565 | 17775 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3875 | 4005 | 7775 | 7905 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2085 | 1895 | 9730 | 9930 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9695 | 9885 | 13630 | 13830 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 165 | 85 | 15585 | 15855 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11615 | 11865 | 19485 | 19755 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 8010 | 7750 | 7750 | 8010 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 21780 | 21440 | 2065 | 1755 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 25340 | 25680 | 13535 | 13845 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 13935 | 13605 | 5770 | 6090 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 21615 | 21525 | 17650 | 17610 |

Table 6.2.7.3-2: IMD analysis for V2X_n1A-n47A

The harmonics and intermodulation products should be evaluated when V2X inter-band con-current operating UE coexists with other systems such as GNSS and ISM. The harmonics and IMD analysis of V2X_n1A-n47A for GNSS and ISM bands is shown in table 6.2.7.3-3. Based on the analysis for GNSS and ISM bands, band n47 and 6th harmonics have an impact on the ISM band (5GHz).

Table 6.2.7.3-3: Harmonic and IMDs analysis of V2X_n1A-n47A UE for GNSS and ISM bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd harmonic n1, Band n47 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd harmonic n1 |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonic n1 |
|  | 5150 | - | 5825 | Yes | Asia | 3rd harmonic n1 |

#### 6.2.7.4	MSD, ∆TIB and ∆RIB values

#### 6.2.7.5	REFSENS requirements

## 6.3	Con-current operation between one NR Uu band and one LTE PC5 band

<<Unchanged section omitted>>

### 6.3.7	V2X_n1A_47A

#### 6.3.7.1	Operating bands for V2X_n1A_47A

The operating bands for V2X_n1A_47A are specified in table 6.3.7.1-1.

Table 6.3.7.1-1: Inter-band con-current V2X operating bands for V2X_n1A_47A

| V2X con-current configuration | E-UTRA / NR Operating Band | Interface | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_n1A_47A | n1 | Uu | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |

#### 6.3.7.2	Channel bandwidths per operating band for V2X_n1A_47A

The channel bandwidths per operating band for V2X_n1A_47A are specified in table 6.3.7.2-1.

Table 6.3.7.2-1: V2X inter-band con-current configurations and bandwidth combination sets for V2X_n1A_47A

| V2X inter-band Configuration | E-UTRA / NR operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n1A_47A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes | 70 | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
|  | 47 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |

#### 6.3.7.3	UE co-existence studies

The UE co-existence studies specified for V2X_n1A-n47A in clause 6.2.6.3 are applicable to V2X_n1A_47A since band 47 and band n47 have the same frequency range.

#### 6.3.7.4	MSD, ∆TIB and ∆RIB values

## MSD test configurations are specified below for V2X_n1A-47A. The MSD values given in tables 6.3.7.4-1. The MSD values given below shall apply to all n1 and 47 BW and SCS combinations.

Table 6.3.7.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 47 | 15 | 50 | 10 | 50 | 10 | 16.6 |
| n1 | 47 | 15 | 50 | 10 | 98 | 20 | 13.8 |

#### 6.3.7.5	REFSENS requirements

--End of changes

# References

[1] R4-2105367, CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A, RAN4#98-bis, Apr. 12-20 2021

[2] R4-2105368, CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A, RAN4#98-bis, Apr. 12-20 2021

[3] R4-2107815, Discussion and TP for TR 38.375 on SMD for V2X_n79A-n47A and V2X_n79A_47A, RAN#99-e, May 19-28 2021

[4] R4-2113420, Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A-47A, RAN4#100-e, August 16 -27 , 2021

[5] R4-2112285, Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP, RAN4#100-e, August 16 -27 , 2021