3GPP TSG-RAN WG4 Meeting #102-e 		R4-2204028

Electronic meeting, 21st Febuary – 3rd March, 2022

Source:	Ericsson

Title:	Discussion on SAN PUSCH demodulation requirements

Agenda item:	10.13.6.2.1

Document for:	discussion

# Introduction

In RAN4#101-bis-e meeting, discussion on NTN demodulation requirements were kicked off. Companies delivered many constructive thinking and proposals. Several agreements were achieved by the end of meeting [1] [2].

Agreements on SAN PUSCH demodulation requirements

- For SAN, RAN4 to define the new demodulation requirements. The test parameters need analysis case by case.

- Do not specify K_offset and K_mac parameters for SAN performance requirements definition

- Do not define satellite performance requirements with HARQ Processes

- Reuse the synchronization method from legacy BS testing

- Consider both 15kHz and 30kHz SCS

- Introduce QPSK for PUSCH requirements

- Define the following requirements and further discuss on Tx/Rx configurations

  - Transform precoding disabled

  - Transform precoding enabled

  - UL timing adjustment

  - repetition Type A

Following WF are agreed in the last meeting.

PUSCH requirements

- Further discuss whether to define the following requirements

  - Option 1: 2-step RA type requirement

  - Option 2: Mapping Type B with non-slot transmission requirements

SCS/CBW set for PUSCH requirements

- FFS on CBW and test metric

  - Option 1: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

  - Option 2: FFS based on the link budget

Modulation order for PUSCH requirements

- FFS whether to introduce 16QAM and 64QAM for PUSCH requirements

Companies are encouraged to further check if it is feasible for requirement

In this contribution, we deliver our view on remaining open issues for SAN PUSCH demodulation requirements.

# Discussion

## 2.1	 General

According to the Table 2-1 in [3], UL CNR for 0.4MHz carrier is normally less than 2.8dB for LEO and even less than -10dB for GEO. the UL CNR could reach to ~6dB for LEO600 deployment. It is still hard to scheduling 16QAM anyway regarding to possible 1Rx configuration.

For QPSK modulation, MCS 2 is used for Rel-15 normal PUSCH requirement, but higher MCS (i.e., MCS4) could be considered for NTN, especially for LOS channel.

Full bandwidth allocation could be feasible if only QPSK is applied. For 15kHz SCS, requirements for 5M/20MHz could be introduced. For 30kHz SCS, requirements for 10M/20MHz could be introduced. Similar applicability rule for different bandwidth in Rel-16 could be reused for NTN requirement if simulation results show no difference between bandwidths. Furthermore, LOS channel could only consider the largest bandwidth.

Proposal 1: Use MCS2 for NLOS channel and consider MCS4 for LOS channel if simulation results can fulfil test metric.

Proposal 2: NTN SAN PUSCH demodulation requirements could be defined as follows:

- NLOS channel: 15kHz SCS 5M/20MHz and 30kHz SCS 10M/20MHz

- LOS channel: 15kHz SCS 20MHz and 30kHz SCS 20MHz

## 2.2	 Normal PUSCH performance

For normal PUSCH performance, 1 DM-RS could be prioritized considering UE pre-compensation. Both mapping type A and B could be considered to keep the test coverage. Test metric uses 70% throughput. Channel model could start from NTN-TDLB100-300 and NTN-TDLD100-300 which assume 100ns delay spread and 0.15ppm residual Doppler shift as start point.

Consider following parameters for normal NTN PUSCH performance.

Table 2.2-1: Test parameters for NTN PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | NTN-TDLB100-300 (NLOS) NTN-TDLD100-300 (LOS) |
| MCS |  | MCS2 for NLOS MCS4 for LOS |
| Transform precoding |  | Disabled/Enabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

Proposal 3: Take simulation assumptions in Table 2.2-1 for NTN PUSCH demodulation requirements discussion.

## 2.3	UL timing adjustment

For NTN UL timing adjustment requirement, only NLOS channel could be considered since LOS channel is quite similar as AWGN. Both mapping type A and B could be considered to keep the test coverage. Use same SCS and bandwidth combination as PUSCH requirement. UE speed could consider normal speed as 120km/h. Other configurations could reuse Rel-15 requirements.

Consider following parameters for NTN UL TA performance.

Table 2.3-1 Test parameters for NTN UL timing adjustment

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 2 |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD (Note1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 5 MHz CBW/15kHz SCS: 12 RB for each UE 20MHz CBW/15kHz SCS: 50 RB for each UE 10MHz CBW/30kHz SCS: 12 RB for each UE 20MHz CBW/30kHz SCS: 25 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 for 5MHz CBW/15kHz SCS, 50 for 20 MHz CBW/15kHz SCS, 12 for 10MHz CBW/30kHz SCS and 25 for 20 MHz CBW/30kHz SCS |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note2) | For FDD: slot #1 in radio frames For TDD: last symbol in slot #3 in radio frames for 15kHz last symbol in slot #7 in radio frames for 30kHz |
|  | SRS resource allocation | 15 kHz SCS: CSRS =5, BSRS =0, for 20 RB CSRS = 21, BSRS =0, for 80 RB 30 kHz SCS: CSRS =5, BSRS =0, for 20 RB CSRS = 11, BSRS =0, for 40 RB |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

Table 2.3-2: Parameters for NTN UL timing adjustment

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLB100-300 |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

Proposal 4: Take simulation assumptions in Table 2.3-1 and 2.3-2 for NTN UL TA demodulation requirements discussion.

## 2.4	PUSCH repetition type A

For NTN PUSCH repetition A requirement, only NLOS scenario should be considered. Both mapping type A and B should be considered to keep the test coverage. Code rate could also follow Rel-16 requirement to use 0.99. Bandwidth could follow normal PUSCH requirement assumptions.

Consider following parameters for NTN PUSCH repetition type A.

Table 2.4-1: Test parameters for NTN PUSCH repetition Type A

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 5 in Table 3 |
| Channel model |  | NTN-TDLB100-300 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | 30 kHz SCS: n2 15 kHz SCS: n2 for FDD and n8 for TDD [Note 3] |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth (15kHz: 5MHz/10MHz; 30kHz: 10MHz/20MHz) |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. Note 3: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary. |  |  |

Proposal 5: Take simulation assumptions in Table 2.4-1 for NTN PUSCH repetition type A demodulation requirements discussion.

## 2.5	2-step RA Msg A

2-step RA could be useful for reducing initial access time due to long RTT in NTN deployment, especially NTN UE could pre-compensate most of timing error based on its GNSS position and satellite ephemeris. NLOS channel model could be used for requirement and MCS could use MCS2 as the start point, and Rel-16 assumption MCS1 can also be considered if MCS2 is not feasible. DM-RS configuration could only consider 1+1 DM-RS regarding NTN UE pre-compensation capability. Timing offset could be further discussed based on NTN UE timing adjustment character.

Consider following parameters for NTN MsgA PUSCH for 2-step RA.

Table 2.5-1: Test parameters for NTN msgA PUSCH for 2-step RA type

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15 kHz SCS: 10 MHz |
|  |  | 30 kHz SCS: 20 MHz |
| MCS |  | 2 or 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID = 0 |
| Time domain resource assignment | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 2 PRBs |
|  | Starting PRB index | 0 |
|  | Frequency hopping | Disabled |
| Time offset (TO) Cycling (µs) | start:step:end | 15k SCS: FFS |
|  |  | 30k SCS: FFS |
| Test Metric | BLER | 0.01 |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. Note 2: For FR1, either pos 1 or pos 2 may be used for the test FRC. A pass with either of these possibilities is sufficient to demonstrate compliance to the core requirement. Note 3: The power ratio between preamble and msgA (msgA-DeltaPreamble) is set to be sufficient to achieve 100% preamble detection. The SNR for the requirement is defined on the msgA PUSCH. |  |  |

Proposal 6: Define NTN SAN demodulation requirements for 2-step RA PUSCH.

Proposal 7: Take simulation assumptions in Table 2.5-1 for NTN MsgA PUSCH for 2-step RA type demodulation requirements discussion.

## 2.6	Mapping Type B with non-slot transmission

Mapping Type B with non-slot transmission is defined for URLLC scenario which is not typical in NTN scenarios due to long delay within the whole link. it is no necessary to define demodulation requirements.

Proposal 8: Do not define requirements for Mapping Type B with non-slot transmission.

# Conclusions

Proposal 1: Use MCS2 for NLOS channel and consider MCS4 for LOS channel if simulation results can fulfil test metric.

Proposal 2: NTN SAN PUSCH demodulation requirements could be defined as follows:

- NLOS channel: 15kHz SCS 5M/20MHz and 30kHz SCS 10M/20MHz

- LOS channel: 15kHz SCS 20MHz and 30kHz SCS 20MHz

Proposal 3: Take simulation assumptions in Table 2.2-1 for NTN PUSCH demodulation requirements discussion.

Table 2.2-1: Test parameters for NTN PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | NTN-TDLB100-300 (NLOS) NTN-TDLD100-300 (LOS) |
| MCS |  | MCS2 for NLOS MCS4 for LOS |
| Transform precoding |  | Disabled/Enabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

Proposal 4: Take simulation assumptions in Table 2.3-1 and 2.3-2 for NTN UL TA demodulation requirements discussion.

Table 2.3-1 Test parameters for NTN UL timing adjustment

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 2 |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD (Note1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 5 MHz CBW/15kHz SCS: 12 RB for each UE 20MHz CBW/15kHz SCS: 50 RB for each UE 10MHz CBW/30kHz SCS: 12 RB for each UE 20MHz CBW/30kHz SCS: 25 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 for 5MHz CBW/15kHz SCS, 50 for 20 MHz CBW/15kHz SCS, 12 for 10MHz CBW/30kHz SCS and 25 for 20 MHz CBW/30kHz SCS |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note2) | For FDD: slot #1 in radio frames For TDD: last symbol in slot #3 in radio frames for 15kHz last symbol in slot #7 in radio frames for 30kHz |
|  | SRS resource allocation | 15 kHz SCS: CSRS =5, BSRS =0, for 20 RB CSRS = 21, BSRS =0, for 80 RB 30 kHz SCS: CSRS =5, BSRS =0, for 20 RB CSRS = 11, BSRS =0, for 40 RB |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

Table 2.3-2: Parameters for NTN UL timing adjustment

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLB100-300 |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

Proposal 5: Take simulation assumptions in Table 2.4-1 for NTN PUSCH repetition type A demodulation requirements discussion.

Table 2.4-1: Test parameters for NTN PUSCH repetition Type A

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 5 in Table 3 |
| Channel model |  | NTN-TDLB100-300 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | 30 kHz SCS: n2 15 kHz SCS: n2 for FDD and n8 for TDD [Note 3] |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth (15kHz: 5MHz/10MHz; 30kHz: 10MHz/20MHz) |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. Note 3: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary. |  |  |

Proposal 6: Define NTN SAN demodulation requirements for 2-step RA PUSCH.

Proposal 7: Take simulation assumptions in Table 2.5-1 for NTN MsgA PUSCH for 2-step RA type demodulation requirements discussion.

Table 2.5-1: Test parameters for NTN msgA PUSCH for 2-step RA type

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15 kHz SCS: 10 MHz |
|  |  | 30 kHz SCS: 20 MHz |
| MCS |  | 2 or 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID = 0 |
| Time domain resource assignment | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 2 PRBs |
|  | Starting PRB index | 0 |
|  | Frequency hopping | Disabled |
| Time offset (TO) Cycling (µs) | start:step:end | 15k SCS: FFS |
|  |  | 30k SCS: FFS |
| Test Metric | BLER | 0.01 |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. Note 2: For FR1, either pos 1 or pos 2 may be used for the test FRC. A pass with either of these possibilities is sufficient to demonstrate compliance to the core requirement. Note 3: The power ratio between preamble and msgA (msgA-DeltaPreamble) is set to be sufficient to achieve 100% preamble detection. The SNR for the requirement is defined on the msgA PUSCH. |  |  |

Proposal 8: Do not define requirements for Mapping Type B with non-slot transmission.

# References

[1]	TR38.811 v15.4.0, Study on New Radio (NR) to support non-terrestrial networks

[2]	TR38.821 v16.1.0, Solutions for NR to support non-terrestrial networks (NTN)

[3]	R4-2204027, Discussion on general issue for NTN demodulation requirements, Ericsson