3GPP TSG-RAN WG4 Meeting #102-e 		R4-2204029

Electronic meeting, 21st Febuary – 3rd March, 2022

Source:	Ericsson

Title:	Discussion on SAN PUCCH demodulation requirements

Agenda item:	10.13.6.2.2

Document for:	discussion

# Introduction

In RAN4#101-bis-e meeting, discussion on NTN demodulation requirements were kicked off. Companies delivered many constructive thinking and proposals. Several agreements were achieved by the end of meeting [1] [2].

Following WF are agreed in the last meeting.

PUCCH requirements

- Define the requirements for PUCCH formant 0/1/2/3/4

  - The Rx configuration is FFS

- FFS whether to define Multi-slot PUCCH format 1, 2 Rx requirement

SCS/CBW set for PUCCH requirements

- Option 1: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz

- Option 2: FFS based on the link budget

In this contribution, we deliver our view on remaining open issues for SAN PUCCH demodulation requirements.

# Discussion

Regarding the long RTT for NTN deployment, the UCI with CSI report might not be useful for SAN scheduling. Network could estimate receiving SNR at UE side, so only HARQ is still worthy to be transferred by UCI.

Proposal 1: Prioritize UCI with HARQ on PUCCH demodulation requirement.

In our contribution for general issue in NTN demodulation [3], the typical antenna configurations on satellite are discussed. PUCCH could follow it and use 1Rx and 2Rx..

Proposal 2: Define NTN PUCCH demodulation requirement with 1Tx 1Rx and 1Tx 2Rx.

Proposal 3: NTN SAN PUCCH demodulation requirements could be defined for 15kHz SCS 5M/20MHz bandwidth and 30kHz SCS 10M/20MHz bandwidth.

- Normal PUCCH performance

For normal PUCCH performance requirements, most of Rel-15 simulation assumptions could be reused. Channel model could only consider NLOS model, such as NTN-TDLB100-300. Test cases could be down selected depend on the typical scenarios.

For format 0, maybe only 1 symbol case is enough.

Table 2-1: Test Parameters for NTN PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ ACK miss <1% |

Table 2-2: Test Parameters for NTN PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@NACK ACK<0.1% SNR@ ACK miss <1% |

Table 2-3: Test Parameters for NTN PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ ACK miss <1% |

Table 2-4: Test Parameters for NTN PUCCH format 3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ BLER <1% |

Table 2-5: Test parameters for NTN PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ BLER <1% |

Proposal 4: Take simulation parameters in Table 2-1 to 2-5 for NTN PUCCH demodulation requirement discussion.

- Multi-slot PUCCH performance

Multi-slot PUCCH could be usful for NTN scenario coverage due to very large path loss. The simulation assumptions and test metric could take Rel-16 requirement as reference. Format 1 might be the most important one to check the HARQ performance with more bits which could be only considered for the requirement. The SCS and bandwidth configuration could only consider 15kHz with 20MHz since 15kHz SCS might be more typical and only 1 bandwidth could be enough for the requirement.

Table 2-6: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 20MHz |
| Test metric | SNR@ NACK ACK <0.1% SNR@ ACK miss <1% |

Proposal 5: Define NTN multi-slot PUCCH demodulation requirements which could take assumptions in Table 2-6 as the start point.

# Conclusions

Proposal 1: Prioritize UCI with HARQ on PUCCH demodulation requirement.

Proposal 2: Define NTN PUCCH demodulation requirement with 1Tx 1Rx and 1Tx 2Rx.

Proposal 3: NTN SAN PUCCH demodulation requirements could be defined for 15kHz SCS 5M/20MHz bandwidth and 30kHz SCS 10M/20MHz bandwidth.

Proposal 4: Take simulation parameters in Table 2-1 to 2-5 for NTN PUCCH demodulation requirement discussion.

Table 2-1: Test Parameters for NTN PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ ACK miss <1% |

Table 2-2: Test Parameters for NTN PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@NACK ACK<0.1% SNR@ ACK miss <1% |

Table 2-3: Test Parameters for NTN PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ ACK miss <1% |

Table 2-4: Test Parameters for NTN PUCCH format 3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ BLER <1% |

Table 2-5: Test parameters for NTN PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 5MHz/20MHz 30kHz: 10MHz/20MHz |
| Test metric | SNR@ BLER <1% |

Proposal 5: Define NTN multi-slot PUCCH demodulation requirements which could take assumptions in Table 2-6 as the start point.

Table 2-6: Test Parameters for multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Channel model | NTN-TDLB100-300 |
| Antenna Configuration | 1Tx1Rx/1Tx2Rx |
| SCS and bandwidth | 15kHz: 20MHz |
| Test metric | SNR@ NACK ACK <0.1% SNR@ ACK miss <1% |

# References

[1]	TR38.811 v15.4.0, Study on New Radio (NR) to support non-terrestrial networks

[2]	TR38.821 v16.1.0, Solutions for NR to support non-terrestrial networks (NTN)

[3]	R4-2204027, Discussion on general issue on NTN demodulation requirements, Ericsson