3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204092

# Electronic Meeting, February 21- March 3, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Introducing missing MPR for NR-U PC5 UL MIMO

Agenda Item:	9.37.2 UE RF requirements [NR_bands_UL_MIMO_PC3_R17-Core]

Document for:	Approval

# Introduction

In R4#101b-e, in was noted in [1] that although n46 and n96 NR-U bands were part of Table 5.2D-1: NR operating bands for UL MIMO in FR1 since release 16, there was no MPR associated to UL MIMO in the specification. Furthermore, last meeting, the lower 6GHz unlicensed band n102 was also added to this table in [2]. In this contribution we make a proposal on how to introduce this specification in Release 17 and seek for input on what should be done in Release 16 specification.

# Discussion

## Support of PC5 NR-U UL MIMO

Similar to NR where the PC3 1Tx MPR is applicable to NR PC3 UL MIMO based on the assumption of 2x23dBm PAs, The 1Tx PC5 NR-U MPR is applicable to NRU PC5 UL MIMO if 2x20dBm architecture is assumed.

Proposal: Based on similar architecture assumptions than for PC3 NR UL MIMO, 1Tx NR-U MPR in Table 6.2F.2-1 is applicable to PC5 NR-U UL MIMO. Similarly, the NR-U PC5 1Tx A-MPR are applicable to PC5 NR-U UL MIMO.

## Introduction in release 17 38.101-1 specification

Given than NR-U requirements are in a specific suffix F clause and similar to the CA sub-clause 6.2F.2A, an UL MIMO sub-clause 6.2F.2D can be added with further sub-clauses for:

- UE maximum output power for UL MIMO

- UE maximum output power reduction for UL MIMO

- UE additional maximum output power reduction for UL MIMO

- Configured transmitted power for UL MIMO

Proposal:

- In Release 17 clause 6.2F.2D is added with relevant sub-clauses and tables and will point at the relevant NR-U specific requirements (UL MIMO power class, MPR, A-MPR) and NR UL MIMO requirements where appropriate (companion CR is provided in [3])

- For Release 16 we seek the group view on whether to add the same changes or remove n46, n96 and n102 from the list of UL MIMO bands in Table 5.2D-1 of 38.101-1.

# Conclusions

In this contribution, we discuss how to introduce the missing NR-U UL MIMO requirements in 38.101-1 and make the following proposals.

Proposal: Based on similar architecture assumptions than for PC3 NR UL MIMO, 1Tx NR-U MPR in Table 6.2F.2-1 is applicable to PC5 NR-U UL MIMO. Similarly, the NR-U PC5 1Tx A-MPR are applicable to PC5 NR-U UL MIMO.

Proposal:

- In Release 17 clause 6.2F.2D is added with relevant sub-clauses and tables and will point at the relevant NR-U specific requirements (UL MIMO power class, MPR, A-MPR) and NR UL MIMO requirements where appropriate (companion CR is provided in [3])

- For Release 16 we seek the group view on whether to add the same changes or remove n46, n96 and n102 from the list of UL MIMO bands in Table 5.2D-1 of 38.101-1.

# References

- R4-2200499 Requirement and signaling aspect of features requiring two transmit paths, Skyworks Solutions, Inc., RAN4#101b-e

- R4-2202422	Introduction of the lower 6GHz unlicensed band	Apple, Skyworks, MediaTek Inc., RAN4#101b-e

- R4-2204093 Draft CR TS 38.101-1: Introducing missing MPR for NR-U PC5 UL MIMO, Skyworks Solutions, Inc., RAN4#102-e