3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204143

# Electronic Meeting, February 21-March 3, 2022

Source:	LG Electronics

Title:	Discussion on CBM based inter-band DL CA within same frequency group

Agenda item:	10.4.2.1.1

Document for:	Discussion

Introduction

In this paper, we provide our view on the remained issues of RF core requirements for FR2 inter-band CA within the same frequency group based on CBM.

Discussion

In the last RAN4 meeting, the following was discussed during GTW and were encouraged to be discussed further.

- Introduce UE capability CBM HPC. - Which architecture UE supports is up to UE implementation - UE requirements - FFS on within same frequency group CBM DL requirements are based on single receiver and multi receiver architecture - between frequency groups CBM DL requirements are based on multi receiver architecture, IBM-requirements, relaxations can be discussed (i.e., relaxation could be not exact the same as IBM) - Fs_inter is introduced further discussed whether Fs_inter is for functional limitation or for performance functional separation - Applies within same frequency group band combinations for single receiver architecture - Applies to UEs not declaring HPC - Is introduced together with other LL n258+n261 UE requirements for both CBM and IBM when there is operator request for band combination within same frequency group - When CBM requirements are introduced for band combination within same frequency group also IBM requirements are introduced (earlier agreement) - REFSENS: Define the minimum CBM sensitivity requirements on the condition of normalized equal PSD for band combinations within same frequency group. - CR for CA_n257-n259, CA_n258-n260 and CA_n260-n261(for both CBM and IBM) i.e. CBM between frequency groups is agreed in RAN4#102 - CBM REFSENS between frequency groups is defined to be same as IBM REFSENS - FFS relaxation value - CR introducing LL combo n258+n261 with Fsinter and CR introducing LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261 are agreed as a package.

For UE supporting FR2-1 inter-band DL CA based on CBM within same frequency group, Fs_inter needs to be introduced either for functional limitation or for performance functional separation. And, LL combo CA_n258-n261, and LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261 are needed to be introduced as a package.

For LL combo CA_n258-n261 with CBM, the sensitivity requirements should be specified depending on the frequency separation from 0MHz to 4000MHz as Table 2.1 and Table 2.2.

Table 2.1:  ΔRIB,P,n reference sensitivity relaxation for inter-band CA with CBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n258-n261 | n258 | [2.5]note1 |
|  | n261 | [2.5] note1 |
| Note 1 : ‘Fs_inter’ is the maximum frequency separation between lower edge of lowest CC and upper edge of highest CC in FR2-1 inter-band DL CA based on CBM which UE can support with corresponding ΔRIB,P,n. If the configured frequency separation is larger than ‘Fs_inter’, additional relaxation is allowed. |  |  |

Table 2.2: Frequency separation class(Fs_inter) for inter-band CA with CBM

| Frequency separation class | Fs_inter (MHz) |
| --- | --- |
| 1 | ≤ 800 |
| 2 | ≤ 1400 |
| 3 | ≤ 2400 |
| 4 | ≤ 3400 |
| 5 | ≤ 4400 |

For LL combo CA_n258-n261 with IBM, the sensitivity requirement of 3.0dB can be considered as Table 2.3 compared to frequency separation of other band combos in Table 2.4.

Table 2.3:  ΔRIB,P,n reference sensitivity relaxation for inter-band CA with IBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n258-n261 | n258 | [3.0] |
|  | n261 | [3.0] |

Table 2.4:  ΔRIB,P,n reference sensitivity relaxation for inter-band CA with IBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) | Freq.Separation Min~Max (MHz) |
| --- | --- | --- | --- |
|  |  | IBM (Table 7.3A.2.3-1) |  |
| CA_n257-n259 | n257 | 4.0 | 10000~17000 |
|  | n259 | 4.0 |  |
| CA_n258-n260 | n258 | 3.5 | 9500~15750 |
|  | n260 | 3.5 |  |
| CA_n260-n261 | n260 | 3.5 | 8650~12500 |
|  | n261 | 3.5 |  |
| CA_n258-n261 | n258 |  | 0~4100 |
|  | n261 |  |  |

For LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261 with IBM, the sensitivity requirements were already defined as Table 2.5.

Table 2.5: ΔRIB,P,n reference sensitivity relaxation for inter-band CA with IBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 4.0 |
|  | n259 | 4.0 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

For LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261 with CBM, the sensitivity requirements have been being discussed as Table 2.6.

Table 2.6: ΔRIB,P,n reference sensitivity relaxation for inter-band CA with CBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | TBD |
|  | n259 | TBD |
| CA_n258-n260 | n258 | TBD |
|  | n260 | TBD |
| CA_n260-n261 | n260 | TBD |
|  | n261 | TBD |

As package, reference sensitivity relaxation of LL combo CA_n258-n261, and LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261, Table 2.7 and Table 2.8 can be considered for IBM and for CBM respectively.

Table 2.7: ΔRIB,P,n reference sensitivity relaxation for inter-band CA with IBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 4.0 |
|  | n259 | 4.0 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |
| CA_n258-n261 | n258 | [3.0] |
|  | n261 | [3.0] |

Table 2.8: ΔRIB,P,n reference sensitivity relaxation for inter-band CA with CBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | TBD |
|  | n259 | TBD |
| CA_n258-n260 | n258 | TBD |
|  | n260 | TBD |
| CA_n260-n261 | n260 | TBD |
|  | n261 | TBD |
| CA_n258-n261 | n258 | [2.5]note1 |
|  | n261 | [2.5] note1 |
| Note 1 : ‘Fs_inter’ is the maximum frequency separation between lower edge of lowest CC and upper edge of highest CC in FR2-1 inter-band DL CA based on CBM which UE can support with corresponding ΔRIB,P,n. If the configured frequency separation is larger than ‘Fs_inter’, additional relaxation is allowed. |  |  |

As package, for EIS spherical coverage requirement relaxation of LL combo CA_n258-n261, and LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261, Table 2.9 and Table 2.10 can be considered for IBM and for CBM respectively.

Table 2.9: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA with IBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 3.5 |
|  | n259 | 3.5 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |
| CA_n258-n261 | n258 | [3.0] |
|  | n261 | [3.0] |

Table 2.10: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA with CBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | TBD |
|  | n259 | TBD |
| CA_n258-n260 | n258 | TBD |
|  | n260 | TBD |
| CA_n260-n261 | n260 | TBD |
|  | n261 | TBD |
| CA_n258-n261 | n258 | [2.5]note1 |
|  | n261 | [2.5] note1 |
| Note 1 : ‘Fs_inter’ is the maximum frequency separation between lower edge of lowest CC and upper edge of highest CC in FR2-1 inter-band DL CA based on CBM which UE can support with corresponding ΔRIB,S,n. If the configured frequency separation is larger than ‘Fs_inter’, additional relaxation is allowed. |  |  |

Based on these, we propose as follows.

Proposal 1: Introduce LL combo n258+n261 UE requirements with Fs_inter for CBM, and without Fs_inter for IBM.

Proposal 2: Agree CR introducing LL combo n258+n261 with Fs_inter and CR introducing LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261 as a package.

- Reference sensitivity relaxation :

  - Table 2.7 for IBM

  - Table 2.8 for CBM

- EIS spherical coverage requirement relaxation

  - Table 2.9 for IBM

  - Table 2.10 for CBM

- Table 2.2 for Frequency separation class(Fs_inter) for inter-band CA with CBM

Proposal 3: Add ‘Fs_inter’ in feature list of NR_RF_FR2_req_enh2.

Conclusion

In this contribution, we provided our views on Fs_inter and reference sensitivity requirements for CBM based inter-band DL within the same frequency group. Proposals are as follows.

Proposal 1: Introduce LL combo n258+n261 UE requirements with Fs_inter for CBM, and without Fs_inter for IBM.

Proposal 2: Agree CR introducing LL combo n258+n261 with Fs_inter and CR introducing LH combos CA_n257-n259, CA_n258-n260 and CA_n260-n261 as a package.

- Reference sensitivity relaxation :

  - Table 2.7 for IBM

  - Table 2.8 for CBM

- EIS spherical coverage requirement relaxation

  - Table 2.9 for IBM

  - Table 2.10 for CBM

- Table 2.2 for Frequency separation class(Fs_inter) for inter-band CA with CBM

Proposal 3: Add ‘Fs_inter’ in feature list of NR_RF_FR2_req_enh2 as Table 2.11

Table 2.11: Feature list of NR_FR_FR2_req_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_RF_FR2_req_enh2 | X-x | Frequency separation class for inter-band DL CA based CBM | Capability to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in CA combination band for inter-band DL CA based on CBM. The UE indicating this capability shall meet the corresponding UE requirements defined in Section TBD. |  | yes | no | UE does not support inter-band DL CA with corresponding UE requirements defined in Section TBD. | Per BC | No | FR2 only |  |  | Optional with capability signalling |

Table 2.7: ΔRIB,P,n reference sensitivity relaxation for inter-band CA with IBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 4.0 |
|  | n259 | 4.0 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |
| CA_n258-n261 | n258 | [3.0] |
|  | n261 | [3.0] |

Table 2.8: ΔRIB,P,n reference sensitivity relaxation for inter-band CA with CBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | TBD |
|  | n259 | TBD |
| CA_n258-n260 | n258 | TBD |
|  | n260 | TBD |
| CA_n260-n261 | n260 | TBD |
|  | n261 | TBD |
| CA_n258-n261 | n258 | [2.5]note1 |
|  | n261 | [2.5] note1 |
| Note 1 : ‘Fs_inter’ is the maximum frequency separation between lower edge of lowest CC and upper edge of highest CC in FR2-1 inter-band DL CA based on CBM which UE can support with corresponding ΔRIB,P,n. If the configured frequency separation is larger than ‘Fs_inter’, additional relaxation is allowed. |  |  |

Table 2.9: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA with IBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 3.5 |
|  | n259 | 3.5 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |
| CA_n258-n261 | n258 | [3.0] |
|  | n261 | [3.0] |

Table 2.10: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA with CBM for power class 3

| NR CA band combinations | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | TBD |
|  | n259 | TBD |
| CA_n258-n260 | n258 | TBD |
|  | n260 | TBD |
| CA_n260-n261 | n260 | TBD |
|  | n261 | TBD |
| CA_n258-n261 | n258 | [2.5]note1 |
|  | n261 | [2.5] note1 |
| Note 1 : ‘Fs_inter’ is the maximum frequency separation between lower edge of lowest CC and upper edge of highest CC in FR2-1 inter-band DL CA based on CBM which UE can support with corresponding ΔRIB,S,n. If the configured frequency separation is larger than ‘Fs_inter’, additional relaxation is allowed. |  |  |

Table 2.2: Frequency separation class (Fs_inter) for inter-band CA with CBM

| Frequency separation class | Fs_inter (MHz) |
| --- | --- |
| 1 | ≤ 800 |
| 2 | ≤ 1400 |
| 3 | ≤ 2400 |
| 4 | ≤ 3400 |
| 5 | ≤ 4400 |

- Reference

[1] R4-2202342, “WF for FR2 DL CA”, Nokia

[2] R4-2202319, “Email discussion summary for [101-bis-e][119] NR_RF_FR2_req_enh2_Part_1”, Moderator (Nokia)