3GPP TSG-RAN WG4 Meeting # 102-e												R4-2204284

Electronic Meeting, February 21 – March 3, 2022

Source: 	OPPO

Title: 	Discussion on signaling characteristics requirements for RedCap UE

Agenda Item:	10.20.3.1.14

Document for:	Discussion

# 1 Introduction

In last RAN4 meeting, a way forward on RRM requirements for RedCap UE was agreed in [1]. In this contribution, we provide our further consideration on the remaining issues of RRM requirements for RedCap UE.

- Discussion

RLM/BFD

SSB-based RLM : evaluation period for Qout and Qin - Double the evaluation period for Qout - keep the legacy evaluation period for Qin - Further discuss in performance part to relax test case SINR by X db to compensate for loss in SINR accuracy, where X is TBD. CSI-RS-based RLM : evaluation period for Qout and Qin - Follow the agreement from corresponding issue for SSB-based RLM evaluation period. Whether to extend the lower bound in RLM evaluation period: - lower bound in RLM evaluation period is extended. If lower bound is extended for RLM evaluation period, how much to extend: - Option 1 (MTK): By factor 2 SSB-based based BFD: evaluation period - Option 1 (Apple, vivo, Oppo, HW, E///, QC): No need to extend the evaluation period for BFD in FR1 and FR2. - Option 2 (MTK): Follow the agreements from RLM. CSI-RS-based BFD: evaluation period - Option 1 (Apple, vivo, Oppo, HW, E///, QC): No need to extend the evaluation period for BFD for FR1 and FR2. - Option 2 (MTK): Follow the agreements from RLM.

As agreed in RAN4, both evaluation period and lower bound in RLM evaluation period are extended. Based on LTE requirements for the cases of using 1Rx, we shall support extending the lower bound by 2 times.

Proposal 1: lower bound in RLM evaluation period is extended by factor 2.

If it is agreed to not extend the BFD evaluation period, then there is no need to extend the lower bound.

Proposal 2: No need to extend the evaluation period for BFD in FR1 and FR2.

BWP switching

New BWP switching delay when only center-frequency is changed in Rel-17 - Option 1 (CMCC, E///, HW, Nokia): Define new BWP switching delay involving only changing of the center-frequency of the BWP without changing its BW, SCS or any other parameter for RF retuning as follows: Frequency Range; Type 1 Delay (us); Type 2 Delay (us). 1; 200; 1050. 2; 200; 1050. - Option 2 (Xiaomi, vivo, Oppo, ZTE, MTK, QC, Apple): RAN4 to reuse the legacy BWP switching delay for RedCap UE in Rel-17.

The background to this BWP switching issue is that when SSBs are not transmitted in the RedCap configured initial DL BWP, the UE may have to perform RF retuning in order to switch to different BWP (e.g. initial BWP) to receive the SSBs. According to RAN1’s latest agreements, applicablility for RedCap UE to use NCD-SSB is identified. We think the BWP switching when only center-frequency is changed is not expected to perform frequently. Also, the BWP switching involves baseband parameters reconfiguration and RF reconfiguration. RedCap UE may take more time for baseband parameters reconfiguration due to inferior demodulation performance. Then, we prefer to reuse the current BWP switching requirements for RedCap UE.

Proposal 3: RAN4 to reuse the legacy BWP switching delay for RedCap UE in Rel-17.

Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 - Option 1 (QC): - RAN4 to define L1 measurement gaps, in addition to the legacy MGs (L3), to perform RLM/BFD/CBD/L1-RSRP based on SSB outside active BWP, as an optional capability for Redcap UEs that indicate the optional ‘not need for NCD-SSB’ capability. Option 2 (HW, E///, Apple, CMCC, Xiaomi, MTK): Do not introduce L1 measurement gaps.

We agree that the UE can perform CSI-RS based RLM/BFD/CBD/L1-RSRP when SSB is outside active BWP for a UE supporting ‘not need for NCD-SSB’ capability. There is no need to introduce L1 measurement gaps.

Proposal 4: Do not introduce L1 measurement gaps.

- Conclusion

Proposal 1: lower bound in RLM evaluation period is extended by factor 2.

Proposal 2: No need to extend the evaluation period for BFD in FR1 and FR2.

Proposal 3: RAN4 to reuse the legacy BWP switching delay for RedCap UE in Rel-17.

Proposal 4: Do not introduce L1 measurement gaps.

- Reference

[1] R4-2202670, WF on RedCap RRM requirements, Ericsson, RAN4#101-bis-e