3GPP TSG-RAN4 Meeting #102-e	R4-2204340

# Electronic Meeting, Feb. 21 – Mar. 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on active DL and UL TCI state list update delay requirements in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce RRM requirements for active DL and UL TCI state list update delay in R17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce RRM requirements for active DL and UL TCI state list update delay in R17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement for for active DL and UL TCI state list update delay is specified in R17 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.5, 8.16.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

8.15.5	Active Downlink TCI state list update delay

The requirements specified in this clause apply to

-	Separate TCI list update delay, while the separate TCI list comprises more than one target separate TCIs, and at least one DL TCI is included.

-	Joint TCI list update delay, while the join TCI list comprises at least one target joint TCI.

Upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new target TCI states at the first slot that is after

n +3Nslotsubframe,µ + (THARQ + KD* TL1-RSRP + TO*(Tfirst-SSB, List + TSSB-proc)) / NR slot length.

Where

-	If any DL TCI is unknown, KD = 1 and TO is TOuk,

-	if all unknown TCI states involve QCL-TypeD, and the source RSs for QCL-TypeD are SSBs, TOuk = 0,

-	otherwise, TOuk = 1

-	If all DL TCIs are known, KD = 0, and TO is TOk,

-	if any target TCI state is not in the active TCI state list, TOk = 1,

-	otherwise, TOk = 0

-	Tfirst-SSB, List  = Ncell * Tfirst-SSB, where Ncell is the number of cells associated with the target DL TCIs that are not in the active TCI list, whose SSBs are overlapped.

-	THARQ, TL1-RSRP, TSSB-proc , TOk and TOuk are defined in clause 8.15.3.

For FR2, when UE receives PDSCH carrying MAC-CE active TCI state list update for

-	joint TCI state list update, or

-	separate TCI list update, while the separate TCI list comprises at least one DL TCI and one UL TCI,

the reference slot n for OTA testing is the later one between endpoints for DL TCI list update delay specified in this clause, and UL TCI list update delay specified in 8.16.5.

<< End of change 1>>

<< Start of change 2>>

8.16.5	Active Uplink TCI state list update delay

The requirements specified in this clause apply to

-	Separate TCI list update delay, while the separate TCI list comprises more than one target separate TCIs, and at least one DL TCI is included.

-	Joint TCI list update delay, while the join TCI list comprises at least one target joint TCI.

Upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to transmit PUCCH, PUSCH or SRS with the new target TCI states at the first slot that is after

n+ 3Nslotsubframe,µ + (THARQ + KU*TL1-RSRP + NM * (Ncell * Tfirst_target-PL-RS + 4* Ncell * Ttarget_PL-RS + 2ms))) / NR slot length,

Where

-	If any UL TCI is unknown, KU = 1 and NM is 1.

-	If all UL TCIs are known, KU = 0, and

-	if the target PL-RS associated with or included in any UL TCI is not maintained by the UE, NM = 1,

-	otherwise, NM = 0

-	Ncell is the number of cells associated with the target UL TCIs whose PL-RSs are SSBs, and the PL-RSs are not maintained, and these SSBs are overlapped

-	THARQ, TL1-RSRP, Tfirst_target-PL-RS and Ttarget_PL-RS are defined in clause 8.16.3.

For FR2, when UE receives PDSCH carrying MAC-CE active TCI state list update for

-	joint TCI state list update, or

-	separate TCI list update, while the separate TCI list comprises at least one DL TCI and one UL TCI,

the reference slot n for OTA testing is the later one between endpoints for DL TCI list update delay specified in 8.15.5, and UL TCI list update delay specified in this clasue.

<< End of change 2>>