3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204370

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for the number of ACK and NACK in CGI reading test case in NR SA for R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2022-2-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In some SA FR2 tests, some configuration numbers are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the CGI reading test case, UE shall report the cell global identifier within 200 ms from the start of T3. In this test case, the test requirement can be simplified as “Test requirement = 45 + 150 (Tidentify_CGI,E-UTRAN) + 5 ms”, i.e., UE should be able to transmit the ACK/NACK to the network during the time period of 45 + 5 ms. Within 50 ms, the original number of ACK/NACK should depend on FDD/TDD pattern. Thus, the current number is incorrect. - For the FDD mode, it would be easy to calculate the number of the ACK/NACK for 45 + 5 ms, i.e., the number of ACK/NACK will be calculated based on 50 ms. - FDD @ 15kHz - the data will not be transmitted within SMTC occasssion - the ACK/NACK will be transmitted after 2 slots when UE receives data (as red arrow in the Fig). - # of ACK/NACK: 50 – 3 (due to no data within SMTC) – 2×2 (Considering worst case, the ACK/NACK may not be transmitted in last two slot, and two times is because time period for 45 ms and 5 ms are separated) = 43 - For the TDD mode, due to UL/DL pattern, it would be difficult to separately calculate the number of ACK/NACK for 45 and 5 ms. Thus, to simplify the calculation, the number of ACK/NACK will be calculated based on 45 ms, i.e., not to consider the remaining 5 ms. - TDD @ 15kHz - the data will not be transmitted within SMTC occasssion - # of ACK/NACK: 18 (there are 18 DL slots within 45 ms) – 3 (no data within SMTC) – 1 (Considering worst case, the ACK/NACK may not be transmitted for the last slot containing data within 45 ms) = 14 - TDD @ 30kHz - the data will not be transmitted within SMTC occasssion - Considering worst case, the DL/UL pattern is shifted. - # of ACK/NACK: 45 (there are 45 DL slots within 45 ms) – 2×3 (no data within SMTC) – 5 (Considering worst case, the ACK/NACK may not be transmitted for the last 5 slots containing data within 45 ms) = 34 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test cell configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.6.7.2.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 200 milliseconds from the start of T3.

Test requirement = RRC Procedure delay with additional margin + Tidentify_CGI,E-UTRAN + reporting delay

= 15 + 30 + 150 + 2ms from the start of T3

= 197 ms, allow 200 ms.

-	The UE shall be scheduled continuously throughout the test, and from the start of T3 until 200 ms at least the number of ACK/NACK specified in NOTE 2 shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of cell 2, where X is defined in Table 8.2.2.2.15-1 . Secondly, given that continuous DL data allocation, additional 43, 14 and 34 ACK/NACK shall be sent for FDD 15 kHz, TDD 15 kHz and TDD 30kHz, respectively, from the start of T3 until 200 ms excludes 150 ms for identifying the cell global identifier of cell 2.

<End of Change 1>