3GPP TSG-RAN WG4 Meeting #102-e		R4-2204382

E-meeting, 21 February – 3 March, 2022

Agenda item:	10.12.2.3.2

Source:	Intel Corporation

Title:	Network assistance signalling for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR

Document for:	Discussion

# Introduction

In RAN #101-bis-e meeting, WF on CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR [1] was approved and multiple agreements were reached on network assistance signalling topic. In this paper we provide our view on remaining open issues.

# Discussion

In the previous RAN4 meeting the following agreement was reached on network assistance signaling of some interference parameters:

- Need of CRS sequence information for LLR weighting - Not restrict UE implementation, CRS sequence not needed from baseline receiver assumption for defining RAN4 minimum performance requirements - By default, Cell ID information is not needed from RAN4 performance requirements aspect; Cell ID can be included into NWA signalling as optional. - FFS the maximum number of cell ID information - Whether UE needs to be indicated the CRS port number - From RAN4 minimum performance requirements aspect, UE follow below default assumption without blind detection as baseline assumption - 4 CRS ports for scenario 2 - Aligned with serving cell for scenario 1 - By default, number of CRS ports no need to be informed via signalling with following default assumption from RAN4 performance requirements aspect - Number of CRS ports information can be included into NWA signalling (optional) - CRS-IM requirements are not applicable if the default configuration is not valid and NWA signalling is not provided. - v-shift information - The baseline assumption: No need to introduce NWA signaling for v-shift information - LTE cell presence and carrier frequency for scenario 2 - From RAN4 minimum performance requirements aspect, it’s not required to blind detect LTE carrier frequency information for CRS-IM receiver baseline assumption. - The information can be awared by following possible ways: - For scenario 2, inter-RAT MO configuration information (LTE cell presence and carrier frequency) can be utilized to perform CRS-IM if configured by NW. - LTE cell carrier frequency information can be informed to UE by NWA signalling for scenario 2 (optional) - If such information not conveyed to UE, UE not expected to enable CRS-IM receiver. - LTE channel bandwidth for scenario 2 - Option 1 (baseline assumption): - For scenario 2, LTE channel bandwidth information can be awared by following possible ways: - With inter-RAT MO configured, 1) UE can use PBCH decoding to obtain channel bandwidth information for CRS-IM if PBCH is within the configured measurement gap, or 2) UE can use power difference detection to obtain channel bandwidth information - For UE capable of obtaining LTE CBW information by PBCH decoding and/or power detection, inter-RAT MO information is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM. - LTE channel bandwidth information can be informed to UE by NWA signalling (optional) - For UE not capable of obtaining LTE CBW information by PBCH decoding and/or power detection, NWA signalling on LTE CBW is needed to perform CRS-IM otherwise UE not expected to perform CRS-IM. - Separate capability will be introduced for UE capable of performing CRS-IM in scenario 2 without the above new NWA signalling on LTE channel bandwidth. - Enabling CRS-IM receiver should not impact on existing RRM procedure and RRM requirements - Other options are not precluded.

Based on these agreements we have the following assumptions on interference cell signals parameters and NWA signalling in case of LLR weighting CRS-IM receiver

Table 1. Summary of views on network assistance signalling.

|  | Scenario 1 | Scenario 2 |
| --- | --- | --- |
| v-shift and/or Physical Cell ID | Not provided Physical Cell ID is optional field | Not provided Physical Cell ID is optional field |
| MBSFN configuration | Assume same as serving cell If it is not the same than NWA signalling | Assume no MBSFN configuration If it is configured than NWA signalling |
| CRS muting usage flag | Assume no CRS muting If it is configured than NWA signalling | Assume no CRS muting If it is configured than NWA signalling |
| Center of the LTE carrier | Assume same as serving cell If it is not the same than NWA signalling | Inter-RAT MO or NWA signalling |
| Bandwidth of the LTE carrier | Assume same as serving cell If it is not the same than NWA signalling | Blind detection in case of configured Inter-RAT MO or NWA signalling |
| Number of CRS antenna ports | Assume same as serving cell If it is not the same than NWA signalling | Assume 4 CRS ports If it is not the true than NWA signalling |

Based on this table we can observe that all parameters are included in the NWA signalling and can be provided under certain conditions. Therefore, the details of NWA signaling design should be further discussed. In the previous RAN4 meeting the following agreement was reached:

- Signalling design when the default NW configuration assumptions are NOT valid - The signalling (when the default NW configuration assumptions are NOT valid) is [per serving cell], and further discuss the maximum number of interference cells can be signalled.

One of the options which we can consider is reusing of LTE NWA signalling design for CRS parameters:

CRS-AssistanceInfoList-r13 ::= SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo-r13 CRS-AssistanceInfo-r13 ::= SEQUENCE { physCellId-r13 PhysCellId, antennaPortsCount-r13 ENUMERATED {an1, an2, an4, spare1}, mbsfn-SubframeConfigList-r13 MBSFN-SubframeConfigList OPTIONAL, -- Need ON ..., [[ mbsfn-SubframeConfigList-v1430 MBSFN-SubframeConfigList-v1430 OPTIONAL -- Need ON ]] }

In case such design is reused, we will have the following design of NR NWA signalling:

Table 2. NWA signalling design Option 1

CRS-AssistanceInfoList ::= SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo CRS-AssistanceInfo ::= SEQUENCE { physCellId PhysCellId, OPTIONAL carrierFreqDL INTEGER (0..16383), OPTIONAL carrierBandwidthDL ENUMERATED {n6, n15, n25, n50, n75, n100}, OPTIONAL nrofCRS-Ports ENUMERATED {an1, an2, an4}, OPTIONAL mbsfn-SubframeConfigList EUTRA-MBSFN-SubframeConfigList OPTIONAL crs-IntfMitigEnabled ENUMERATED {enabled} OPTIONAL }

Another possible way is to have the sequence for each parameter:

Table 3. NWA signalling design Option 2

CRS-AssistanceInfo ::= SEQUENCE { physCellIdList SEQUENCE (SIZE (1..maxCellReport)) OF PhysCellId, OPT carrierFreqDLlist SEQUENCE (SIZE (1..maxCellReport)) OF carrierFreqDL, OPT carrierBandwidthDLlist SEQUENCE (SIZE (1..maxCellReport)) OF carrierBandwidthDL OPT nrofCRS-PortsList SEQUENCE (SIZE (1..maxCellReport)) OF nrofCRS-Ports OPT mbsfn-SubframeConfigList SEQUENCE (SIZE (1..maxCellReport)) OF EUTRA-MBSFN-SubframeConfigList OPT crs-IntfMitigEnabledList SEQUENCE (SIZE (1..maxCellReport)) OF crs-IntfMitigEnabled OPT } carrierFreqDL INTEGER (0..16383), carrierBandwidthDL ENUMERATED {n6, n15, n25, n50, n75, n100}, nrofCRS-Ports ENUMERATED {an1, an2, an4}, crs-IntfMitigEnabled ENUMERATED {enabled}

One of the benefits of such design is that in case NWA signalling is needed, but the parameters are same for all neighboring cells, which is possible for most of parameters except Cell ID, BS can provide one common information for these parameters and providing of multiple same values is not required.

Proposal 1:	Use one of the NWA signalling design options listed in Table 2 or 3.

As for maximum number of interference cells for which NWA signalling is provided, we can reuse assumptions from LTE and consider 8, because based on our understanding there is no big change in NR deployment structure in comparison to LTE.

Proposal 2:	Maximum number of interference cells, for which NWA signalling is provided, is equal to 8.

Another open question is NWA signalling granularity: per cell or per UE. Usually, per UE granularity is considered in case NWA signalling should be adopted depending on UE capability. The only parameter, which probably requires signalling depending on UE capability, is LTE channel bandwidth for Scenario 2. However, in case NWA signalling Option 2 (Table 2) will be used and LTE channel bandwidth is same for all neighboring cells, BS just needs to provide the one value with information about this parameter to all UEs (in case of per cell NWA signalling) or subset of UEs (in case of per UE signalling). We think that from network point of view it will be simpler to provide broadcast information to all UEs rather than checking the capability of each UE and adjust the signalling for each UE. Therefore, we suggest to consider per cell signalling. Also, based on above discussion, we suggest to consider signalling of LTE channel bandwidth as mandatory for Scenario 2 to enable CRS-IM processing.

Proposal 3:	Consider per Serving Cell granularity for NWA signalling.

Proposal 4:	Consider signalling of LTE channel bandwidth as mandatory for Scenario 2 to enable CRS-IM processing.

# Conclusion

In this paper we provided our view on network assistance signalling for CRS-IM receiver for scenarios with overlapping spectrum for LTE and NR and made the following proposals:

Proposal 1:	Use one of the NWA signalling design options listed in Table 2 or 3.

Table 2. NWA signalling design Option 1

CRS-AssistanceInfoList ::= SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo CRS-AssistanceInfo ::= SEQUENCE { physCellId PhysCellId, OPTIONAL carrierFreqDL INTEGER (0..16383), OPTIONAL carrierBandwidthDL ENUMERATED {n6, n15, n25, n50, n75, n100}, OPTIONAL nrofCRS-Ports ENUMERATED {an1, an2, an4}, OPTIONAL mbsfn-SubframeConfigList EUTRA-MBSFN-SubframeConfigList OPTIONAL crs-IntfMitigEnabled ENUMERATED {enabled} OPTIONAL }

Table 3. NWA signalling design Option 2

CRS-AssistanceInfo ::= SEQUENCE { physCellIdList SEQUENCE (SIZE (1..maxCellReport)) OF PhysCellId, OPT carrierFreqDLlist SEQUENCE (SIZE (1..maxCellReport)) OF carrierFreqDL, OPT carrierBandwidthDLlist SEQUENCE (SIZE (1..maxCellReport)) OF carrierBandwidthDL OPT nrofCRS-PortsList SEQUENCE (SIZE (1..maxCellReport)) OF nrofCRS-Ports OPT mbsfn-SubframeConfigList SEQUENCE (SIZE (1..maxCellReport)) OF EUTRA-MBSFN-SubframeConfigList OPT crs-IntfMitigEnabledList SEQUENCE (SIZE (1..maxCellReport)) OF crs-IntfMitigEnabled OPT } carrierFreqDL INTEGER (0..16383), carrierBandwidthDL ENUMERATED {n6, n15, n25, n50, n75, n100}, nrofCRS-Ports ENUMERATED {an1, an2, an4}, crs-IntfMitigEnabled ENUMERATED {enabled}

Proposal 2:	Maximum number of interference cells, for which NWA signalling is provided, is equal to 8.

Proposal 3:	Consider per Serving Cell granularity for NWA signalling.

Proposal 4:	Consider signalling of LTE channel bandwidth as mandatory for Scenario 2 to enable CRS-IM processing.

# References

- R4-2203131 “WF on general part and 15 kHz NR SCS scenario for CRS-IM receiver” , China Telecom, RAN4 #101-bis-e, January 2022.