3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204434

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for RF requirements due to Tx power imbalance between IAB-MT and IAB-DU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Case-6 timing is introduced in Rel-17 in eIAB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce RF requirement on for IAB node when transmitting simultaneously at IAB-MT and co-located IAB-DU with manufacture declared power imbalance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RF requirement is not clear when IAB-MT and the co-located IAB-DU perform simultaneous transmission with imbalanced power |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2,9.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Example how to clarify the modulation quality requirement>

9.6.2	OTA modulation quality

9.6.2.1	IAB-DU OTA modulation quality

The requirements in clause 9.6.2 for BS type 1-O and type 2-O in TS 38.104 [2] apply to IAB-DU type 1-O and type 2-O respectively.

When the transmission operations are simultaneously performed by an IAB-DU and an IAB-MT within an IAB node, the requirements of OTA modulation quality should be met when the power imbalance between IAB-DU and IAB-MT is within the manufacturer declared power imbalance.

9.6.2.2	IAB-MT OTA modulation quality

9.6.2.2.1	General

Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). Details about how the EVM is determined are specified in Annex D for FR1 and Annex E for FR2.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

When the transmission operations are simultaneously performed by an IAB-DU and an IAB-MT within an IAB node, the requirements of OTA modulation quality should be met when the power imbalance between IAB-DU and IAB-MT is within the manufacturer declared power imbalance.

<Example ends>

<Example how to clarify the ACLR requirement>

9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

9.7.3.1	General

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON period.

When the transmission operations are simultaneously performed by an IAB-DU and an IAB-MT within an IAB node, the requirements of OTA ACLR in table 9.7.3.3-1, 9.7.3.3-3, 9.7.3.3-4, 9.7.3.4-1, 9.7.3.4-3, 9.7.3.4-4 should be met when the power imbalance between IAB-DU and IAB-MT is within the manufacturer declared power imbalance.

<Example ends>