3GPP TSG-RAN WG4 Meeting #102-e     	R4-2204540

E-meeting, February 21st – March 3rd, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on eDRX enhancements for RedCap UEs

Agenda item:	10.20.3.2

Document for:	Discussion

# 1	Introduction

In this contribution,  we present Nokia’s views on the eDRX enhancements for RedCap UEs. The following topics are discussed:

Idle state serving cell eDRX requirements

Idle state cell reselection eDRX requirements

2	Discussion

2.1	Idle state serving cell eDRX requirements

In the last RAN4 meeting, the following was agreed regarding serving cell eDRX requirements:

Issue 1-2-1: Nserv and N1 for eDRX length up to 10.24s (FR2) Moderator Note: The following table can be referred for Nserv (FR2) requirement discussion. eDRX cycle length [s]; FR2 Scaling Factor (N1); Nserv [number of eDRX cycles]. 2.56; [TBD]; N1*2. 5.12. 10.24. Agreement: N1 = 3 for eDRX = 2.56 Tentative agreement: N1 = 3 for eDRX = 5.12 and 10.24s. Issue 1-2-2: Serving cell requirements for eDRX length larger than 10.24s Agreement: The framework of requirements listed below are agreeable Tentative agreement: Use the number in the table and put the formula Nserv*DRX_cycle1.28*1.28 in the NOTES of the table, same for all cases when number and formula are used to express the same requirements FR1 Nserv for 10.24s<eDRX cycle≤10485.76s eDRX cycle length [s]; DRX cycle length [s]; PTW length [s] (number of 1.28s periods); Scaling Factor (N1) for FR1; Nserv [number of DRX cycles]. 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 0.32; [TBD]; 1; N1*2. 0.64; [TBD]; N1*2. 1.28; [TBD]; N1*2. 2.56; [TBD]; N1*2. Note: FFS on whether M1 will be used or not for DRX = 0.32 and 0.64s. FR2 Nserv for 10.24s<eDRX cycle≤10485.76s eDRX cycle length [s]; DRX cycle length [s]; PTW length [s] (number of 1.28s periods); Scaling Factor (N1) for FR2; Nserv [number of DRX cycles]. 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 0.32; [TBD]; [TBD]; N1*2. 0.64; [TBD]; [TBD]; N1*2. 1.28; [TBD]; [TBD]; N1*2. 2.56; [TBD]; [TBD]; N1*2. Note: FFS on whether M1 will be used or not for DRX = 0.32 and 0.64s. Agreement: agree the following table for FR1 Serving cell requirements for eDRX length larger than 10.24s when DRX = 1.28 and 2.56s eDRX cycle length [s]; DRX cycle length [s]. PTW length [s] (number of 1.28s periods). 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 1.28; ≥2.56 (2). 2.56; ≥5.12 (4). Issue 1-2-2-1: FR1 PTW length (N1=1), whether M1 should be considered for FR1/FR2 Nserv when DRX = 0.32 and 0.64s Option 1: Keep M1 (MTK Huawei xiaomi Nokia oppo xiaomi) Option 2: Do not use M1(Apple CMCC Ericsson) Recommended WF: FFS Issue 1-2-2-2: N1 for FR2 Nserv requirements Tentative agreement: Option 1: N1= [8] for DRX= 0.32s; [5] for DRX=0.64s; [4] for DRX= 1.28s; [ 3] for DRX =2.56s

N1 in FR2

In the last meeting, there was no consensus on the values of N1 to be considered for eDRX length up to 10.24s. In the baseline requirements from Rel-15, the N1 scaling factor is reduced from 8 to 3, as the DRX cycle length increases from 0.32s to 2.56s. In our view, the same value could be assumed for eDRX cycle lengths up to 10.24s.

N1 = 3 for eDRX cycle length ≤ 10.24s.

Additionally, there was no consensus with the scaling factor N1 to be used for the requirements with PTW in FR2. As mentioned in the previous meeting, we can follow the tentative agreement proposed by the moderator.

For 20.48s ≤ eDRX cycle length,  N1= 8 for DRX= 0.32s;  N1 = 5 for DRX=0.64s;   N1 = 4 for DRX= 1.28s; N1 = 3 for DRX =2.56s;

- Serving cell requirements for eDRX length larger than 10.24s

In the last meeting, it was discussed whether to consider M1 for serving cell requirements with eDRX length larger than 10.24s. The scaling factor M1 was introduced for power saving in short DRX cycles, and the justification behind the proposal to not use this scaling factor is that most of the power saving in eDRX cycle would come from the time spent outside PTW.

Also, in the last meeting, it was discussed if the eDRX cycle length of 20.48s is feasible or not in FR2. The reasoning for questioning if this eDRX cycle is feasible or not is that it would require the same number of “on occasions” as the number of “on occasions” in non eDRX operation depending on the eDRX / DRX configuration.

Consider the example in the figures below. In the first figure, we have a UE configured with DRX cycle = 640 ms. If we take a time of 20.48s, there would be 32 DRX cycles in this period. In the second figure, we have a UE configured with DRX cycle = 320 ms and eDRX cycle of 20.48s. Considering that PTW length is given by Nserv * DRX_cycle, if we consider N1 = 8, M1 = 2 and Nserv = 2*M1*N1, PTW would be 10.24s, the number of DRX cycles within a 20.48s period would be 32 as well, just in the case in Fig 1.

Despite the number of DRX cycles being the same in both cases, it is important to notice that with the eDRX configuration, the UE would be half of the eDRX cycle outside PTW, i.e., it could go to a deep sleep state for half of the eDRX cycle. However, according to TS 38.840, the total transition time (ramp down and ramp up) for a deep sleep is only 20 ms. So, in both cases, the UE could go to a deep sleep between the DRX cycles.  Of course, the number of wake ups within a time period depends on the exact combination of DRX and eDRX cycles, but for 20.48s and 40.96s the best option for the UE, in terms of number of wake ups, would be an eDRX cycle equal to 5.12s and 10.24s.

In FR2, there is limited power saving in the configuration of eDRX cycles equal to 20.48s and 40.96s, when compared to the configuration of eDRX cycles of 5.12s or 10.24s. The exact power saving, though, depends on the exact combination of eDRX and DRX cycle, and our view is that requirements could be specified also for 20.48s and 40.96s eDRX cycles, even if some configurations are not as beneficial as others.

Figure 1 - DRX operation with DRX cycle = 0.64s

Figure 2 - eDRX operation with eDRX cycle = 20.48s and DRX cycle = 0.32 s

Considering that in eDRX operation, most of the decrease in the power consumption comes from sleep during the time outside PTW, M1 might not be necessary to be considered in FR2. This is considered in the tables below.

Define Nserv for 20.48 ≤ eDRX_IDLE_cycle ≤ 10485.76s in FR1 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) for FR1 | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 1.28 (1) | 1 | N1*2 |
|  | 0.64 | ≥ 2.56 (2) |  | N1*2 |
|  | 1.28 | ≥ 2.56 (2) |  | N1*2 |
|  | 2.56 | ≥ 5.12 (4) |  | N1*2 |

Define Nserv for 20.48 ≤ eDRX_IDLE_cycle ≤ 10485.76s in FR2 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) for FR2 | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 5.12 (4) | 8 | N1*2 |
|  | 0.64 | ≥ 6.4 (5) | 5 | N1*2 |
|  | 1.28 | ≥ 10.24 (8) | 4 | N1*2 |
|  | 2.56 | ≥ 15.36 (12) | 3 | N1*2 |

2.2 	Idle state cell reselection eDRX requirements

In the last RAN4 meeting, the framework for the requirements with eDRX was agreed, but the exact requirements are still open for discussion.

Agreement: The framework of requirements listed below are agreeable Agreement: Use the number in the table and put the formula Nserv*DRX_cycle1.28*1.28 in the NOTES of the table, same for all cases when number and formula are used to express the same requirements FR1 Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra when eDRX<= 10.24s eDRX cycle length [s]; Scaling Factor (N1) FR1; Tdetect,NR_Intra [s] (number of eDRX cycles); Tmeasure,NR_Intra [s] (number of eDRX cycles); Tevaluate,NR_Intra [s] (number of eDRX cycles). 2.56; 1; TBD; TBD; TBD. 5.12; TBD; TBD; TBD. 10.24; TBD; TBD; TBD. FR1 Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra when 10.24 < eDRX eDRX cycle length [s]; DRX cycle length [s]; PTW length [s] (number of 1.28s periods); FR1 Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 0.32; TBD; 1; TBD; TBD; TBD. 0.64; TBD; TBD; TBD. 1.28; TBD; TBD; TBD. 2.56; TBD; TBD; TBD. FR2 Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra when eDRX<= 10.24s eDRX cycle length [s]; FR2 Scaling Factor (N1); Tdetect,NR_Intra [s] (number of eDRX cycles); Tmeasure,NR_Intra [s] (number of eDRX cycles); Tevaluate,NR_Intra [s] (number of eDRX cycles). 2.56; TBD; TBD; TBD; TBD. 5.12; TBD; TBD; TBD. 10.24; TBD; TBD; TBD. FR2 Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra when 20.48 ≤ eDRX eDRX cycle length [s]; DRX cycle length [s]; PTW length [s] (number of 1.28s periods); FR2 Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 0.32; TBD; TBD; TBD; TBD; TBD. 0.64; TBD; TBD; TBD; TBD. 1.28; TBD; TBD; TBD; TBD. 2.56; TBD; TBD; TBD; TBD. Note: when eDRX = 20.48s and 40.96s requirements are FFS.

Idle-state cell reselection requirements without PTW

- In the last meeting, there was no consensus on the exact values of Tdetec,NR_intra, Tmeasure,NR_intra and Tevaluate,NR_intra in FR1 and FR2. During the meeting, the moderator presented compromise proposals for FR1 and FR2, and we can agree to these proposals, which are repeated below:

Define Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra when eDRX_IDLE_cycle ≤ 10.24 in FR1 as:

| eDRX cycle length [s] | Scaling Factor (N1) FR1 | Tdetect,NR_Intra [s] (number of eDRX cycles) | Tmeasure,NR_Intra [s] (number of eDRX cycles) | Tevaluate,NR_Intra [s] (number of eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 | 1 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 |  | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 |  | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |

Define Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra when when eDRX_IDLE_cycle ≤ 10.24 in FR2 as:

| eDRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of eDRX cycles) | Tmeasure,NR_Intra [s] (number of eDRX cycles) | Tevaluate,NR_Intra [s] (number of eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 |  | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 |  | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |

Idle-state cell reselection requirements without PTW

Define Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra  when 20.48 ≤ eDRX cycle in FR1 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | FR1 Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 1.28 (1) | 1 | eDRX_cycle_length*23PTW/DRX_cycle_length (23) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥ 1.28 (1) |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥ 2.56 (2) |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥ 5.12 (4) |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Define Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra  when 20.48 ≤ eDRX cycle in FR2 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | FR1 Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 1.28 (1) | 8 | eDRX_cycle_length*23PTW/DRX_cycle_length (23) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥ 1.28 (1) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥ 2.56 (2) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥ 5.12 (4) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

- Conclusions

In this contribution, Nokia’s views on eDRX enhancements are presented. The following observations and proposals are discussed:

- N1 = 3 for eDRX cycle length ≤ 10.24s.

- For 20.48s ≤ eDRX cycle length,  N1= 8 for DRX= 0.32s;  N1 = 5 for DRX=0.64s;   N1 = 4 for DRX= 1.28s; N1 = 3 for DRX =2.56s;

- In FR2, there is limited power saving in the configuration of eDRX cycles equal to 20.48s and 40.96s, when compared to the configuration of eDRX cycles of 5.12s or 10.24s. The exact power saving, though, depends on the exact combination of eDRX and DRX cycle, and our view is that requirements could be specified also for 20.48s and 40.96s eDRX cycles, even if some configurations are not as beneficial as others.

Define Nserv for 20.48 ≤ eDRX_IDLE_cycle ≤ 10485.76s in FR1 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) for FR1 | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 1.28 (1) | 1 | N1*2 |
|  | 0.64 | ≥ 2.56 (2) |  | N1*2 |
|  | 1.28 | ≥ 2.56 (2) |  | N1*2 |
|  | 2.56 | ≥ 5.12 (4) |  | N1*2 |

Define Nserv for 20.48 ≤ eDRX_IDLE_cycle ≤ 10485.76s in FR2 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) for FR2 | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 5.12 (4) | 8 | N1*2 |
|  | 0.64 | ≥ 6.4 (5) | 5 | N1*2 |
|  | 1.28 | ≥ 10.24 (8) | 4 | N1*2 |
|  | 2.56 | ≥ 15.36 (12) | 3 | N1*2 |

Define Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra when eDRX_IDLE_cycle ≤ 10.24 in FR1 as:

| eDRX cycle length [s] | Scaling Factor (N1) FR1 | Tdetect,NR_Intra [s] (number of eDRX cycles) | Tmeasure,NR_Intra [s] (number of eDRX cycles) | Tevaluate,NR_Intra [s] (number of eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 | 1 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 |  | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 |  | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |

Define Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra when when eDRX_IDLE_cycle ≤ 10.24 in FR2 as:

| eDRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of eDRX cycles) | Tmeasure,NR_Intra [s] (number of eDRX cycles) | Tevaluate,NR_Intra [s] (number of eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 |  | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 |  | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |

Define Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra  when 20.48 ≤ eDRX cycle in FR1 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | FR1 Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 1.28 (1) | 1 | eDRX_cycle_length*23PTW/DRX_cycle_length (23) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥ 1.28 (1) |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥ 2.56 (2) |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥ 5.12 (4) |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Define Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra  when 20.48 ≤ eDRX cycle in FR2 as:

| eDRX cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | FR1 Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 1.28 (1) | 8 | eDRX_cycle_length*23PTW/DRX_cycle_length (23) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥ 1.28 (1) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥ 2.56 (2) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥ 5.12 (4) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

References

- R4-2202672, “WF on eDRX and RRM measurement relaxations requirements for RedCap UE”, vivo

- TR 38.840, Study on User Equipment power saving in NR, Release 16