3GPP TSG-RAN WG4 Meeting # 102-e										R4-2204561

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.5.3.2

Source: 	CMCC

Title: 	Discussion on repeater emission related radiated requirements

Document for:	Discussion

1. Introduction

In last RAN4 #101 bis e-meeting, a new WF on FR1 NR repeaters RF requirement is approved. Remaining issues include

- Channel bandwidth for UL ACLR

- Inside OBUE

In this contribution, we focus on remaining emission requirements for FR2.

2. Discussion

## 2.1 Channel bandwidth for UL ACLR

In last meeting, the channel bandwidth for repeater UL ACLR/CACLR is still FFS to give time for further check. Some options are proposed like min (400MHz, passband bandwidth) is to consider the scenario when repeater hold several consecutive carries. However, the passband width may not be comprised by multiple 400MHz carriers, instead it may comprise several carriers with different carrier BW, e.g. 400MHz+100MHz. When we test ACLR for such repeater, we use the same bandwidth as the lowest or highest carrier at the edge of repeater’s passband. So we need to update bandwidth define as:

Min (BW of the highest or lowest carrier in the edge of passband, passband bandwidth)

Fig 1. The schematic of ACLR for repeater hold several consecutive carriers with different BW

Proposal 1: the channel bandwidth for FR2 UL ACLR/CACLR is suggested as Min (BW of the highest or lowest carrier in the edge of passband, passband bandwidth).

## 2.2 inside OBUE

The agreement of inside OBUE in last meeting is listed as below:

For the OBUE requirement For UL side, adjacent PRBs (ref. UE in-band emission) For DL side, adjacent carriers (ref. absolute ACLR) The actual limit in dBm, both for UL and DL is still FFS

At first we discuss the DL inside OBUE requirements.

The candidate requirements include OBUE, equivalent relative ACLR in absolute limit, absolute ACLR. The comparison of these three requirements is listed as below.

Table 1. The comparison of OBUE, relative ACRL and absolute ACLR for category B at 24.25-33.4GHz

| Prated,t,TRP assuming 400MHz BW | OBUE at frequency offset 0.1*BWcontiguous f < fB dBm/MHz | Equivalent relative ACLR dBm/MHz | Absolute ACLR dBm/MHz |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| <23 | -20 | P-26(400M)-28(ACLR)<-20 | -13 | -20 | -20 |
| 23~30 | -13>P-43>-20 | P-26-28(ACLR)=P-54<-20 | -13 | -20 | -20 |
| 30~34 | -13 | P-26-28(ACLR)=P-54<-20 | -13 | -20 | -20 |
| 34~41 | -13 | -13>P-26-28(ACLR)=P-54>-20 | -13 | -20 | -20 |
| >41 | -13 | P-26-28(ACLR)=P-54>-13 | -13 | -20 | -20 |

Table 2. The comparison of OBUE, relative ACRL and absolute ACLR for category B at 37-52.6GHz

| Prated,t,TRP assuming 400MHz BW | OBUE at frequency offset 0.1*BWcontiguous f < fB dBm/MHz | Equivalent relative ACLR dBm/MHz | Absolute ACLR dBm/MHz |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| <21 | -20 | P-26(400M)-26(ACLR)<-20 | -13 | -20 | -20 |
| 21~28 | -13>P-41>-20 | P-26-26(ACLR)=P-52<-20 | -13 | -20 | -20 |
| 28~32 | -13 | P-26-26(ACLR)=P-52<-20 | -13 | -20 | -20 |
| 32~39 | -13 | -13>P-26-26(ACLR)=P-52>-20 | -13 | -20 | -20 |
| >39 | -13 | P-26-26(ACLR)=P-52>-13 | -13 | -20 | -20 |

In theory, we should define the limits that are more stringent than OBUE but the more relax one between absolute ACLR and relative ACLR.

Observation 1: the principle to define inside OBUE is to choose the more stringent limit between gNB OBUE and ACLR. Here the ACLR is the more relax one between relative ACLR and absolute ACLR.

Therefore, the inside OBUE is defined as below:

Table 3: inside OBUE limits for 24.25-33.4GHz

| Prated,t,TRP assuming 400MHz BW | Inside OBUE dBm/MHz |  |  |
| --- | --- | --- | --- |
|  | WA | MR | LA |
| <23 | -20 | -20 | -20 |
| 23~30 | P-43 | -20 | -20 |
| 30~34 | -13 | -20 | -20 |
| 34~41 | -13 | -13 | -13 |
| >41 | -13 | -13 | -13 |

Table 4: inside OBUE limits for 37-52.6GHz

| Prated,t,TRP assuming 400MHz BW | Inside OBUE dBm/MHz |  |  |
| --- | --- | --- | --- |
|  | WA | MR | LA |
| <21 | -20 | -20 | -20 |
| 21~28 | P-41 | -20 | -20 |
| 28~32 | -13 | -20 | -20 |
| 32~39 | -13 | -13 | -13 |
| >39 | -13 | -13 | -13 |

Proposal 2: it’s suggested to define inside OBUE as in above table 3 and table 4 for DL.

For UL, it’s suggested to apply the same approach as conducted part.

Proposal 3: it’s suggested to reuse the same approach to define inside OBUE as conducted part for UL.

3. Conclusions

In this contribution, NR repeater emission related radiated requirements are discussed with following observation and proposals:

Proposal 1: the channel bandwidth for FR2 UL ACLR/CACLR is suggested as Min (BW of the highest or lowest carrier in the edge of passband, passband bandwidth).

Observation 1: the principle to define inside OBUE is to choose the more stringent limit between gNB OBUE and ACLR. Here the ACLR is the more relax one between relative ACLR and absolute ACLR.

Table 3: inside OBUE limits for 24.25-33.4GHz

| Prated,t,TRP assuming 400MHz BW | Inside OBUE dBm/MHz |  |  |
| --- | --- | --- | --- |
|  | WA | MR | LA |
| <23 | -20 | -20 | -20 |
| 23~30 | P-43 | -20 | -20 |
| 30~34 | -13 | -20 | -20 |
| 34~41 | -13 | -13 | -13 |
| >41 | -13 | -13 | -13 |

Table 4: inside OBUE limits for 37-52.6GHz

| Prated,t,TRP assuming 400MHz BW | Inside OBUE dBm/MHz |  |  |
| --- | --- | --- | --- |
|  | WA | MR | LA |
| <21 | -20 | -20 | -20 |
| 21~28 | P-41 | -20 | -20 |
| 28~32 | -13 | -20 | -20 |
| 32~39 | -13 | -13 | -13 |
| >39 | -13 | -13 | -13 |

Proposal 2: it’s suggested to define inside OBUE as in above table 3 and table 4 for DL.

Proposal 3: it’s suggested to reuse the same approach to define inside OBUE as conducted part for UL.

4. Reference