3GPP TSG-RAN WG4 Meeting #102-e	R4-2204584

Electronic Meeting, 21 February – 3 2022

Agenda Item:	10.16.10.2

Source:	Ericsson

Title:	UE demodulation and CSI reporting requirements for FR2-2

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on UE demodulation and CSI reporting requirements for FR2-2. The general related aspects have been gathered in a separate paper [1].

# 2	Prior agreements on UE demodulation and CSI reporting requirements

In this section, we refer to RAN1 simulation parameter and RAN4 RF agreements [5, 6] to build on our view on UE demodulation and CSI requirements in 52.5 GHz – 71 GHz band.

- Applicable numerology

  - SCS = 120 KHz, min CBW = 100 MHz and max CBW = 400 MHz

  - SCS = 480 KHz [optional], min CBW = 400 MHz and max CBW = 1600 MHz

  - SCS = 960 KHz [optional], min CBW = 400 MHz and max CBW = 2000 MHz

- Maximum number of DL MIMO layers

  - A UE with 2Rx branches and 1 DL MIMO layers is supported.

  - A UE with 2Rx branches and 2 DL MIMO layers is FFS.

- Maximum modulation order

  - Up to a 64QAM modulation is supported.

  - [MCS = 22] is optional

- Inter-band NR DC/CA combinations

  - RAN4 agreed to support FR1+FR2-2 DC/CA combinations in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2, with an anchor in FR1 [2].

Table 2-1: Band combinations for inter-band CA between FR1 and FR2-2

| NR CA Band | NR Band |
| --- | --- |
| CA_n41-n263 | n41, n263 |
| CA_n77-n263 | n77, n263 |
| CA_n79-n263 | n79, n263 |

Further combinations considering band [n264] is FFS for FR2-2.

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements with:

- Number of receive antennas: 2Rx

- Modulation order: Up to 64QAM

- Both single carrier (FR2-2) and NR-DC FR1 + FR2-2 scenarios

Proposal 2: RAN4 defines the UE demodulation requirements with:

- FR2-2 TDD: SCS = 120 kHz with min CBW = 100 MHz and Max CBW = 400 MHz

- FR2-2 TDD: SCS = 480 kHz with min CBW = 400 MHz and Max CBW = 1600 MHz

# 3	UE Demodulation Requirements

## 3.1	Scope of requirements

It has been stated in [3] that, it is recommended to strive for maximum commonality for the system design for licensed and unlicensed operation for FR2-2 band, and maximize the re-use of the existing NR design. However, RAN4 needs to accommodate the demodulation requirements for PDSCH, PDCCH, and PBCH, with the FR2-2 propagation environments and RF constraints.

In[1], we can refer to the low delay spread in such an environment where new channel models should be proposed accordingly. Furthermore, while the limited use cases’ mobility, thus the Doppler spread, could not be the highest issue for 120 KHz SCS, phase noise (PN) should be investigated and PN models should be proposed.

## 3.2 Phase Noise Characteristics and Reference Signals

RAN4 was not able to agree on a single representative phase noise model during the study item. However, we believe that the parametrized phase noise model in [3] (set 1) and [4], described in Table 3.2-1, can be used for tests. It is worth reminding that the design margin (DM) should be set to 0 dB for BS while to be set to 5 dB for UE.

Table 3.2-1: Parameters of phase noise model equation with design margin

| Parameter | Value/expression | Parameter | Value |
| --- | --- | --- | --- |
| PSD0 | fc3.55e92∙10DM/10 |  |  |
| fz,1 | 1.6e3 | αz,1 | 2 |
| fz,2 | 200e3 | αz,2 | 1 |
| fz,3 | fc44.8 | αz,3 | 2 |
| fp,1 | 1 | αp,1 | 3 |
| fp,2 | 1e6 | αp,2 | 2 |

In [3] (set 2), a second phase noise model referred to as set 2 has been proposed as well for evaluation.

Furthermore, the PTRS enhancement has been raised in RAN1 to help the ICI compensation at the receiver, and the assessment results led to the Observation 1 and 2.

Observation 1: In Rel-17, NR operation in FR2-2 will only support Rel-15 patterns for CP-OFDM.

Proposal 3: Consider the parametrized phase noise model in [4] and use Rel-15 PTRS pattern for tests.

## 3.3	PDSCH demodulation requirements

In [3], it has been recommended for an operating system in FR2-2 band that the maximum FFT size and the maximum RBs used per carrier should be limited to 4096 and 275 RBs, respectively.

In Rel-15/16, RAN4 has defined many PDSCH demodulation requirements considering the RAN1 physical layer specification. For FR2-2, including performance results considering PN and new channel models are of great significance.

Proposal 4: Define PDSCH demodulation requirements for UE with the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDL model as defined in of TR38.901 Clause 7.7.2: - TDL-A (5ns, 10ns, 30 ns DS) |
| Antenna configuration | For TDL model: 2x2 ULA Low |
| Velocity | 3 km/h, 30 km/h |
| PA Model | None |
| gNB TRP PN Model | TR38.803 example 2 BS PN profile and [3] Set 1 |
| UE PN Model | TR38.803 example 2 UE PN profile and [3] Set 1 |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 (Rank 2 is FFS) |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): - MCS 7 (QPSK), - MCS 16 (16QAM), - [MCS 22] (64QAM). Note: It is assumed that NohPRB = 0 for MCS calculations. |

FR2-2 TDD, SCS 120 KHz

For the sake of compatibility with FR2-1, it is recommended to keep same PDSCH configurations for SCS 120 KHz in FR2-2. Below is our proposed test setup for FR2-2 TDD for SCS 120 KHz. However, we will highlight the needed updates/modifications

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 (Note 1) | 100MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-1 |
| 1-2 (Note 1) | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-3 |
| 1-3 Note 1) | 100MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-6 |
| 2-1 (Note 1) | 400MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-1 |
| 2-2 (Note 1) | 400MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-3 |
| 2-3 (Note 1) | 400MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-6 |
| 3-1 | TBD / 120kHz | QPSK 0.3 Rank 2 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-1 |
| 3-2 | TBD / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-3 |
| Note 1 Tests (1,2)-1, (1,2)-2, and (1,2)-3 will consider Rank 1 instead of Rank 2 as stated in TS 38.101-4 Section 7.2.2.2.1. |  |  |  |  |  |  |  |

The detailed simulation parameters are provided in Appendix A.1.

FR2-2 TDD, SCS 480 KHz

The PDSCH configurations for SCS 480 KHz should be defined in FR2-2. Below is our proposed test setup for FR2-2 TDD for SCS 480 KHz.

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 400MHz / 480kHz | QPSK 0.3 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | New |
| 1-2 | 400MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | New |
| 1-3 | 400MHz / 480kHz | 64QAM 0.43 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | New |
| 2-1 | 1600MHz / 480kHz | QPSK 0.3 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | New |
| 2-2 | 1600MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | New |
| 2-3 | 1600MHz / 480kHz | 64QAM 0.43 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | New |

## 3.4	PDCCH demodulation

We propose to define PDCCH with all the possible aggregation levels 2, 4, 8, and 16 by reusing the existing test case specified in TS38.101-4 Section 7.3.2.2 for SCS 120 KHz in FR2-2.

Proposal 5: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

FR2-2 TDD, SCS 120 KHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna config | Metric (Pm-dsg) | Reference from TS38.101-4 7.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100 | 60 | 1 | 2 | TDL-A (5, 10, 30) ns | 1x2 low | 1% | 1Tx Test 1 |
| 1-2 | 100 | 60 | 1 | 4 | TDL-A (5, 10, 30) ns | 1x2 low | 1% | 1Tx Test 2 |
| 1-3 | 100 | 60 | 1 | 8 | TDL-A (5, 10, 30) ns | 2x2 low | 1% | 2Tx Test 1 |
| 1-4 | 100 | 60 | 2 | 16 | TDL-A (5, 10, 30) ns | 2x2 low | 1% | 2Tx Test 2 |
| 2-1 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 1x2 low | 1% | New |
| 2-2 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 2x2 low | 1% | New |

The detailed simulation parameters are provided in Appendix A.2.

FR2-2 TDD, SCS 480 KHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna config | Metric (Pm-dsg) | Reference from TS38.101-4 7.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | New |
| 1-2 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 2x2 Low | 1% | New |
| 2-1 | 1600 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | New |
| 2-2 | 1600 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 2x2 Low | 1% | New |

## 3.5	PBCH demodulation

RAN4 defines the PBCH demodulation requirements (i.e., MIB decoding performance) in Rel-15 and we propose to extend the configurations in FR2-2. As an initial starting point, we can consider the SCS 120 KHz as an extension to FR2-1 detailed in TS38.101-4, Table 7.4.2.2-2 and Table 7.4.2.2-3, whether the SS/PBCH block index is known or not, respectively.

Proposal 6: Define PBCH demodulation requirements with the following test setup

| Test number | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | Reference from TS38.101-4 SS/PBCH block index A : not known B : known |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 100 MHz / 120 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | A |
| 1-2 | 400 MHz / 120 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | B |
| 2-1 (Note 1) | 400 MHz / 480 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | A |
| 2-2 (Note 1) | 1600 MHz / 480 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | B |
| Note 1: SSB SCS 480 KHz is new. |  |  |  |  |  |

The detailed simulation parameters are provided in Appendix A.3.

## 3.6	SDR

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs such as rank 1/2 and QPSK/16QAM/64QAM. RAN4 only need to evaluate the applicability rule for FR2-2 where rank 1 is supported as shown in Table 3.6-1.

Table 3.6-1	MCS indexes for indicated UE capabilities applicable for FR2-2.

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 0.75 | [22] |
|  | 1 | 6 | 0.4 | 14 |
|  | 1 | 4 | 1 | 16 |
|  | 1 | 4 | 0.8 | 16 |
|  | 1 | 4 | 0.75 | 16 |
|  | 1 | 4 | 0.4 | 10 |
|  | 1 | 2 | 1 | 9 |
|  | 1 | 2 | 0.8 | 9 |
|  | 1 | 2 | 0.75 | 9 |
|  | 1 | 2 | 0.4 | 4 |

Proposal 7: Define SDR test for UE in FR2-2 considering 2Rx UE.

# 4	CSI reporting requirements

RAN4 has defined CQI, PMI and RI reporting requirements in Rel-15. For Rel-17 and FR2-2 band, RAN1 simulation parameters’ agreement has been to consider PDSCH MIMO rank equal to 1. Consequently, we propose to define CQI reporting requirements only, since we do not need to verify RI. Furthermore, the PMI reporting’s ports could be FFS.

Regarding the CQI reporting, we propose to define both the static condition and fading condition requirements for UE with 2Rx. Since the existing CQI reporting tests verifies the CQI using 256 QAM for PDSCH FR2-1 (see 38.101-4 Table 8.1.1.3-1), we need to define the CQI reporting requirements using 64QAM for 2Rx UE.

Proposal 8: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

For the CQI reporting under fading condition, we should evaluate whether the maximum CBW of 100 MHz for FR2-2 is enough to define a wideband CQI reporting test.

Proposal 9: Define the wideband CQI reporting under fading condition for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

# 5	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.2 | PDSCH demodulation requirements |  |
| 7.2.1 | 1RX requirements |  |
| 7.2.2 | 2RX requirements |  |
| 7.2.2.2 | TDD |  |
| 7.2.2.2.[4] | Minimum requirements for UEs in FR2-2 | New section |
| 7.3 | PDCCH demodulation requirements |  |
| 7.3.1 | 1RX requirements |  |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.1 | 1 Tx Antenna performances | Updates on FR2-2 |
| 7.3.2.2.2 | 2 Tx Antenna performances | Updates on FR2-2 |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Updates on FR2-2 UEs |

Table 2	Specification structure for UE CSI reporting requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 8.2.2 | 2RX requirements |  |
| 8.2.2.2 | TDD |  |
| 8.2.2.2.1 | CQI reporting definition under AWGN | Updates on FR2-2 |
| 8.2.2.2.2 | CQI reporting under fading conditions | Updates on FR2-2 |

# 6	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements with:

- Number of receive antennas: 2Rx

- Modulation order: Up to 64QAM

- Both single carrier (FR2-2) and NR-DC FR1 + FR2-2 scenarios

Proposal 2: RAN4 defines the UE demodulation requirements with:

- FR2-2 TDD: SCS = 120 KHz with min CBW = 100 MHz and Max CBW = 400 MHz

- FR2-2 TDD: SCS = 480 KHz with min CBW = 400 MHz and Max CBW = 1600 MHz

Observation 1: In Rel-17, NR operation in FR2-2 will only support Rel-15 PTRS patterns for CP-OFDM.

Proposal 3: Consider the parametrized phase noise model in [4] and use Rel-15 PTRS pattern for tests.

Proposal 4: Define PDSCH demodulation requirements for UE with the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz optional |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDL model as defined in of TR38.901 Clause 7.7.2: - TDL-A (5ns, 10ns, 30 ns DS) |
| Antenna Configuration | For TDL model: 2x2 ULA Low |
| Mobility | 3 km/h, 30 km/h |
| PA Model | None |
| gNB TRP PN Model | TR38.803 example 2 BS PN profile and [3] Set 1 |
| UE PN Model | TR38.803 example 2 UE PN profile and [3] Set 1 |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 (Rank 2 is FFS) |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| CSI-RS / TRS | CSI-RS/TRS is assumed to be off (for RS overhead) |
| MCS/TBS | From MCS Table 1 (TS38.214): - MCS 7 (QPSK), - MCS 16 (16QAM), - [MCS 22] (64QAM). Note: It is assumed that NohPRB = 0 for MCS calculations. |

FR2-2 TDD, SCS 120 KHz

For the sake of compatibility with FR2-1, it is recommended to keep same PDSCH configurations for SCS 120 KHz in FR2-2. Below is our proposed test setup for FR2-2 TDD for SCS 120 KHz. However, we will highlight the modifications

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 (Note 1) | 100MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-1 |
| 1-2 (Note 1) | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-3 |
| 1-3 (Note 1) | 100MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-6 |
| 2-1 (Note 1) | 400MHz / 120kHz | QPSK 0.3 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-1 |
| 2-2 (Note 1) | 400MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-3 |
| 2-3 (Note 1) | 400MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-6 |
| 3-1 | TBD / 120kHz | QPSK 0.3 Rank 2 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-1 |
| 3-2 | TBD / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-2 | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | Test 2-3 |
| Note 1 Tests (1,2)-1, (1,2)-2, and (1,2)-3 will consider Rank 1 instead of Rank 2 as stated in TS 38.101-4 Section 7.2.2.2.1. |  |  |  |  |  |  |  |

The detailed simulation parameters are provided in Appendix A.1.

FR2-2 TDD, SCS 480 KHz

The PDSCH configurations for SCS 480 KHz should be defined in FR2-2. Below is our proposed test setup for FR2-2 TDD for SCS 480 KHz.

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 400MHz / 480kHz | QPSK 0.3 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | To be added |
| 1-2 | 400MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | To be added |
| 1-3 | 400MHz / 480kHz | 64QAM 0.43 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | To be added |
| 2-1 | 1600MHz / 480kHz | QPSK 0.3 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | To be added |
| 2-2 | 1600MHz / 480kHz | 16QAM 0.48 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | To be added |
| 2-3 | 1600MHz / 480kHz | 64QAM 0.43 Rank 1 | TBD | TDL-A (5, 10, 30) ns | 2x2 ULA Low | 70% of peak rate | To be added |

Proposal 5: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

FR2-2 TDD, SCS 120 KHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna config | Metric (Pm-dsg) | Reference from TS38.101-4 7.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100 | 60 | 1 | 2 | TDL-A (5, 10, 30) ns | 1x2 low | 1% | 1Tx Test 1 |
| 1-2 | 100 | 60 | 1 | 4 | TDL-A (5, 10, 30) ns | 1x2 low | 1% | 1Tx Test 2 |
| 1-3 | 100 | 60 | 1 | 8 | TDL-A (5, 10, 30) ns | 2x2 low | 1% | 2Tx Test 1 |
| 1-4 | 100 | 60 | 2 | 16 | TDL-A (5, 10, 30) ns | 2x2 low | 1% | 2Tx Test 2 |
| 2-1 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 1x2 low | 1% | New |
| 2-2 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 2x2 low | 1% | New |

The detailed simulation parameters are provided in Appendix A.2.

FR2-2 TDD, SCS 480 KHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna config | Metric (Pm-dsg) | Reference from TS38.101-4 7.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | New |
| 1-2 | 400 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 2x2 Low | 1% | New |
| 2-1 | 1600 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | New |
| 2-2 | 1600 | TBD | TBD | TBD | TDL-A (5, 10, 30) ns | 2x2 Low | 1% | New |

Proposal 6: Define PBCH demodulation requirements with the following test setup

| Test number | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | Reference from TS38.101-4 SS/PBCH block index A : not known B : known |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 100 MHz / 120 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | A |
| 1-2 | 400 MHz / 120 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | B |
| 2-1 (Note 1) | 400 MHz / 480 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | A |
| 2-2 (Note 1) | 1600 MHz / 480 KHz | TDL-A (5, 10, 30) ns | 1x2 Low | 1% | B |
| Note 1: SSB SCS 480 KHz is new. |  |  |  |  |  |

The detailed simulation parameters are provided in Appendix A.3.

Proposal 7: Define SDR test for UE in FR2-2 considering 2Rx UE.

Proposal 8: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

Proposal 9: Define the wideband CQI reporting under fading condition for 2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

# References

- R4-2204031, “General view on demodulation requirements for NR extended to 71 GHz”, Ericsson.

- R4-2202410, “Draft CR to add FR1+FR2-2 DC/CA combinations”, Ericsson.

- 3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”, (Release 17).

- R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson.

- R4-2202365, “WF on general aspects and system parameters of FR2-2”, Intel.

- R4-2202364, “Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2”, Vivo.

# Appendix	Simulation assumption

## A.1	PDSCH

### FR2-1

Please refer to 38.101-4 Table 7.5A.1-1

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| SCS |  | kHz | 120 |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 for DL slots 9 for special slots |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

## A.2	PDCCH

### FR2-1

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1 6 for test 2 | 2 |
| Interleaver size |  | 3 for test 1 2 for test 2 | 3 |
| Shift index |  | 0 |  |

Also refer to TS38.101-4 Table 7.3.2.2-1

## A.3	PBCH

## FR2-1

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | FR2.120-1 |
| Note 1: as specified in clause 4.1 of TS 38.213 [11] Note 2: as specified in clause 11.1 of TS 38.213 [11] |  |  |

See 38.101-4 Table 7.4.2.2-1

## A.4	CQI reporting in static condition

### A.4.1	Test metric

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

| For FR2-2 Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |
| Subcarrier spacing |  |  | kHz | 120 |
| Duplex Mode |  |  |  | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |
| SNRBB |  |  | dB | TBD |
| Propagation channel |  |  |  | AWGN |
| Antenna configuration |  |  |  | 2x2 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | fd-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |
| ReportConfigType |  |  |  | Periodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |
| aperiodicTriggeringOffset |  |  |  | Not configured |
| Codebook configuration |  | Codebook Type | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8.375 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| Measurement channel |  |  |  | TBD |

## A.5	CQI reporting in fading condition

### A.5.1	Test metric

- A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time.

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ.

- When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to [0.02].

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |
| Subcarrier spacing |  |  | kHz | 120 |
| Duplex Mode |  |  |  | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |
| SNRBB |  |  | dB | TBD |
| Propagation channel |  |  |  | TDL-A (5,10,30) ns |
| Antenna configuration |  |  |  | 2x2 ULA High |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | fd-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | Not configured |
|  | aperiodicTriggeringOffset |  |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 6 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 1.375 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| Measurement channel |  |  |  | TBD |