3GPP RAN WG4 Meeting #102e		R4-2204590

Electronic Meeting, Feb 21-Mar 3, 2022

Agenda item:	10.16.3.1

Source:	Murata Manufacturing Co., Ltd.

Title:	Views on FR2-2 FWA UE

Document for:	Discussion

1	Introduction

In RAN4 #101-bis-e, several topics regarding FR2-2 FWA were discussed and agreed to investigate the number of antenna element and RF requirements firstly [1]. In order to investigate the number of antenna elements, we need to consider not only device characteristics but also the required communication range for FR2-2 FWA and its use case. This contribution shows our views on the number of antenna elements and RF requirements for FR2-2 FWA UEs.

2	Discussion

In order to investigate the number of antenna elements and RF requirements, we need to consider network performance as well as device characteristics point of view. In TR38.808, the deployment scenarios include indoor hotspot, dense urban, urban micro, urban macro, rural, factor hall, and indoor D2D scenarios, and its ISD are described. Among them, ISD of Dense urban scenario (outdoor A and C) is the largest and it is 150m. So the required communication range is 75.5m considering the BS and UE height.

Observation 1:　 Considering the deployment scenarios, communication range between UE and BS should be larger than 75.5m.

In RAN4 #101-bis-e, several companies proposed the value of min peak EIRP [2][3]. Table 1 shows the communication range which is calculated by UMi pathloss equation of TR38.901. In Table 1, UE EIRP is the proposed values in RAN4 #101-bis-e and BS REFSENS is the average sensitivity of Wide Area BS in TS38.104. As to it, the communication range is less than 75.5m, so there is the possibility that the use case similar to Dense urban scenario cannot be supported.

Table 1. Communication range when using proposed in [2][3].

| Band (Frequency) | BS REFSENS [dBm] | UE EIRP [dBm] | Pathloss [dB] | Communication range [m] | Calculation condition |
| --- | --- | --- | --- | --- | --- |
| n263 (70GHz) | -98.5 | [25.9 – 26.0] (R4-2201925, R4-2200238) | 124.4 | 60 | UMi NLOS |

Observation 2: 　In n263, the communication range is 60m when min peak EIRP is [25.9-26.0] dBm (UMi NLOS Scenario) , so we concern that the use case similar to Dense urban scenario cannot be supported.

Proposal 1: 　　 Before determining the number of antenna elements, RAN4 needs to discuss the required communication range and use case.

To investigate the minimum requirement, we should consider device characteristics as well. So we analysed the RF requirements and Table 2 shows Tx link budget to estimate minimum peak EIRP for band n263.

Table 2. The minimum peak EIRP evaluation for PC1 in band n263

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Frequency range | GHz | 57-71 |  |
| Pout per element | dBm | 5 | 5 |
| # of antennas in an array |  | 32 | 64 |
| Total conducted power per polarization | dBm | 20 | 23 |
| Avg antenna element gain | dBi | 4 | 4 |
| Antenna roll-off loss versus frequency | dB | 2.5 | 2.5 |
| Realized antenna array gain | dBi | 16.5 | 19.5 |
| Polarization gain | dB | 2.5 | 2.5 |
| Mismatch and transmission line loss including load pull | dB | 4 | 4 |
| Beam forming loss (phase shifter and amplitude error) | dB | 1 | 1 |
| Finite beam table | dB | 0.4 | 0.5 |
| Beam forming loss (one beam table fits all) | dB | 0.5 | 0.5 |
| Form factor integration losses | dB | 7 | 7 |
| Total implementation loss (worst case) | dB | 12.9 | 13 |
| Peak EIRP(Minimum) | dBm | 26.2 | 32.1 |

According to the above table, min peak EIRP of 32 antenna element UE is 26.2dBm, and its communication range is 60m. It is less than the half of ISD. On the other hand, min peak EIRP of 64 antenna element UE is 32.1 dBm and its communication range is 90m. This is enough for Dense urban use case. So we believe 64 antenna element would be preferred for FWA UE in n263.

Observation 3:　 It is difficult for 32 antenna element UE to support Dense urban use case. To support this scenario UE seems to need 64 antenna elements.

Proposal 2:　    RAN4 uses 64 antenna elements for FWA discussion as baseline.

Proposal 3:　    Min peak EIRP of PC1 is 32.1 dBm in n263.

3	Conclusions

This contribution provides our view on the communication range and the min peak EIRP for PC1 in band n263.

Observation 1:　 Considering the deployment scenarios, communication range between UE and BS should be larger than 75.5m.

Observation 2: 　In n263, the communication range is 60m when min peak EIRP is [25.9-26.0] dBm (UMi NLOS Scenario) , so we concern that the use case similar to Dense urban scenario cannot be supported.

Proposal 1: 　　 Before determining the number of antenna elements, RAN4 needs to discuss the required communication range and use case.

Observation 3:　 It is difficult for 32 antenna element UE to support Dense urban use case. To support this scenario UE seems to need 64 antenna elements.

Proposal 2:　    RAN4 uses 64 antenna elements for FWA discussion as baseline.

Proposal 3:　    Min peak EIRP of PC1 is 32.1 dBm in n263.

4	References

R2-2202328, “Email discussion summary for [101-e][128] NR_ext_to_71GHz_Part_2,” Qualcomm Incorporated.

R4-2201925, “UE Tx requirements for 52.6-71GHz,” Intel Corporation.

R4-2200238, “60GHz UE TX,” Qualcomm Incorporated.