3GPP TSG RAN WG4 Meeting #102-e	R4-2204648

Electronic Meeting, Feb. 21- Mar. 3, 2022

Agenda item:			10.23.2.1

Source:	vivo

Title:	Simulation results for TRS based PDC

Document for:	Information

- Introduction

In this contribution, we provide simulation results for RTT-based PDC with TRS with simulation assumptions provided in the WF [1].

# Discussion

Simulation results for TRS based PDC are provided in Table 1.

Table 1. simulation results for TRS based PDC

| SCS(kHz) | BW(RB) | Sample rate (Tc) | AWGN |  | TDL-A |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 sample | 4 samples | 1 sample | 4 samples |
| 15kHz | 24 | 256 | 139.8 | 134.5 | 350.8 | 198.8 |
|  | 52 | 128 | 65.8 | 63.8 | 193.5 | 118.3 |
|  | 104 | 64 | 31.8 | 31.0 | 127.5 | 99.0 |
|  | 268 | 32 | 15.0 | 14.9 | 108.0 | 48.6 |
| 30kHz | 24 | 128 | 69.7 | 65.2 | 197.0 | 113.9 |
|  | 52 | 64 | 32.9 | 31.2 | 129.2 | 91.3 |
|  | 132 | 32 | 15.4 | 15.0 | 107.1 | 48.3 |
|  | 272 | 16 | 7.5 | 7.4 | 42.3 | 38.4 |

For RTT-based PDC with TRS, it can be seen from simulation results in Table 1 that measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel. Improvement on measurement accuracy can be seen under TDL-A fading channel.

Observation 1: Measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel.

Observation 2: Measurement accuracy can be improved with 4 samples compared to 1 sample measurement under TDL-A fading channel.

# Summary

Following observations are made based on simulation results.

Observation 1: Measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel.

Observation 2: Measurement accuracy can be improved with 4 samples compared to 1 sample measurement under TDL-A fading channel.

- Appendix: Simulation assumptions

Simulation assumptions are provided in the WF [1].

Table 1 General parameters

| Parameter | Value |
| --- | --- |
| Cell layout | 1 serving cell |
| Duplex modes | FDD and TDD |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) |
| Data and CCH load in non-TRS symbols | - 50% utilization in time - 100% RE utilization |
| Cyclic prefix | Normal |
| DRX | OFF |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD |
| Propagation conditions [TS 38.101-4] | AWGN, [TDL-A (30 ns delay spread, 5Hz)] |
| Es/Iot [dB] | -3 |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |
| UE measurement bandwidth | Full carrier bandwidth |

Table 2 CSI-RS for tracking transmission configuration parameters

| Parameter | Value |
| --- | --- |
| Number of transmit TRS antennas | 1 |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |
| Number with CSI-RS Resource sets with trs-Info | 1 |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: 24(5MHz), 52 (20MHz),132 (50MHz), 272 (100MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(5MHz), 64 (20MHz), 32 (50MHz) |
| TRS periodicity | 40ms |
| TRS resources number | 2, 4 |
| TRS symbol location | {4,8} |
| TRS frequency density | 3 |
| TRS samples | 1, 4 |
| Evaluation period | 40 ms, 200 ms |

At least the following performance characteristics are to be provided for TUE-RX:

- 90%-ile of the TUE-RX errors for each cell

and,

- TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).

- References

- R4-2202782	WF on NR_IIOT_URLLC_enh RRM requirements, Nokia, Nokia Shanghai Bell