3GPP TSG-RAN4 Meeting #102- e	R4-2204734

Electronic Meeting, January 17-25, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 					UE RF requirements for the sum method

Agenda Item: 			9.35.3

Document for:	Approval

# 1	Introduction

This contribution shares concrete examples of the requirements for PC2+PC3 based on the sum method.

# 2	UE RF requirements

## 2.1	The sum method

The sum method here is defined as follows.

Proposal 1: The sum method is defined as follows.

- If a UE supporting the sum method capability, e.g., fullPowerUL-CA, for a CA band configuration also reports PC2 capability.

  - This is needed for legacy networks which do not understand fullPowerUL-CA

- If the UE indicates fullPowerUL-CA, PPowerClass,CA in PCMAX_H is replaced with 10*log10∑pPowerClass,c while PPowerClass,CA in PCMAX_L is NOT replaced with the 10*log10∑pPowerClass,c which means PPowerClass,CA is PC2 in PCMAX_L

- If the UE reports the additional PC(s) of each band within the UE capabilities of an UL CA band configuration and that is understood to override any per-band PC signalling that can limit the power

  - This address a case that a PC per band is not the same as PC within a band combination like TxD.

## 2.2	Maximum Output Power (MOP)

Since we still believe that in order to make a UE with fullPowerUL-CA to deliver higher power than a UE with a traditional PC2 only, we propose to tighten the lower tolerance a little bit. And it would be feasible assuming that one Tx can deliver 21 dBm and the other can deliver 23 dBm.

Proposal 2: If PCMAX_L is not increased and if the exiting MOP tolerance is reused, the MOP would be 27.8 dBm +2/-2.8 dB

The MOP requirement would look as below. Note that we don’t need to create a new column for 27.8 dBm anymore.

Table 6.2A.1.3-1 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  | 266, 7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n79A |  |  | 266, 7 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 7: When the UE supports PC3 within one band, supports PC2 within the other band within the Uplink CA configuration and if the field of UE capability fullPowerUL-CA is present, the maximum output power is the sum of the individual bands’ power class within the UL CA configuration, where the tolerance is +2/-2.8 dB |  |  |  |  |  |  |  |  |

## 2.3	Configured Power

---------------------an example of changes for configured power with relaxation of the lower tolerance------------------------

##### 6.2A.4.1.3	Configured transmitted power for Inter-band CA

Unnecessary part is omitted

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (DtC,c),  pPowerClass.c/(MAX(mprc·∆mprc, a-mprc)·DtC,c ·DtIB,c·DtRxSRS,c) , pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.3-1 without taking into account the tolerance specified in the Table 6.2A.1.3-;

-	If the field of UE capability fullPowerUL-CA is present, PPowerClass,CA in PCMAX_H is replaced with 10log10∑ pPowerClass,c. In case IE e.g., powerClassNRPart-r17 as defined in TS 38.331 [9] is indicated, PPowerClass, CA should use that value for calculating 10log10∑ pPowerClass,c instead

-	pPowerClass,c is the linear value of the maximum UE power for serving cell c specified in Table 6.2.1-1 without taking into account the tolerance;

Unnecessary part is omitted

# 3	Conclusion

This contribution shared specific examples of the requirements for MOP and configured power with the sum method. The contribution also proposes the followings.

Proposal 1: The sum method is defined as follows.

- If a UE supporting the sum method capability, e.g., fullPowerUL-CA, for a CA band configuration also reports PC2 capability.

  - This is needed for legacy networks which do not understand fullPowerUL-CA

- If the UE indicates fullPowerUL-CA, PPowerClass,CA in PCMAX_H is replaced with 10*log10∑pPowerClass,c while PPowerClass,CA in PCMAX_L is NOT replaced with the 10*log10∑pPowerClass,c which means PPowerClass,CA is PC2 in PCMAX_L

- If the UE reports the additional PC(s) of each band within the UE capabilities of an UL CA band configuration and that is understood to override any per-band PC signalling that can limit the power

  - This address a case that a PC per band is not the same as PC within a band combination like TxD.

Proposal 2: If PCMAX_L is not increased and if the exiting MOP tolerance is reused, the MOP would be 27.8 dBm +2/-2.8 dB