3GPP TSG-RAN WG4 Meeting # 102-e                                                        R4-2204746			R4-22xxxxx

# Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-11-21:  DC_1A-7A-38A_n3A-n78A

Agenda Item:	9.19.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC_1A-7A-38A_n3A-n78A for TR37.717-11-21[1] .

- Reference

- TR 37.717-11-21  Dual Connectivity (DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations, v0.7.0

- Text Proposal

----- Start of TP -----

  - 8.x		DC_1-7-38_n3-n78

    - 8.x.1	Operating bands for DC

Table 8.x.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-38_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 8.x.2	Configuration for DC

Table 8.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A | CA_1A-7A-38A | CA_n3A-n78A |

    - 8.x.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

    - 8.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-38_n3-n78 | 1 | 0.6 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-38_n3-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 38 | 0 |
|  | n3 | 0 |
|  | n78 | 0.8 |

    - 8.x.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

----- End of TP -----