3GPP TSG-RAN WG4 Meeting # 102-e								R4-2204876

Electronic Meeting, February 21- March 3, 2022

Agenda item:	10.16.8.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on interruption requirements for FR2-2

Document for:	Discussion

- Introduction

The requirements of NR operation in FR2-2 was discussed in last RAN4 meeting, and the agreements are captured in the WF [1] and draft CR are endorsed in [2]. In this paper, we further provide our views on introducing interruption requirements for CBW change for NR-DC.

- Discussion

In last meeting, it is agreed that some of agreed interruption requirements for NR-CA also apply to NR-DC, which is shown as follows:

- Following interruption requirements for NR-CA agreed in RAN4#100e apply to NR-DC case: - Interruptions at SCell addition/release (intra-band CA), - Interruptions at Scell activation/deactivation (intra-band CA) - Interruptions during measurements on deactivated SCC - Interruptions due to Active BWP switching - Interruptions at NR SRS carrier based switching

In RAN4#100e, the interruption requirements for CBW change for NR-CA was agreed [3] as follows:

Issue 3-3-7: Interruptions due to UE-specific CBW change Agreements: - Update Table 8.2.2.2.1-1 in TS 38.133 with 480/960 kHz subcarrier spacing as below: Interruption length X NR Slot; Interruption length X (slots). length (ms). 0; 1; 1. 1; 0.5; 1. 2; 0.25; 3. 3; 0.125; 5. 5; 0.03125; 17. 6; 0.015625; 33.

In RAN4#101-e, it is also discussed whether interruption requirements for CBW change can apply to NR-DC. However, somehow it is not discussed in RAN4#101-bis-e. From our understanding, it is straightforward that CBW requirements for NR-CA can also apply to NR-DC.

Proposal 1: Interruption requirements of CBW change for NR-CA can apply to NR-DC.

However, it could be observed from current spec that interruption requirements for NR-DC are missing due to some implementation issue. Thus, we have a Rel-16 CR to add the missing requirements in Rel-16 [4] a CR [5] for FR2-2.

- Conclusions

Proposal 1: Interruption requirements of CBW change for NR-CA can apply to NR-DC.

References

[1] R4-2202659 WF on NR extension to 71 GHz RRM requirements (Part 2), Intel

[2] R4-2202661 Draft CR to 38.133 Introducing interruption requirements of FR2-2 to cover 52.6-71GHz, Vivo

[3] R4-2115351 WF on R17 NR_ext_to_71GHz_RRM_1, Qualcomm

[4] R4-2205364 CR on CBW change requirements R16, Huawei, HiSilicon

[5] R4-2204877 Draft CR on interruption requirements for FR2-2, Huawei, Hisilicon