3GPP TSG-RAN4 Meeting # 102-e	R4-2204932

# Electronic Meeting, 21st Feb.- 3rd Mar., 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | <CR#> | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  | Date: |  |  | 2022-2-9 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | System parameters for FR2-2 needs to be covered in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update General, Operating bands, UE channel bandwidth and Channel/Sync Raster for FR2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not cover the system parameters for FR2-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5	Operating bands and channel arrangement

5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1. Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

The present specification covers FR2 operating bands.

5.2	Operating bands

NR is designed to operate in the FR2 operating bands defined in Table 5.2-1.

Table 5.2-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n259 | 39500 MHz | – | 43500 MHz | 39500 MHz | – | 43500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| n262 | 47200 MHz | – | 48200 MHz | 47200 MHz | – | 48200 MHz | TDD |
| n263 | 57000 MHz | – | 71000 MHz | 57000 MHz | – | 71000 MHz | TDD1 |
| NOTE 1: This band is restricted to operation with shared spectrum channel access as defined in [TS 37.213]. |  |  |  |  |  |  |  |

## 5.2A	Operating bands for CA

### 5.2A.1	Intra-band CA

NR intra-band contiguous and non-contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2, where all operating bands are within FR2.

Table 5.2A.1-1: Intra-band contiguous and non-contiguous CA operating bands in FR2-1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n257 | n257 |
| CA_n258 | n258 |
| CA_n259 | n259 |
| CA_n260 | n260 |
| CA_n261 | n261 |

Table 5.2A.1-2: Intra-band contiguous CA operating bands in FR2-2

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n263 | n263 |

5.3	UE Channel bandwidth

5.3.1	General

The UE channel bandwidth supports a single NR RF carrier in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A | N.A | N.A | N.A |
| 120 | 32 | 66 | 132 | 264 | N.A | N.A | N.A |
| 4801 | N.A | N.A | N.A | FFS | FFS | FFS | N.A |
| 9601 | N.A | N.A | N.A | FFS | FFS | FFS | FFS |
| Note 1: This SCS is optional in this release of the specification. |  |  |  |  |  |  |  |

5.3.3	Minimum guardband and transmission bandwidth configuration

In FR2-1, the minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz) in FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N. A |
| 120 | 1900 | 2420 | 4900 | 9860 |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1.

In FR2-2, the minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-2

Table 5.3.3-2: Minimum guardband for each UE channel bandwidth and SCS (kHz) in FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
| 120 | 2420 | 9860 | N.A | N. A | N. A |
| 480 | N. A | FFS | FFS | FFS | N. A |
| 960 | N. A | FFS | FFS | FFS | FFS |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-2.

The minimum guardband of receiving BS SCS 240 kHz SS/PBCH block for each UE channel bandwidth is specified in table 5.3.3-3 for FR2-1.

Table: 5.3.3-3: Minimum guardband (kHz) of SCS 240 kHz SS/PBCH block in FR2-1

| SCS (kHz) | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- |
| 240 | 3800 | 7720 | 15560 |

NOTE:	In FR2-1, the minimum guardband in Table 5.3.3-3 is applicable only when the SCS 240 kHz SS/PBCH block is received adjacent to the edge of the UE channel bandwidth within which the SS/PBCH block is located.

Figure 5.3.3-1: Void

The number of RBs configured in any channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.3-2 UE PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol due to BS transmission of SSB, the minimum guardband on each side of the carrier is the guardband applied at the configured channel bandwidth for the numerology that is transmitted immediately adjacent to the guard band.

If multiple numerologies are multiplexed in the same symbol and the UE channel bandwidth is > 200 MHz, the minimum guardband applied adjacent to 60 kHz SCS shall be the same as the minimum guardband defined for 120 kHz SCS for the same UE channel bandwidth.

Figure 5.3.3-3 Guard band definition when transmitting multiple numerologies

Note:	Figure 5.3.3-3 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

5.3.4	RB alignment

For each numerology, its common resource blocks are specified in Clause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to "Reference point A" in the unit of the numerology. The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth [13] and will fulfil the minimum UE guardband requirement specified in Clause 5.3.3.

### 5.3.5	Channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: Channel bandwidths for each NR band

| Operating band | SCS (kHz) | UE channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 400 |  |  | 800 |  |  | 1600 |  |  | 2000 |  |  |
| n257 |  |  | 60 |  |  | 50 |  |  | 100 |  |  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 120 |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 4001 |  |  |  |  |  |  |  |  |  |  |  |
| n258 |  |  | 60 |  |  | 50 |  |  | 100 |  |  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 120 |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 4001 |  |  |  |  |  |  |  |  |  |  |  |
| n259 |  |  | 60 |  |  | 50 |  |  | 100 |  |  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 120 |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 4001 |  |  |  |  |  |  |  |  |  |  |  |
| n260 |  |  | 60 |  |  | 50 |  |  | 100 |  |  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 120 |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 4001 |  |  |  |  |  |  |  |  |  |  |  |
| n261 |  |  | 60 |  |  | 50 |  |  | 100 |  |  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 120 |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 4001 |  |  |  |  |  |  |  |  |  |  |  |
| n262 |  |  | 60 |  |  | 50 |  |  | 100 |  |  | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 120 |  |  | 50 |  |  | 100 |  |  | 200 |  |  | 4001 |  |  |  |  |  |  |  |  |  |  |  |
| n263 |  | 120 |  |  |  |  |  | 100 |  |  |  |  |  | 400 |  |  |  |  |  |  |  |  |  |  |  |
|  | 4802 |  |  |  | 400 | 800 | 1600 |  |
|  | 9602 |  |  |  | 400 | 800 | 1600 | 2000 |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: This SCS is optional in this release of the specification. Editor’s note: Optional/mandatory CHBW discussion is still on-going and further modifications are possible. |  |  |  |  |  |  |  |  |

5.4	Channel arrangement

5.4.1	Channel spacing

5.4.1.1	Channel spacing for adjacent NR carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent NR carriers is defined as following:

For NR operating bands with 60 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 + {-20 kHz, 0 kHz, 20 kHz} for ∆FRaster equals to 60 kHz

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 + {-40 kHz, 0 kHz, 40 kHz} for ∆FRaster equals to 120 kHz

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

5.4.2	Channel raster

5.4.2.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequency is designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [2016667...3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

The mapping between the channel raster and corresponding resource element is given in Clause 5.4.2.2. The applicable entries for each operating band are defined in clause 5.4.2.3

5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRB mod 2 = 0 | NRB mod 2 = 1 |
| --- | --- | --- |
| Resource element index k | 0 | 6 |
| Physical resource block number nPRB |  |  |

k, nRB , NRB are as defined in TS 38.211 [9].

5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

-	For NR operating bands with 60 kHz channel raster above 24 GHz, ΔFRaster = I ×ΔFGlobal , where I ϵ {1,2}.  Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

-	In frequency bands with two ΔFRaster, the higher ΔFRaster applies to channels using only the SCS that equals the higher ΔFRaster  and the SSB SCS that is equal to or larger than the higher ΔFRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 60 | 2054166 – <1> – 2104165 |
|  | 120 | 2054167 – <2> – 2104165 |
| n258 | 60 | 2016667 – <1> – 2070832 |
|  | 120 | 2016667 – <2> – 2070831 |
| n259 | 60 | 2270833 – <1> – 2337499 |
|  | 120 | 2270833 – <2> – 2337499 |
| n260 | 60 | 2229166 – <1> – 2279165 |
|  | 120 | 2229167 – <2> – 2279165 |
| n261 | 60 | 2070833 – <1> – 2084999 |
|  | 120 | 2070833 – <2> – 2084999 |
| n262 | 60 | 2399166 – <1> – 2415832 |
|  | 120 | 2399167 – <2> – 2415831 |
| n2631 | 60 | 2562499 – <1> –2795832 |
| NOTE 1: The following NREF are allowed for operation in Band n263: see Table 5.4.2.3-2. |  |  |

Table 5.4.2.3-2: Allowed NREF (NR-ARFCN) for operation in Band n263

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 100 MHz | 2563332 2564999 2566666 2568332 2569999 2571666 2573332 2574999 2576666 2578332 2579999 2581666 2583332 2584999 2586666 2588332 2589999 2591666 2593332 2594999 2596666 2598332 2599999 2601666 2603332 2604999 2606666 2608332 2609999 2611666 2613332 2614999 2616666 2618332 2619999 2621666 2623332 2624999 2626666 2628332 2629999 2631666 2633332 2634999 2636666 2638332 2639999 2641666 2643332 2644999 2646666 2648332 2649999 2651666 2653332 2654999 2656666 2658332 2659999 2661666 2663332 2664999 2666666 2668332 2669999 2671666 2673332 2674999 2676666 2678332 2679999 2681666 2683332 2684999 2686666 2688332 2689999 2691666 2693332 2694999 2696666 2698332 2699999 2701666 2703332 2704999 2706666 2708332 2709999 2711666 2713332 2714999 2716666 2718332 2719999 2721666 2723332 2724999 2726666 2728332 2729999 2731666 2733332 2734999 2736666 2738332 2739999 2741666 2743332 2744999 2746666 2748332 2749999 2751666 2753332 2754999 2756666 2758332 2759999 2761666 2763332 2764999 2766666 2768332 2769999 2771666 2773332 2774999 2776666 2778332 2779999 2781666 2783332 2784999 2786666 2788332 2789999 2791666 2793332 2794999 |
| 400 MHz | 2565832 2572499 2579166 2585832 2592499 2599166 2605832 2612499 2619166 2625832 2632499 2639166 2645832 2652499 2659166 2665832 2672499 2679166 2685832 2692499 2699166 2705832 2712499 2719166 2725832 2732499 2739166 2745832 2752499 2759166 2765832 2772499 2779166 2785832 2792499 |
| 800 MHz | 2569166 2582499 2595832 2609166 2622499 2635832 2649166 2662499 2675832 2689166 2702499 2715832 2729166 2742499 2755832 2769166 2782499 |
| 1600 MHz | 2575832 2602499 2629166 2655832 2682499 2709166 2735832 2762499 |
| 2000 MHz | 2579166 2612499 2645832 2679166 2712499 2745832 2779166 |

5.4.3	Synchronization raster

5.4.3.1	Synchronization raster and numbering

The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present.

A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are in Table 5.4.3.1-1.

The resource element corresponding to the SS block reference frequency SSREF is given in clause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block is defined separately for each band.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

5.4.3.2	Synchronization raster to synchronization block resource element mapping

The mapping between the synchronization raster and the corresponding resource element of the SS block is given in Table 5.4.3.2-1.

Table 5.4.3.2-1: Synchronization raster to SS block resource element mapping

| Resource element index k | 120 |
| --- | --- |

k is the subcarrier number of SS/PBCH block defined in TS 38.211 clause 7.4.3.1 [9].

5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n257 | 120 kHz | Case D | 22388 - <1> - 22558 |
|  | 240 kHz | Case E | 22390 - <2> - 22556 |
| n258 | 120 kHz | Case D | 22257 - <1> - 22443 |
|  | 240 kHz | Case E | 22258 - <2> - 22442 |
| n259 | 120 kHz | Case D | 23140 – <1> – 23369 |
|  | 240 kHz | Case E | 23142 – <2> – 23368 |
| n260 | 120 kHz | Case D | 22995 - <1> - 23166 |
|  | 240 kHz | Case E | 22996 - <2> - 23164 |
| n261 | 120 kHz | Case D | 22446 - <1> - 22492 |
|  | 240 kHz | Case E | 22446 - <2> - 22490 |
| n262 | 120 kHz | Case D | 23586 – <1> – 23641 |
|  | 240 kHz | Case E | 23588 – <2> – 23640 |
| n2632 | 120 kHz | Case D | 24151 - <3> - 24961 |
|  | 480 kHz | Case F | 24151- <15> -24961 |
|  | 9603 kHz | Case G | 24151 - <8>- 24959 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: The following SS raster entries are allowed for operation in band n263: see Table 5.4.3.3-2. NOTE 3: This SSB SCS is not used for initial access. |  |  |  |

Table 5.4.3.3-2. Allowed GSCN for operation in band n263

| SSB SCS | The allowed GSCN for operation in band n263 |
| --- | --- |
| 120 kHz | 24156 24162 24168 24173 24179 24185 24191 24197 24202 24208 24214 24220 24226 24231 24237 24243 24249 24254 24260 24266 24272 24278 24283 24289 24295 24301 24307 24312 24318 24324 24330 24335 24341 24347 24353 24359 24364 24370 24376 24382 24388 24393 24399 24405 24411 24416 24422 24428 24434 24440 24445 24451 24457 24463 24469 24474 24480 24486 24492 24497 24503 24509 24515 24521 24526 24532 24538 24544 24550 24555 24561 24567 24573 24579 24584 24590 24596 24602 24607 24613 24619 24625 24631 24636 24642 24648 24654 24660 24665 24671 24677 24683 24688 24694 24700 24706 24712 24717 24723 24729 24735 24741 24746 24752 24758 24764 24769 24775 24781 24787 24793 24798 24804 24810 24816 24822 24827 24833 24839 24845 24851 24856 24862 24868 24874 24879 24885 24891 24897 24903 24908 24914 24920 24926 24932 24937 24943 24949 24955 24960 |
| 480 kHz | 24171 24194 24217 24240 24263 24286 24309 24333 24356 24379 24402 24425 24448 24471 24495 24518 24541 24564 24587 24610 24633 24657 24680 24703 24726 24749 24772 24796 24819 24842 24865 24888 24911 24934 24958 |
| 960 kHz | 24167 24190 24213 24237 24260 24283 24306 24329 24352 24375 24399 24422 24445 24468 24491 24514 24538 24561 24584 24607 24630 24653 24676 24700 24723 24746 24769 24792 24815 24838 24862 24885 24908 24931 24954 |