3GPP TSG-RAN WG4 Meeting #102-e		R4-2204985
Electronic Meeting, 21st Feb. – 03rd Mar., 2022

Agenda item:	10.1.3.3

Source:	OPPO

Title:	PDP pass/fail limit for FR1

Document for:	Approval

1	Introduction

In the last meeting of RAN4 #101-bis-e, the PDP reference for FR1 CDL-C UMa channel model is agreed. However, the PDP pass/fail limits for Paths from 30dB to 40dB is left to be determined in RAN4 #102-e meeting, with two options [1].

This paper presents our views and proposals on the pass/fail limits for Paths from 30dB to 40dB.

2	Discussion

The agreed PDP reference for FR1 CDL-C UMa is captured below, while the power of paths below 30dB are marked in red.

| 2.45GHz |  |  |  |
| --- | --- | --- | --- |
| Combined Clusters index | Delay(ns) | Power of Beam 1 (dB) | Power of Beam 2 (dB) |
| 1 | 0 | -34.3 | -27.9 |
| 2-5 | 80 | -19.5 | 0.0 |
| 6-8 | 235 | 0.0 | -18.4 |
| 9-10 | 290 | -33.0 | -27.8 |
| 11 | 450 | -35.8 | -27.9 |
| 12 | 480 | -34.0 | -28.0 |

| 3.6GHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Combined Clusters index | Delay(ns) | Power of Beam 1 (dB) | Combined Clusters index | Delay(ns) | Power of Beam 2 (dB) |
| 1 | 0 | -34.2 | 1 | 0 | -27.8 |
| 2-5 | 80 | -19.3 | 2-5 | 80 | 0.0 |
| 6-8 | 235 | 0.0 | 6-8 | 235 | -18.3 |
| 9 | 290 | -34.7 | 9-10 | 290 | -28.9 |
| 10 | 450 | -35.8 | 11 | 450 | -28.1 |
| 11 | 480 | -34.7 | 12 | 480 | -28.8 |

For 2.45GHz, the clusters with power from 30dB to 40dB all come from Beam 1 with Cluster Index 1, 9-10, 11, 12.

For 3.6GHz, the clusters with power from 30dB to 40dB also come from Beam 1 with Cluster Index 1, 9, 10, 11.

The following tables summarize six labs’ measurement results (some of rough values getting from submitted curves of each lab) on those clusters.

| 2.45GHz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clusters Index | Reference | Lab A | Lab B | Lab C | Lab D | Lab E | Lab F | Roughly Max delta with reference |
| 1 | -34.3 | -35 | -31.4 | -33.03 | -33.84 | -32.5 | -33 | 3 |
| 9-10 | -33 | -28 | -30.8 | -29.72 | -36.5 | -32 | -31 | 5 |
| 11 | -35.8 | -34 | -32.9 | -33.22 | -35.53 | -33 | -36 | 3 |
| 12 | -34 | -36 | -31.1 | -31.46 | -33.09 | -32.5 | -33 | 3 |

| 3.6GHz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clusters Index | Reference | Lab A | Lab B | Lab C | Lab D | Lab E | Lab F | Roughly Max delta with reference |
| 1 | -34.2 | -35 | -31.5 | -34.25 | -34.48 | -32.5 | -34 | 3 |
| 9 | -34.7 | -29 | -32 | -30.82 | -32.17 | -32.5 | -32.5 | 6 |
| 10 | -35.8 | -35 | -33.1 | -33.24 | -36.83 | -33 | -36 | 3 |
| 11 | -34.7 | -36 | -32 | -33 | -35.13 | -32.5 | -35 | 3 |

The largest deviations to the reference come from Cluster 9-10 for 2.45GHz and Cluster 9 for 3.6GHz. And the deviations on other clusters are relatively small, i.e. about 3dB.

Observation: The largest deviations to the reference come from Cluster 9-10 for 2.45GHz and Cluster 9 for 3.6GHz.

Generally speaking, smaller tolerance on pass/fail limit will provide more accurate channel model for MIMO OTA measurement. Thus, PDP pass/fail limit should be defined with the reasonable smallest values. However, as the email discussion in RAN4 #101-bis-e meeting [2], the large deviation around 290ns is probably not caused by imperfect channel model simulation, but the measured power is actually the sidelobe of the strongest cluster (Cluster 6-8) instead of Cluster 9-10. Based on this understanding, it is acceptable to give more tolerance on such clusters, while keep other clusters with relatively small tolerances.

In summary, it is proposed to make the option 2 of the WF as the PDP pass/fail limit for CDL-C UMa, i.e. +/-10dB at 290ns and +/-5dB for others.

Proposal: It is proposed to make the option 2 of the WF as the PDP pass/fail limit for CDL-C UMa, i.e. +/-10dB at 290ns and +/-5dB for others.

3	Conclusion

Proposal: It is proposed to make the option 2 of the WF as the PDP pass/fail limit for CDL-C UMa, i.e. +/-10dB at 290ns and +/-5dB for others.

References

[1] R4-2203063 WF on NR MIMO OTA

[2] R4-2203096 Summary_324_2nd round