3GPP TSG-RAN4 Meeting #102-e	R4-2205056

# Electronic meeting, February 21 – March 3, 2022

Source:		Ericsson

Title:			pCR to TS 38.108 – in-band selectivity and blocking

Agenda item:		10.13.3.4

Document for:		Approval

# Background

Based on the various agreements from past RAN4 meetings, the following test is proposed for TS 38.108:

- In-band selectivity and blocking

Note this pCR is assuming 3 SAN classes will be defined: LEO600, LEO1200 and GEO.

For ACS, the interferer level is calculated as in TR 38.817-2:

Interfering signal power level = BS noise floor + ACS + 4.7dB = -174 dBm/Hz+10*log10(BW) + NF + ACS + 4.7 dB

With SAN ACS = [38] dBc, values are kept in [] then.

For in-band and narrow band blocking, those requirements would not be needed for SAN.

# Proposal

It is proposed that the proposed text related to in-band selectivity and blocking here after is included in TS 38.108 [1].

# References

- TS 38.108, Satellite Access Node radio transmission and reception

# Text proposal

<Start of the change>

## 7.4	In-band selectivity and blocking

### 7.4.1	Adjacent Channel Selectivity (ACS)

#### 7.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at TAB connector for SAN type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 7.4.1.2	Minimum requirements for Satellite Access Node

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SAN, the wanted and the interfering signal coupled to the SAN type 1-H TAB connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each SAN channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The ACS requirement is applicable outside the Satellite Access Node RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Satellite Access Node RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the TAB connector for SAN type 1-H.

Table 7.4.1.2-1: Satellite Access Node ACS requirement

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB | GEO SAN: [-57] LEO1200 SAN: [-60] LEO600 SAN: [-60] |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth. NOTE 2: PREFSENS depends on the SAN channel bandwidth as specified in tables 7.2.2-1. |  |  |

Table 7.4.1.2-2: Satelltie Access Node ACS interferer frequency offset values

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Satellite access node RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 |  |
| 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal |
| 15 | ±2.5125 | 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 |  |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 | 20 MHz DFT-s-OFDM NR signal |
| 60 | ±9.4725 | 15 kHz SCS, 100 RBs |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

### 7.4.2 	In-band blocking

#### 7.4.2.1	General

The in-band blocking characteristics is a measure of the receiver's ability to receive a wanted signal at its assigned channel at the TAB connector for SAN type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.4.2.2	Minimum requirements for Satellite Access Node

In-band blocking requirement is not applicable for SAN.

<End of the change>