3GPP TSG-RAN WG4 Meeting #102-e	R4-2205082

# Electronic Meeting, 21st February 2022 – 3rd March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft CR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR: FRC for CA PDSCH demodulation requirements for HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR1_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FRC tables used for PDSCH demodulation requirements for HST-SFN and HST-DPS in CA are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of FRC tables used for PDSCH demodulation requirements for HST-SFN and HST-DPS in CA. * Table A.3.2.1.1-13 and 14 for FDD HST-SFN CA * Table A.3.2.1.1-15 and 16 for FDD HST-DPS CA * Table A.3.2.2.2-19, 20 and 21 for TDD HST-SFN CA * Table A.3.2.2.2-22, 23, and 24 for TDD HST-DPS CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test equipement cannot configure the FRCs for PDSCH demodulation tests for HST-SFN/HST-DPS in CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision of R4-2202983 (Added FRC for CBW=35/45MHz in FDD) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

Table A.3.2.1.1-13: PDSCH Reference Channel for FDD HST-SFN with CA scenario

| Parameter | Unit | Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-13.1 FDD | R.PDSCH.1-13.2 FDD | R.PDSCH.1-13.3 FDD | R.PDSCH.1-13.4 FDD | R.PDSCH.1-13.5 FDD |
| Channel bandwidth | MHz | 5 | 15 | 20 | 25 | 30 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 79 | 106 | 133 | 160 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 12040 | 37896 | 51216 | 64552 | 77896 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 2 | 5 | 7 | 8 | 10 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,11,12 (Note 3) | Bits | 24000 | 77136 | 104352 | 131568 | 158784 |
| For Slots i = 3,…, 10, 13, …, 19 | Bits | 25200 | 79632 | 106848 | 134064 | 161280 |
| Max. Throughput averaged over 2 frames | Mbps | 11.438 | 36.001 | 48.655 | 61.324 | 74.001 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.1.1-14: PDSCH Reference Channel for FDD HST-SFN with CA scenario

| Parameter | Unit | Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-14.1 FDD | R.PDSCH.1-14.2 FDD | R.PDSCH.1-14.3 FDD | R.PDSCH.1-14.4 FDD |  |
| Channel bandwidth | MHz | 40 | 50 | 35 | 45 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 216 | 270 | 188 | 242 |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 13 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |  |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |  |
| Number of DMRS REs |  | 18 | 18 | 18 | 18 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 104496 | 131176 | 90176 | 116792 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | CBs | 13 | 16 | 11 | 14 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,2,11,12 (Note 3) | Bits | 215232 | 269664 | 187008 | 241440 |  |
| For Slots i = 3,…, 10, 13, …, 19 | Bits | 217728 | 272160 | 189504 | 243936 |  |
| Max. Throughput averaged over 2 frames | Mbps | 99.271 | 124.617 | 85.667 | 110.952 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.1.1-15: PDSCH Reference Channel for FDD HST-DPS with CA scenario

| Parameter | Unit | Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-15.1 FDD | R.PDSCH.1-15.2 FDD | R.PDSCH.1-15.3 FDD | R.PDSCH.1-15.4 FDD | R.PDSCH.1-15.5 FDD |
| Channel bandwidth | MHz | 5 | 15 | 20 | 25 | 30 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 79 | 106 | 133 | 160 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 16136 | 51216 | 67584 | 86040 | 104496 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 2 | 7 | 9 | 11 | 13 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,11,12 (Note 3) | Bits | 36000 | 115704 | 156528 | 197352 | 238176 |
| For Slots i = 3,…, 10, 13, …, 19 | Bits | 37800 | 119448 | 160272 | 201096 | 241920 |
| Max. Throughput averaged over 2 frames | Mbps | 15.3292 | 48.655 | 64.2048 | 81.738 | 99.2712 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.1.1-16: PDSCH Reference Channel for FDD HST-DPS with CA scenario

| Parameter | Unit | Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-16.1 FDD | R.PDSCH.1-16.2 FDD | R.PDSCH.1-16.3 FDD | R.PDSCH.1-16.4 FDD |  |
| Channel bandwidth | MHz | 40 | 50 | 35 | 45 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 216 | 270 | 188 | 242 |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 17 | 17 | 17 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |  |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |  |
| Number of DMRS REs |  | 18 | 18 | 18 | 18 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 139376 | 176208 | 120936 | 155776 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | CBs | 17 | 21 | 15 | 19 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,2,11,12 (Note 3) | Bits | 322848 | 404496 | 280512 | 362160 |  |
| For Slots i = 3,…, 10, 13, …, 19 | Bits | 326592 | 408240 | 284256 | 365904 |  |
| Max. Throughput averaged over 2 frames | Mbps | 132.407 | 167.3976 | 114.8892 | 147.9872 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

Table A.3.2.2.2-19: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-SFN with CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-19.1 TDD | R.PDSCH.2-19.2 TDD | R.PDSCH.2-19.3 TDD | R.PDSCH.2-19.4 TDD | R.PDSCH.2-19.5 TDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 11 | 24 | 38 | 51 | 65 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 27 | 27 | 27 | 27 | 27 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 5248 | 11528 | 18432 | 24576 | 31240 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 1 | 2 | 3 | 3 | 4 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,21,22 (Note 3) | Bits | 10560 | 23040 | 36480 | 48960 | 63024 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 11088 | 24192 | 38304 | 51408 | 65520 |
| Max. Throughput averaged over 2 frames | Mbps | 7.0848 | 15.5628 | 24.8832 | 33.1776 | 42.174 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.2.2-20: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-SFN with CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-20.1 TDD | R.PDSCH.2-20.2 TDD | R.PDSCH.2-20.3 TDD | R.PDSCH.2-20.4 TDD | R.PDSCH.2-20.5 TDD |
| Channel bandwidth | MHz | 30 | 50 | 60 | 80 | 90 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 78 | 133 | 162 | 217 | 245 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 27 | 27 | 27 | 27 | 27 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 37896 | 64552 | 77896 | 104496 | 118896 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 5 | 8 | 10 | 13 | 15 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,21,22 (Note 3) | Bits | 76128 | 131568 | 160800 | 216240 | 244464 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 78624 | 134064 | 163296 | 218736 | 246960 |
| Max. Throughput averaged over 2 frames | Mbps | 51.1596 | 87.1452 | 105.1596 | 141.0696 | 160.5096 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.2.2-21: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-SFN with CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-21.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 273 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 27 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 131176 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 16 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 1,2,21,22 (Note 3) | Bits | 272688 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 275184 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 177.0876 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.2.2-22: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-DPS with CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-22.1 TDD | R.PDSCH.2-22.2 TDD | R.PDSCH.2-22.3 TDD | R.PDSCH.2-22.4 TDD | R.PDSCH.2-22.5 TDD |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 11 | 24 | 38 | 51 | 65 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 2408 | 5120 | 8192 | 11016 | 14088 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 7040 | 15624 | 24576 | 32776 | 42016 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 1 | 1 | 2 | 2 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 1 | 2 | 3 | 4 | 5 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,21,22 (Note 3) | Bits | 15840 | 34560 | 54720 | 73440 | 94536 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 5544 | 12096 | 19152 | 25704 | 32760 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 16632 | 36288 | 57456 | 77112 | 98280 |
| Max. Throughput averaged over 2 frames | Mbps | 9.9856 | 22.1164 | 34.816 | 46.4508 | 59.5392 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.2.2-23: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-DPS with CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-23.1 TDD | R.PDSCH.2-23.2 TDD | R.PDSCH.2-23.3 TDD | R.PDSCH.2-23.4 TDD | R.PDSCH.2-23.5 TDD |
| Channel bandwidth | MHz | 30 | 50 | 60 | 80 | 90 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 78 | 133 | 162 | 217 | 245 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16896 | 28680 | 34816 | 47112 | 53288 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 50184 | 86040 | 104496 | 139376 | 159880 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 3 | 4 | 5 | 6 | 7 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 6 | 11 | 13 | 17 | 19 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,2,21,22 (Note 3) | Bits | 114192 | 197352 | 241200 | 324360 | 366696 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 39312 | 67032 | 81648 | 109368 | 123480 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 117936 | 201096 | 244944 | 328104 | 370440 |
| Max. Throughput averaged over 2 frames | Mbps | 71.1276 | 121.89 | 148.0328 | 197.58 | 226.4956 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

Table A.3.2.2.2-24: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and HST-DPS with CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-24.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 273 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 17 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.43 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 18 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 58384 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 176208 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 7 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 21 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 1,2,21,22 (Note 3) | Bits | 409032 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 137592 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,20,23,…,39} | Bits | 412776 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 249.5576 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------