3GPP TSG-RAN WG4 Meeting #102-e	R4-2205096

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.20.4.2.2

Source:	Ericsson

Title:	PDCCH/PBCH demodulation requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#101-bis-e agreed on RedCap demodulation and CQI reporting requirements [1]. This contribution discusses the open issues on PDCCH and PBCH decoding requirements for RedCap.

# 2	PDCCH demodulation

RAN4 agreed to define PDCCH demodulation requirements for both RedCap 1Rx UE and 2Rx UE with the same configuration. As it was discussed in the last RAN4 meeting, we prefer to verify PDCCH at least with AL4 and AL8 for 1Rx/2Rx considering the test coverage of RedCap UE. We also prefer to verify AL16 especially for 1Rx UE because RRM agreed to configure AL16 for RLM out-of-synch PDCCH transmission parameters to compensate the coverage due to single Rx. We think it is worth ensuring the performance of PDCCH with AL16 for RedCap UE.

As we discussed in our companion paper [2], we also propose to remove 1Rx case from FR2.

Proposal 1: Define the following PDCCH demodulation requirements for RedCap UE.

FR1 FDD SCS=15kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | New |
| 1Rx UE | 10 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.1.2-1 Test 2 | New |
| 1Rx UE | 10 | 48 | 2 | 16 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 5 | New |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 | Reuse |
| 2Rx UE | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.1.2-1 Test 2 | Reuse |
| 2Rx UE | 10 | 48 | 2 | 16 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 5 | Reuse |

FR1 TDD SCS=30kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | New |
| 1Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | New |
| 1Rx UE | 20 | 48 | 2 | 16 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 3 | New |
| 2Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | New |
| 2Rx Ue | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | New |
| 2Rx UE | 20 | 48 | 2 | 16 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 3 | New |

FR2 TDD SCS=120kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x1 | 1% | 7.3.2.2.1-1 Test 2 | New |
| 1Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x1 low | 1% | 7.3.2.2.2-1 Test 1 | New |
| 1Rx UE | 100 | 60 | 2 | 16 | TDLA30-75 | 2x1 low | 1% | 7.3.2.2.2-1 Test 2 | New |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 2 | Reuse |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 1 | Reuse |
| 2Rx UE | 100 | 60 | 2 | 16 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2 | Reuse |

# 3	PBCH demodulation

- RAN4 define PBCH demodulation requirements by reusing the existing PBCH test setup for 2Rx with SS/PBCH block index is unknown. - For the case with SS/PBCH block index is known, - Option 1: RAN4 define the case with SS/PBCH block index is known for 1Rx UE - Option 2: RAN4 don’t define the case with SS/PBCH block index is known for 1Rx UE

We are fine not to define PBCH demodulation requirements for 1Rx UE when SS/PBCH index is known, to minimize the simulation campaign in RAN4. For 2Rx UE with SCS=30kHz, we think RAN4 can reuse the same SNR test points because SS/PBCH block BW is same regardless of channel bandwidth.

As we discussed in our companion paper [2], we also propose to remove 1Rx case from FR2.

Proposal 2: Define the following PBCH demodulation requirements for RedCap UE.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New |
| 1Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x1 | 1% | Unknown | 7.4.2.2-2 Test 1 | New |
| 1Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x1 | 1% | Unknown | 7.4.2.2-2 Test 2 | New |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |

Figure 1 shows our simulation results for PBCH demodulation performance with 1Rx.

Figure 1	Simulation results for PBCH demodulation.

# 4	Summary

Proposal 1: Define the following PDCCH demodulation requirements for RedCap UE.

FR1 FDD SCS=15kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | New |
| 1Rx UE | 10 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.1.2-1 Test 2 | New |
| 1Rx UE | 10 | 48 | 2 | 16 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 5 | New |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 | Reuse |
| 2Rx UE | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.1.2-1 Test 2 | Reuse |
| 2Rx UE | 10 | 48 | 2 | 16 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 5 | Reuse |

FR1 TDD SCS=30kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | New |
| 1Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | New |
| 1Rx UE | 20 | 48 | 2 | 16 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 3 | New |
| 2Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | New |
| 2Rx Ue | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | New |
| 2Rx UE | 20 | 48 | 2 | 16 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 3 | New |

FR2 TDD SCS=120kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x1 | 1% | 7.3.2.2.1-1 Test 2 | New |
| 1Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x1 low | 1% | 7.3.2.2.2-1 Test 1 | New |
| 1Rx UE | 100 | 60 | 2 | 16 | TDLA30-75 | 2x1 low | 1% | 7.3.2.2.2-1 Test 2 | New |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 2 | Reuse |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 1 | Reuse |
| 2Rx UE | 100 | 60 | 2 | 16 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2 | Reuse |

Proposal 2: Define the following PBCH demodulation requirements for RedCap UE.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New |
| 1Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x1 | 1% | Unknown | 7.4.2.2-2 Test 1 | New |
| 1Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x1 | 1% | Unknown | 7.4.2.2-2 Test 2 | New |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |

# References

- R4-2203032 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R4-2205094, “General issues on UE demodulation and CSI requirements for RedCap”, Ericsson.