3GPP TSG-RAN WG4 Meeting #102-e	R4-2205098

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.20.4.3.2

Source:	Ericsson

Title:	PMI/RI reporting requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#101-bis-e agreed on RedCap demodulation and CQI reporting requirements [1]. This contribution discusses the open issues on PMI and RI reporting requirements for RedCap.

# 2	PMI reporting requirements

- RAN4 discuss further whether to define PMI reporting tests for RedCap UE. - Option 1: Define for both 1Rx UE and 2Rx UE - Option 2: Not define for both 1Rx UE and 2Rx UE - Option 3 Define only for 2Rx UE - If PMI reporting test is defined, discuss further applied test cases

We think it may be beneficial to define the PMI reporting test with Type-I precoder considering the beam forming gain. To minimize the cost to rerun the simulation, we propose to limit to the mandate Tx ports, that is, 4 and 8 for both 1Rx/2Rx. For 1Rx case, we think it is sufficient to define one test case with 4Tx because of the limitation of rank 1.

As we discussed in our companion paper [2], we also propose to remove 1Rx case from FR2.

Proposal 1: Define the following PMI reporting tests for RedCap UE.

FR1 FDD SCS=15kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 1 XP high | New |
| 1Rx | Table 6.3.2.1.2-1 Test 1 | Single PMI, Type-I, rank 1 | 10 / 15 | 8 | 8 x 1 XP high | New |
| 2Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 2 XP high | Reuse |
| 2Rx | Table 6.3.2.1.2-1 Test 1 | Single PMI, Type-I, rank 2 | 10 / 15 | 8 | 8 x 2 XP high | Reuse |

FR1 TDD SCS=30kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 1 XP high | New |
| 1Rx | Table 6.3.2.2.2-1 Test 1 | Single PMI, Type-I, rank 1 | 20 / 30 | 8 | 8 x 1 XP high | New |
| 2Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 2 XP high | New |
| 2Rx | Table 6.3.2.2.2-1 Test 1 | Single PMI, Type-I, rank 2 | 20 / 30 | 8 | 8 x 2 XP high | New |

FR2 TDD SCS=120kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 8.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 1 ULA low | New |
| 1Rx | Table 8.3.2.2.1-1 Test 2 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 1 ULA low | New |
| 2Rx | Table 8.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 2 ULA low | Reuse |
| 2Rx | Table 8.3.2.2.1-1 Test 2 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 2 ULA low | Reuse |

# 3	RI reporting test for 2Rx UE

- RAN4 discuss further whether to define RI reporting tests for RedCap 2Rx UE. - Option 1: Define RI tests. It requires to rerun the simulation with CQI table 1 for FR1 FDD and TDD. - Option 2: Not define RI tests.

We have discussed whether RAN4 need simulation campaign for RI tests. As it was discussed in RAN4#101-bis-e, RAN4 need to rerun the simulations for FR1 FDD and TDD because we need to use CQI table 1. For FR2, we can reuse the existing requirements because Rel-15 FR2 RI tests assume CQI table 1. However if RF group agrees the 2 layer MIMO is optional for RedCap FR2 UEs, this requirements should be optional the capability supporting 2 layer MIMO.

We have no strong view for RI tests, but we think it is sufficient to define CQI reporting and PMI reporting test for RedCap UE.

Proposal 2: Not define RI tests for RedCap.

# 4	Summary

Proposal 1: Define the following PMI reporting tests for RedCap UE.

FR1 FDD SCS=15kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 1 XP high | New |
| 1Rx | Table 6.3.2.1.2-1 Test 1 | Single PMI, Type-I, rank 1 | 10 / 15 | 8 | 8 x 1 XP high | New |
| 2Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 2 XP high | Reuse |
| 2Rx | Table 6.3.2.1.2-1 Test 1 | Single PMI, Type-I, rank 2 | 10 / 15 | 8 | 8 x 2 XP high | Reuse |

FR1 TDD SCS=30kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 1 XP high | New |
| 1Rx | Table 6.3.2.2.2-1 Test 1 | Single PMI, Type-I, rank 1 | 20 / 30 | 8 | 8 x 1 XP high | New |
| 2Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 2 XP high | New |
| 2Rx | Table 6.3.2.2.2-1 Test 1 | Single PMI, Type-I, rank 2 | 20 / 30 | 8 | 8 x 2 XP high | New |

FR2 TDD SCS=120kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 8.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 1 ULA low | New |
| 1Rx | Table 8.3.2.2.1-1 Test 2 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 1 ULA low | New |
| 2Rx | Table 8.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 2 ULA low | Reuse |
| 2Rx | Table 8.3.2.2.1-1 Test 2 | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 2 ULA low | Reuse |

Proposal 2: Not define RI tests for RedCap.

# References

- R4-2203032 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R4-2205094, “General issues on UE demodulation and CSI requirements for RedCap”, Ericsson.

# Appendix: RI test cases (if agreed).

FR1 FDD SCS=15kHz

| 1Rx UE or 2Rx UE | Reference | RI test | CBW / SCS | CQI table | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 6.4.2.1-1 Test 1 | Fix RI=2 and follow RI | 10 / 15 | Table 1 | 2 x 2 ULA low | New |
| 2Rx UE | Table 6.4.2.1-1 Test 2 | Fix RI=1 and follow RI | 10 / 15 | Table 1 | 2 x 2 ULA low | New |
| 2Rx UE | Table 6.4.2.1-1 Test 3 | Fix RI=1 and follow RI | 10 / 15 | Table 1 | 2 x 2 ULA high | New |

FR1 TDD SCS=30kHz

| 1Rx UE or 2Rx UE | Reference | RI test | CBW / SCS | CQI table | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 6.4.2.2-1 Test 1 | Fix RI=2 and follow RI | 20 / 30 | Table 1 | 2 x 2 ULA low | New |
| 2Rx UE | Table 6.4.2.2-1 Test 2 | Fix RI=1 and follow RI | 20 / 30 | Table 1 | 2 x 2 ULA low | New |
| 2Rx UE | Table 6.4.2.2-1 Test 3 | Fix RI=1 and follow RI | 20 / 30 | Table 1 | 2 x 2 ULA high | New |

FR2 TDD SCS=120kHz

| 1Rx UE or 2Rx UE | Reference | RI test | CBW / SCS | CQI table | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 8.4.2.2-1 Test 1 | Fix RI=2 and follow RI | 100 / 120 | Table 1 | 2 x 2 ULA low | Reuse |
| 2Rx UE | Table 8.4.2.2-1 Test 2 | Fix RI=1 and follow RI | 100 / 120 | Table 1 | 2 x 2 ULA low | Reuse |
| 2Rx UE | Table 8.4.2.2-1 Test 3 | Fix RI=1 and follow RI | 100 / 120 | Table 1 | 2 x 2 XP high | Reuse |