3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205104

# Electronic Meeting, 22nd Feb – 3th Mar, 2022

Source: 	Ligado Networks, Inmarsat

Title: 	Discussion on GEO SAN ACLR

Agenda Item:	10.13.2.3

Document for:	Approval

# Introduction

The NTN satellite access node (SAN) ACLR value of 24 dBc was agreed to in RAN4#101-bis-e meeting [1]. There, the NTN ACLR determination appears to have been based on co-existence scenario, Case 3, using a LEO NTN SAN [2]. However, the use of LEO NTN SAN results in a significant overestimate of the ACLR value for a GEO NTN SAN. This is because the simulations do not factor in the considerable net path loss increase that will be experienced by a GEO SAN due to larger distances. From a satellite payload design perspective, this 24 dBc ACLR value translates to a Noise Power Ratio (NPR) of approximately 18.2 dBc [3]. This is an excessive value for a GEO satellite featuring high Aggregate EIRP (AEIRP).

Based on the analysis presented in this paper, it is proposed that, for GEO NTN SAN, an ACLR value of 13.5 dBc should be specified.

# Discussion

Table 1 shows the net path loss difference between GEO and LEO DL links.

Table 1: EIRP and path loss differences between GEO and LEO links

| Using Set 1 parameters | LEO-600 | LEO-1200 |
| --- | --- | --- |
| EIRP Density (per MHz) difference (GEO satellite EIRP – LEO satellite EIRP) | 25 dB (= 59 dBW – 34 dBW) | 19 dB (= 59 dBW– 40 dBW) |
| Path loss difference (GEO satellite path loss –LEO satellite path loss) L band is used (worst case) | 35 dB (= 187 dB – 152 dB) | 29 dB (=187 dB – 158 dB) |
| Net difference in favour of GEO | 10 dB | 10 dB |

Observation 1: For Case 3, the received signal strength at the TN UE from a GEO NTN satellite beam is expected to be ~10 dB lower than that received from the LEO NTN satellite beam.

Table 2 shows a comparative analysis of the noise floor rise based on the agreed upon ACLR value of 24 dBc. In both GEO and LEO scenarios, aggregate interference from 7 beams is taken into consideration, for the worst case of the TN cluster being in the centre of the central (nadir) beam.

Table 2: Estimated Noise Floor Rise with ACLR of 24 dBc

| Parameter | Units | LEO-600 | LEO-1200 | GEO | Note |
| --- | --- | --- | --- | --- | --- |
| Frequency | MHz | 1542 | 1542 | 1542 | Band n255 Centre Frequency |
| Typical satellite carrier EIRP Density | dBW/MHz | 34 | 40 | 59 | 3GPP NTN Simulation Model (Table 2.3-2) |
| ACLR | dBc | 24 | 24 | 24 |  |
| OOCE PSD EIRP | dBW/Hz | -50 | -44 | -25 | Calculated |
| Propagation distance | km | 600 | 1,200 | 35,786 | Altitude: 3GPP NTN Simulation Model (Table 2.3-2) |
| Pathloss to Satellite | dB | 152 | 158 | 187 | Calculated Free space pathloss |
| Total No. of satellite beams | # | 7 | 7 | 7 | 3GPP NTN Simulation Model |
| Isolation from Overhead Beam | dB | 0 | 0 | 0 | 3GPP NTN Simulation Model |
| # of Adjacent Beams | # | 6 | 6 | 6 | 3GPP NTN Simulation Model |
| Adjacent Beam Isolation | dB | -10.65 | -10.65 | -10.65 | 3GPP NTN Simulation Model |
| Average satellite beam discrimination | dB | 6.6 | 6.6 | 6.6 | Based on Simulations |
| Blockage factor % | % | 0% | 0% | 0% | Worst Case Assumption |
| TN UE antenna gain | dBi | 0 | 0 | 0 | Worst Case Assumption |
| Rx OOCE (Interference) | dBW/Hz | -199.9 | -199.9 | -210.4 | Calculated |
| TN UE NF (dB) | dB | 9.0 | 9.0 | 9.0 | 3GPP NTN Simulation Model |
| Thermal Noise floor | dBW/Hz | -195.0 | -195.0 | -195.0 | Calculated |
| TN receiver Noise Floor Rise | dB | 1.2 | 1.2 | 0.1 | Calculated |
| Difference in TN receiver noise floor rise relative to Reference | dB | Reference | 0.0 | -1.1 | Calculated |

Observation 2: For Case 3, the noise floor rise at the TN UE as a result of ACI from the GEO NTN satellite beams is expected to be 1.1 dB lower than that received from the LEO NTN satellite beams when ACLR value of 24 dB is used for both.

Table 3.1 shows the ACLR value for a GEO SAN that would result in the same amount of rise in the noise floor as due to the LEO SAN.

Table 3.1: ACLR estimate for GEO

(ACLR providing same noise rise as in LEO cases)

| Parameter | Units | GEO |
| --- | --- | --- |
| Frequency | MHz | 1542 |
| Typical satellite carrier EIRP Density | dBW/MHz | 59 |
| ACLR | dBc | 13.5 |
| OOCE PSD EIRP | dBW/Hz | -14.5 |
| Propagation distance | km | 35,786 |
| Pathloss to Satellite | dB | 187 |
| Total No. of satellite beams | # | 7 |
| Isolation from Overhead Beam | dB | 0 |
| # of Adjacent Beams | # | 6 |
| Adjacent Beam Isolation | dB | -10.65 |
| Average satellite beam discrimination | dB | 6.6 |
| Blockage factor % | % | 0% |
| TN UE antenna gain | dBi | 0 |
| Rx OOCE (Interference) | dBW/Hz | -199.9 |
| TN UE NF (dB) | dB | 9.0 |
| Thermal Noise floor | dBW/Hz | -195.0 |
| TN receiver Noise Floor Rise | dB | 1.2 |
| Difference in TN receiver noise floor rise relative to Reference | dB | 0.0 |

Observation 3: For Case 3, ACLR of 13.5 dBc for the GEO SAN will result in the same noise rise and degradation as that observed due to the LEO SAN.

Proposal 1: Use separate ACLR values for GEO and LEO SANs.

Proposal 2: Specify ACLR value of 13.5 dBc for a GEO SAN.

# Conclusion

In the contribution, based on the observations it is proposed that the minimum GEO SAN ACLR value of 13.5 dBc be specified.

Observation 1: For Case 3, the received signal strength at the TN UE from a GEO NTN satellite beam is expected to be ~10 dB lower than that received from the LEO NTN satellite beam.

Observation 2: For Case 3, the noise floor rise at the TN UE as a result of ACI from the GEO NTN satellite beams is expected to be 1.1 dB lower than that received from the LEO NTN satellite beams when ACLR value of 24 dB is used for both.

Observation 3: For Case 3, ACLR of 13.5 dBc for the GEO SAN will result in the same noise rise and degradation as that observed due to the LEO SAN.

Proposal 1: Use separate ACLR values for GEO and LEO SANs.

Proposal 2: Specify ACLR value of 13.5 dBc for a GEO SAN.

# References

[1] R4-2202990, WF on [307] NTN_Solutions_Part2, Samsung

[2] TR 38.863, v0.2.0, NTN related RF and co-existence aspects

[3] High-Linearity RF Amplifier Design, Peter B. Kenington, Artech House Inc, 2000, pp. 39-43