3GPP TSG-RAN WG4 Meeting #102-e                              R4-2205251

Electronic Meeting, 21 February– 3 March, 2022

Source: 	Huawei, HiSilicon

Title: 	Updated TP for TR 37.717-31-11 DC_3C-7A-32A_n78A

Agenda Item:	9.16.2

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #94e meeting. This contribution provides a TP for TR 37.717-31-11 to finish the UE RF requirements for the band combination.

- References

[1]	RP-212901, “Revised WID on Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson

- Text Proposal

<TP for TR 37.717-31-11>

- 5.1.18	DC_3-7-32_n78

5.1.18.1	Configuration for EN-DC

Table 5.1.18.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-32A_n78A DC_3C-7A-32A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |

5.1.18.2	∆TIB and ∆RIB values

Table 5.1.18.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-32_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.18.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-32_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

5.1.18.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 have been solved by fallback band combinations DC_3A-32A_n78A and DC_7A-32A_n78A.

<End of TP >