3GPP TSG-RAN WG4 Meeting #102-e                              R4-2205252

Electronic Meeting, 21 February– 3 March, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.717-41-11 DC_1A-3A-7A-32A_n78A and DC_1A-3C-7A-32A_n78A

Agenda Item:	9.17.2

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #94e meeting. This contribution provides a TP for TR 37.717-41-11 to finish the UE RF requirements for the band combination.

- References

[1]	RP-213251, “Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1U)”, Nokia

- Text Proposal

<TP for TR 37.717-41-11>

5.1.X	DC_1-3-7-32_n78

5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-7A-32A_n78A DC_1A-3C-7A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |

5.1.X.2	∆TIB and ∆RIB values

Table 5.1.X.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-32_n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |

Table 5.1.X.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-32_n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | 32 | 0 |
|  | n78 | 0.5 |

5.1.X.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

<End of TP >