3GPP TSG-RAN4 Meeting #102-e	R4-2205349

# Electronic Meeting, 21 February – 3 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to eMTC inter-frequency measurement requirements in Idle mode R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2022-03-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The margin for inter-frequency cell reselection are incorrect for eMTC enhanced coverage. The margin is defined as 8dB for newly detectable cell but 6dB for already detected cell. In addition, for connected mode, the relative RSRP accuracy requirements in section 9.1.21 for mode B is 8dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the margin in clause 4.7.2.2.3 from 6dB to 8dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eMTC inter-frequency measurement requirements in idle mode are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage

The requirements in this subclause apply if UE is in the enhanced coverage area of the serving cell. The UE is considered to be in enhanced coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The UE shall be able to identify new inter-frequency cells and perform RSRP or RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided. The UE shall not cause any interruption to the paging reception and acquisition of SI while performing measurement on serving or any neighbor cells.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Kcarrier*Tdetect,EUTRAN_Inter_EC, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 8 dB for reselections based on ranking. Kcarrier is the number of inter-frequency carriers in the neighbour cell list. An inter frequency cell is considered to be detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.8 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,E-UTRAN_Inter_EC . If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure RSRP or RSRQ at least every Kcarrier*Tmeasure,EUTRAN_Inter_EC for identified lower or equal priority inter-frequency cells. If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter RSRP or RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 4 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN_Inter_EC/2.

The UE shall not consider a E-UTRA neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Kcarrier*Tevaluate,E-UTRAN_Inter_EC, when Treselection = 0 provided that the reselection criteria is met by a margin of at least 8 dB for reselections based on ranking.

If Treselection timer has a non zero value and the inter-frequency cell is better ranked than the serving cell, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,EUTRAN_Inter_EC, Tmeasure,EUTRAN_Inter_EC and Tevaluate, E-UTRAN_inter_EC are specified in Table 4.7.2.2.3-1. For UE configured with eDRX_IDLE cycle, Tdetect,EUTRAN_Inter_EC, Tmeasure,EUTRAN_Inter_EC and Tevaluate, E-UTRAN_inter_EC are specified in Table 4.7.2.2.3-3. Additionally, the requirements in Table 4.7.2.2.3-3 apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,EUTRAN_Inter_EC, Tmeasure,EUTRAN_Inter_EC and Tevaluate, E-UTRAN_inter_EC when multiple PTWs are used.

Table 4.7.2.2.3-1: Tdetect,EUTRAN_Inter_EC, Tmeasure,EUTRAN_Inter_EC and Tevaluate,E-UTRAN_Inter_EC

| SCH Ês/Iot of neighboring cell: Q2 [dB] | DRX cycle length [s] | Tdetect,EUTRAN_Inter_EC [s] (number of DRX cycles) | Tmeasure,EUTRAN_Inter_EC [s] (number of DRX cycles) | Tevaluate,E-UTRAN_inter_EC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0.32 | 330.24 (1032) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 330.24 (516) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 524.8 (410) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 1039.36 (406) | 2.56 (1) | 15.36 (6) |
| Q2-6 | 0.32 | 16.64 (52) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 23.04 (36) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 38.4 (30) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 66.56 (26) | 2.56 (1) | 15.36 (6) |

Table 4.7.2.2.3-2: Void

Table 4.7.2.2.3-3: Tdetect,EUTRAN_Inter_EC, Tmeasure,EUTRAN_Inter_EC and Tevaluate, E-UTRAN_inter_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_Inter_EC [s] (number of DRX or eDRX cycles Note 4) for neighboring cell with SCH Es/IoT: -15≤ Q2 < -6 [dB] | Tdetect,EUTRAN_Inter_EC [s] (number of DRX or eDRX cycles Note 4) for neighboring cell with SCH Es/IoT: Q2-6 [dB] | Tmeasure,EUTRAN_Inter_EC [s] (number of DRX or eDRX cycles Note 4) | Tevaluate,E-UTRAN_inter_EC [s] (number of DRX or eDRX cycles Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
| 5.12 | N/A | N/A | 2078.72 (406) | 133.12 (26) | 5.12 (1) | 30.72 (6) |
| 10.24 ≤ eDRX_IDLE cycle lengt | 0.32 | ≥1.28 (1) | Note 3 (406) | Note 3 (26) | 0.32 (1) | Note 3 (6) |
|  | 0.64 | ≥1.28 (1) |  |  | 0.64 (1) | Note 3 (6) |
|  | 1.28 | ≥1.28 (1) |  |  | 1.28 (1) | Note 3 (6) |
|  | 2.56 | ≥2.56 (2) |  |  | 2.56 (1) | Note 3 (6) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: The detection period and the evaluation period depend on the number N of DRX cycles and are calculated according to the formula below: . NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. |  |  |  |  |  |  |

For higher priority cells, a UE may optionally use a shorter value for Tmeasure,EUTRAN_Inter_EC,which shall not be less than Max(0.64 s, one DRX cycle).

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 of TS 36.304 [1] are fulfilled then the UE’s inter-frequency measurement is not required to meet Tdetect,EUTRAN_Inter_EC, Tmeasure,EUTRAN_Inter_EC and Tevaluate, E-UTRAN_inter_EC as defined in Table 4.7.2.2.3-1 and Table 4.7.2.2.3-3.

<End of Change 1>