3GPP TSG-RAN WG4 Meeting #102-e	R4-2205354

Electronic Meeting, 21 February – 3 March, 2022

Title: 	Discussion on accuracy requirements for positioning measurement

Source: 	Huawei, HiSilicon

Agenda item:	5.1.3.2

Document for:	Discussion

Introduction

In RAN4#101-e, the remaining issues in RRM accuracy requirements for positioning measurement were discussed, and the outcome of the discussions are captured in WF [1]. Based on our understanding, the following issues are to be further discussed:

- Group delay calibration margin for RSTD

- Frequency drift margin for RSTD

- PRS-RSRP requirements under the extreme condition

- Group delay calibration margin for Rx-Tx

- Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment

- Test cases

In this paper we will provide our views on the remaining issues for positioning accuracy requirements

Discussion

  - Group delay calibration margin for RSTD

Group delay calibration margin for RSTD - in FR1 when the reference and target PRS resources are in the same PFL (single PFL case) PRS BW (MHz); Margin (Tc). ≥ 5; [TBD]. ≥ 10; [TBD]. ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. - in FR2 when the reference and target PRS resources are in the same PFL (single PFL case). FFS if additional margin should be added for OTA calibration. PRS BW (MHz); Margin (Tc). ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. ≥ 200; [TBD].

For RSTD, if reference cell and neighbour cell are measured with different Rx paths or at different times, the measurements will experience different calibration errors which cannot cancel out. As a result, a margin to account for the ‘relative’ calibration error is needed.

The calibration error should depend on the PRS BW due to the fact that the group delay is dependent on BW, so it has to be calibrated for each BW instead of the highest BW. On the other hand the calibration accuracy is limited by the RS BW used for the calibration. The calibration error should depend on FR1/FR2 because in FR2 the calibration is OTA so there can be some challenges to get very high accuracy.

Our suggested values for different PRS BWs and different FRs (for single PFL case) are:

Proposal 1: Add the following group delay calibration margin for RSTD accuracy for single PFL case.

Table 1: Calibration margin for RSTD FR1 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | [40] |
| ≥ 10 | [32] |
| ≥ 20 | [32] |
| ≥ 50 | [16] |
| ≥ 100 | [16] |

Table 2: Calibration margin for RSTD FR2 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | [32] |
| ≥ 50 | [32] |
| ≥ 100 | [16] |
| ≥ 200 | [16] |

  - Frequency drift margin for RSTD

Frequency drift margin for RSTD - RAN4 will define a single value for the frequency drift margin based on a single value of the maximum time offset - FFS on the exact value

Basically, when the reference resource and the neighbor resource are measured at different time, there is a clock drift due to frequency error, which means even UE perfectly estimates the receive timing of the two resources, it can still not calculate RSTD correctly (due to incorrect understanding about the absolute time in between the two measurements).

The error due to frequency drift depends on the offset between the two measurements (resources instances) and the frequency drift. In RF requirements, the allowed frequency drift is +/-0.1 ppm, so if the offset is 160ms, the error will be +/-32Tc. It is noted that the error does not increase infinitely with the offset because UE still maintains the timing for the serving cell data transmission, however, the required accuracy for serving cell data transmission (i.e. Te) can be much larger than what is expected for positioning.

For defining the frequency drift margin for RSTD accuracy, there is a trade-off between the maximum offset and the value of the margin. The larger the time offset is allowed, the larger the margin value will be needed. To keep the accuracy requirements still meaningful and reasonable, we suggest to define the margin as +/-32Tc based on maximum offset of 160ms.

The above values mean there would be practically no applicable requirement for RSTD measurement with reference resource and neighbor resource on different PFLs. This is because UE may measure multiple PLFs sequentially (this is also the assumption in defining the measurement period requirements), and in this case, the time gap between the reference resource instance and the neighbor resource instance will very likely be larger than the maximum offset of 160ms. We are open to further discuss how to handle the case of multiple PFL case or more generically the case where the time distance between reference PRS resource instance and target PRS resource instance is large.

Proposal 2: For RSTD accuracy requirements, define the frequency drift margin as +/-32Tc based on maximum offset of 160ms. FFS whether and how to handle larger time offset.

  - PRS-RSRP requirements under the extreme condition

RSRP requirements under extreme conditions - FFS on the PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - [1.5]dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2

Our first preference is to not define PRS-RSRP accuracy requirements for extreme condition. It is noted that for both RSTD and UE Rx-Tx, we do not have accuracy requirements for extreme condition, and it would be a bit inconsistent that requirements for extreme condition are only define one specific PRS measurement. If requirements for extreme condition are of strong interests, we can define them in Rel-17 or later.

If RAN4 is anyway going to define requirements for extreme condition, we suggest the following relaxations based on current SS-RSRP requirements for intra- and inter-frequency.

- 3dB for absolute accuracy for FR1 (see Table 10.1.4.1.1-1)

- 3dB for absolute accuracy for FR2 (see Table 10.1.5.1.1-1)

- 1.5dB for relative accuracy for FR1 (see Table 10.1.4.1.2-1)

- 3dB for relative accuracy for FR2 (see Table 10.1.5.1.2-1)

Proposal 3: RAN4 does not define PRS-RSRP accuracy requirements for extreme condition in Rel-16.

  - Group delay calibration margin for Rx-Tx

Group delay calibration margin for UE Rx-Tx time difference - in FR1 PRS BW (MHz); Margin (Tc). ≥ 5; [TBD]. ≥ 10; [TBD]. ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. - in FR2 . FFS if additional margin should be added for OTA calibration. PRS BW (MHz); Margin (Tc). ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. ≥ 200; [TBD].

In section 2.1, we discussed the group delay calibration margin for RSTD. For UE Rx-Tx, one difference is that the group delay of both Rx path and Tx path need to be compensated, so the calibration error from both paths need to be accounted. Another difference is for Rx-Tx only one PFL is measured, so there is no dependence on the number of PFLs. Similar as for RSTD, group delay is dependent on BW and FR1/FR2.

In RAN4#100-e, some companies suggested [2] to define the margin as [4] ns * (100/BW). In our view, the value of 4ns may be only accounting for the static part of the group delay, e.g. the delay of different physical components which does not change or change slowly over time. However, it is not enough to accommodate the uncertainty in UL and DL timing determination.

Such uncertainty or error could depend on the frequencies of UE internal clocks, including those in digital and analog domains. For example, if the UL BW is 100MHz, there could be a +/-8ns uncertainty in determining the UL timing if we only consider the RF clock frequency (~16ns sampling interval). This uncertainty is more like random jitter, and will add together with the (semi)-static physical component delay calibration error when we compare the UE reported Rx-Tx and the ideal Rx-Tx.

Considering all the imperfections, we suggest the following values for the group delay calibration margin.

Proposal 4: Add the following group delay calibration margin for UE Rx-Tx accuracy.

Table 3: Calibration margin for UE Rx-Tx FR1

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | [96] |
| ≥ 10 | [80] |
| ≥ 20 | [72] |
| ≥ 50 | [32] |
| ≥ 100 | [24] |

Table 4: Calibration margin for UE Rx-Tx FR2

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | [72] |
| ≥ 50 | [32] |
| ≥ 100 | [24] |
| ≥ 200 | [24] |

  - Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment

Applicability of UE Rx-Tx time difference accuracy requirements under TA adjustment FFS on Applicability of UE Rx-Tx time difference accuracy requirements under TA adjustment - Option 1. - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment - Option 2. - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - the UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

We support option 2.

It is noted that based on current requirements for UE autonomous timing adjustment, the time scale of the UL timing change is on a similar scale as NW commanded TA. For example, for 30kHz SCS, the TA step size is 8Ts, while the step size for UE autonomous timing adjustment is 5.5Ts. In this sense, we do not see clear reason to define different applicability rules for UE autonomous timing adjustment and NW commanded TA.

The problem to apply the accuracy requirements is how to define the ideal value. With UE autonomous timing adjustment, UE could have more than one UL timing during the measurement period, and it’s unclear which one should be used to determine the ideal Rx-Tx. This may also have implications on how UE does the averaging across multiple samples.

In RAN4#101-e, some companies commented that for UE autonomous timing adjustment, the UL is following the DL, and this is the difference from the NW commanded TA. We agree that this is the case for the serving cell, where the ideal UE Rx-Tx for the serving cell does not change much with UE autonomous timing adjustment (but still the accuracy is Te as UE is only required to meet the UL timing accuracy), but this is clearly not the case for neighbour cells, for which the ideal UE Rx-Tx changes.

Proposal 5: Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment is defined as:

- UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

- UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

Conclusions

In this paper we provided our views on remaining issues for positioning accuracy requirements.

Proposal 1: Add the following group delay calibration margin for RSTD accuracy for single PFL case.

Table 1: Calibration margin for RSTD FR1 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | [40] |
| ≥ 10 | [32] |
| ≥ 20 | [32] |
| ≥ 50 | [16] |
| ≥ 100 | [16] |

Table 2: Calibration margin for RSTD FR2 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | [32] |
| ≥ 50 | [32] |
| ≥ 100 | [16] |
| ≥ 200 | [16] |

Proposal 2: For RSTD accuracy requirements, define the frequency drift margin as +/-32Tc based on maximum offset of 160ms. FFS whether and how to handle larger time offset.

Proposal 3: RAN4 does not define PRS-RSRP accuracy requirements for extreme condition in Rel-16.

Proposal 4: Add the following group delay calibration margin for UE Rx-Tx accuracy.

Table 3: Calibration margin for UE Rx-Tx FR1

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | [96] |
| ≥ 10 | [80] |
| ≥ 20 | [72] |
| ≥ 50 | [32] |
| ≥ 100 | [24] |

Table 4: Calibration margin for UE Rx-Tx FR2

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | [72] |
| ≥ 50 | [32] |
| ≥ 100 | [24] |
| ≥ 200 | [24] |

Proposal 5: Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment is defined as:

- UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

- UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

Reference

- R4-2120270, WF on NR Positioning RRM performance requirements maintenance, Intel Corporation