3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205399

Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	Impact on existing UE positioning and RRM requirements

Agenda item:	10.21.2.4

Document for:	Discussion

# 1	Introduction

In RAN4 101-e-bis, there were discussions on latency reduction for positioning enhancement. Some of the WF [1] are copied below.

Issue 2-2-1: Can Rel-15 SRS (SRS-Resource) be used for gNB Rx-Tx time difference in Rel-17? Open issues: - Can Rel-15 SRS (SRS-Resource) be used for gNB Rx-Tx time difference in Rel-17? - Option 1: - Yes - Option 2: - No Issue 2-2-3: Impact of SRS antenna switching on gNB Rx-Tx time difference accuracy, if Rel-15 SRS (SRS-Resource is supported for gNB Rx-Tx time difference. Open issues: - Are gNB Rx-Tx time difference requirements impacted due to SRS antenna port switching, if SRS antenna port switches during the measurement? - Option 1: Yes - Option 2: No

- This paper discusses the issues listed above and also provide our view on the other related issues.

2	Discussions

As we discussed during the last meeting, R15 SRS as well as R16 positioning-dedicated SRS can both be used as the SRS for UE Rx-Tx and gNB Rx-Tx measurements. At least from RAN4 perspective, this case is not forbidden. If specifications from other groups are cited, then we feel that a LS would be best to seek clarification from the source group to avoid misinterpretation.

R15 SRS as well as R16 positioning-dedicated SRS can both be used as the SRS for UE Rx-Tx and gNB Rx-Tx measurements.

Then, R15 SRS might be switched to another antenna port and thus, positioning accuracy might be affected. In general, we can agree on the statement that if SRS antenna port switching happens during UE/gNB Rx-Tx or UL RTOA, there can be impact on positioning measurement and its accuracy.

If SRS antenna port switching happens during UE/gNB Rx-Tx, there can be impact on positioning measurement and its accuracy.

Regarding whether SRS AP switching can be avoided, we think there is no need to specify whether this can be avoided by the network or not in specifications. This issue is dependent on the previous issue, and corresponding requirements / UE behaviors can be defined. If under some certain cases, such SRS AP switching can be avoided, then it will be done up to the network. If under some cases it cannot be avoided, then the corresponding requirements apply.

There is no need to specify whether this can be avoided by the network or not in specifications.

3	Conclusion

- Proposal 1: R15 SRS as well as R16 positioning-dedicated SRS can both be used as the SRS for UE Rx-Tx and gNB Rx-Tx measurements.

Proposal 2: If SRS antenna port switching happens during UE/gNB Rx-Tx, there can be impact on positioning measurement and its accuracy.

Proposal 3: There is no need to specify whether this can be avoided by the network or not in specifications.

References

- R4-2202776