3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2205418

Electronic Meeting, Feb. 21 – March 3, 2022

Source:	Ericsson

Title:	Propagation Delay Compensation Enhancements for Time Synchronization

Agenda item:	10.23.2.1

Document for:	Approval

# 1	Introduction

In this tdoc we review RAN1 decision regarding propagation delay compensation, analyze the required performance and propose how to define RTT-based PDC accuracy requirements.

# 2	Discussion

In WF from RAN4#101-bis-e [1]:

## ... 1.9 Measurement accuracy requirements (Sub-topic 1-9) Measurement accuracy requirements – gNB … For PDC RTT gNB Rx-Tx time difference measurement accuracy requirements re-use existing gNB Rx-Tx requirements for 3dB side condition and SCS 15/30kHz (Rel-17 38.133, 13.2.2). WF: FFS: 60KHz and FR2. ... Measurement accuracy requirements - UE WF for next meeting: PRS: - Shall RAN4 define requirements based on Rel-16 or Rel-17, or for Rel-16 and Rel-17 PRS? - Number of samples assumed (1 or 4 samples)? TRS: - 1 or 4 samples Companies are encouraged to perform first simulation results for the next RAN4 meeting (RAN4#102) …

Also from [1], but related to simulation assumptions for CSI-RS/TRS UE Rx-Tx Time Difference:

- ... 3.1 Simulation assumptions Table 1 General parameters Parameter; Value. FR1; FR2. Cell layout; 1 serving cell. Duplex modes; FDD and TDD. TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4); - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz); - TDDConf.3.1 (120 kHz). Data and CCH load in non-TRS symbols; - 50% utilization in time - 100% RE utilization. Cyclic prefix; Normal. DRX; OFF. Carrier frequency / BW / SCS / duplex mode; - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD; - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD. Propagation conditions [TS 38.101-4]; AWGN, [TDL-A (30 ns delay spread, 5Hz)]. Es/Iot [dB]; -3. Number of UE receive antennas; 2 rx (uncorrelated with equal gain, no rx beamforming). UE measurement bandwidth; Full carrier bandwidth. Table 2 CSI-RS for tracking transmission configuration parameters Parameter; Value. Number of transmit TRS antennas; 1. Cell ID, TRP ID, CSI-RS Resource Set ID; 1. Number with CSI-RS Resource sets with trs-Info; 1. TRS transmission bandwidth (in PRBs) - full carrier BW; - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: 24(10MHz), 52 (20MHz),132 (50MHz), 272 (100MHz); - 120 kHz: 32(50MHz), 64 (100MHz),128 (200MHz). Sample rate (Tc); - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz); FFS. TRS periodicity; 40ms. TRS resources number; 2, 4. TRS symbol location; {4,8}. TRS frequency density; 3. TRS samples; 1, 4. Evaluation period; 40 ms, 200 ms. At least the following performance characteristics are to be provided for TUE-RX: - 90%-ile of the TUE-RX errors for each cell and, - TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).

Regarding “FFS: 60KHz and FR2.”. In  R1-2108635 LS in [2]:

Agreements:

Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization.

In R1-2112834 LS in [3]

Ask RAN4 to define the following for RTT-based propagation delay compensation:

- UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking

- UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate)

- gNB Rx-Tx time difference absolute accuracy based on SRS (including reuse existing spec if appropriate)

Observation 1: The 15 and 30 kHz SCS from RAN1 LS were for evaluation using control-to-control and smart grid set of requirements. The final LS states no SCS limitations.

Proposal 1: Requirements for PDC shall be stated for all SCS.

Regarding the issue of:

PRS:

- Shall RAN4 define requirements based on Rel-16 or Rel-17, or for Rel-16 and Rel-17 PRS?

- Number of samples assumed (1 or 4 samples)?

We believe, as analyzed in our previous contribution [4], that existing UE Rx-Tx Time Difference requirements in rel-16 TS 38.133 section 10.1.25 can be reused, since the control-to-control and smart grid set of evaluation requirements can be fulfilled.

Observation 2: The control-to-control and smart grid set of evaluation requirements can be fulfilled with rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements.

Proposal 2: Reuse rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements, for UE Rx-Tx time difference measurement accuracy based on PRS.

Proposal 3: In order to facilitate reuse of rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements, for UE Rx-Tx time difference measurement accuracy based on PRS we can base PDC requirements on 4 samples.

Also from [1], but related to simulation assumptions for CSI-RS/TRS UE Rx-Tx Time Difference:

- ... 3.1 Simulation assumptions Table 1 General parameters Parameter; Value. FR1; FR2. Cell layout; 1 serving cell. Duplex modes; FDD and TDD. TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4); - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz); - TDDConf.3.1 (120 kHz). Data and CCH load in non-TRS symbols; - 50% utilization in time - 100% RE utilization. Cyclic prefix; Normal. DRX; OFF. Carrier frequency / BW / SCS / duplex mode; - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD; - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD. Propagation conditions [TS 38.101-4]; AWGN, [TDL-A (30 ns delay spread, 5Hz)]. Es/Iot [dB]; -3. Number of UE receive antennas; 2 rx (uncorrelated with equal gain, no rx beamforming). UE measurement bandwidth; Full carrier bandwidth. Table 2 CSI-RS for tracking transmission configuration parameters Parameter; Value. Number of transmit TRS antennas; 1. Cell ID, TRP ID, CSI-RS Resource Set ID; 1. Number with CSI-RS Resource sets with trs-Info; 1. TRS transmission bandwidth (in PRBs) - full carrier BW; - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: 24(10MHz), 52 (20MHz),132 (50MHz), 272 (100MHz); - 120 kHz: 32(50MHz), 64 (100MHz),128 (200MHz). Sample rate (Tc); - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz); FFS. TRS periodicity; 40ms. TRS resources number; 2, 4. TRS symbol location; {4,8}. TRS frequency density; 3. TRS samples; 1, 4. Evaluation period; 40 ms, 200 ms. At least the following performance characteristics are to be provided for TUE-RX: - 90%-ile of the TUE-RX errors for each cell and, - TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).

Observation 3: Our simulation results are reported in R4-2205419, Simulation results for Propagation Delay Compensation.

# 3	Summary

Observation 1: The 15 and 30 kHz SCS from RAN1 LS were for evaluation using control-to-control and smart grid set of requirements. The final LS states no SCS limitations.

Proposal 1: Requirements for PDC shall be stated for all SCS.

Observation 2: The control-to-control and smart grid set of evaluation requirements can be fulfilled with rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements.

Proposal 2: Reuse rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements, for UE Rx-Tx time difference measurement accuracy based on PRS.

Proposal 3: In order to facilitate reuse of rel-16 TS 38.133 section 10.1.25 Ue Rx-Tx Time Difference Measurements requirements, for UE Rx-Tx time difference measurement accuracy based on PRS we can base PDC requirements on 4 samples.

Observation 3: Observation 3: Our simulation results are reported in R4-2205419, Simulation results for Propagation Delay Compensation [5].

# References

- R4-2202701, WF on NR_IIOT_URLLC_enh_RRM, Nokia, Nokia Shanghai Bell

- R1-2108635 LS in

- R1-2112834 LS in

- R4-2201584, Propagation Delay Compensation Enhancements for Time Synchronization, Ericsson

- R4-2205419, Simulation results for Propagation Delay Compensation, Ericsson