# 3GPP TSG-RAN WG4 Meeting # 102-e						R4-2205528

# Electronic Meeting, 21st  Feb –3rd March, 2022

Source:		Ericsson

Title:			On measurement of the TX coherent transmission

Agenda item:		10.18.2.4

Document for:		Approval

# Introduction

In this paper, we continue to discuss our view on the RF requirement measurement aspect of phase /power discontinuity tolerance based on WF [1].

# Discussion

## Reference point

In WF[1], it is agreed to define the reference point of measurement:

## Issue 3-2: Reference point for phase/amplitude tolerance test

Candidate options:

- Option 1: Define the reference point for phase/amplitude tolerance requirement in Figure 1 for both options of defining RF requirements in Issue 3-1.

- Option 2: The reference point would be between “Channel estimation” and “equalization”, since the amplitude and phase values can be taken directly from the channel estimation.

- Option 3: For testing phase and power offset: Reference point to calculate the phase continuity and power consistency is after IDFT (time domain).

It also has below agreement related to this:

Agreement: the test reference point should be after FFT operation in test equipment.

Agreement: The reference point is between “Channel estimation” and “equalization”, since the amplitude and phase values can be taken directly from the channel estimation.

Though the channel estimation could be derived using the DMRS reference signal,  there are different RMC for either DFT-s-OFDM and CP-OFDM. Then question is that whether DFT-s-OFDM or CP-OFDM should be tested.  The MPR in TS 38.101-1 is different for different waveform and it can be observed that DFT-s-OFDM has lower MPR at both edge/outer and inner RB allocation compared with CP-OFDM waveform for QPSK. The maximum MPR difference is 2 dB. Though the MPR is allowance UE may use, it could be expected in general that for UE with DFT-s-OFDM it will deliver more output power than UE with CP-OFDM waveform and this is benefit on the coverage.

Consider both DFT-s-OFDM RMC and CP-OFDM RMC in the RF test

## CFO correction

In WF[1], it is agreed also below:

## Issue 3-4: Impact of CFO in the test

Candidate options:

- For UE side:

- Option 1: In DMRS bundling test for the UE, assume UE does not change the frequency during the bundle.

- Option 2: UE frequency remains within existing frequency error limits.

  - A UE that has a frequency error requirement of+/- 0.1ppm

- For BS/TE side:

- Option 1: Frequency correction in the JCE test is slot by slot basis.

- Option 2: Frequency correction in the JCE test is applied to the whole bundle.

WF recommendation:

- For TE side, agree on Option 1, i.e., Frequency correction in the JCE test is slot by slot basis.

This WF need to be revisited, the correction on all samples within the bundle should be done with reference to the first sample within the bundled time slots. As the measurement point will be taken as the difference of channel estimation in phase domain in two time slots, the phase accumulation by the frequency error on the first sample in each time slot need to be corrected.  For example, the frequency error is f1, the reference time slot is first time slot and the measured time slot is the second time slots. The first sample in the first symbol of the 2nd time slot need to be correct with f1* one time slot* 2pi.  The option 2 on TE side should be used for frequency correction to avoid wrong test result.

The option 2 should be used for correct test result.

## Text proposal for the Annex F.9

- The annex for the channel coefficient and measurement point definition:

- Annex F.9 Phase offset measurement for DMRS bundling

The measurement point for phase offset measurement is defined in Figure F.9-1.

Figure F.9-1: measurement point for phase offset for DMRS bundling

Annex F.9.1 Modified test signal

The post-FFT modulated signal before the equalization is modified according to:

Z''(t,f)=FFTz(v-Δt)⋅e-j2πΔfv.ej2πfΔt

where

z(v) is the time domain samples of the signal under test within the bundled time slots.

To minimize the error, the signal under test should be modified with respect to a set of parameters following the procedure explained below.

Notation:

Δt is the sample timing difference between the FFT processing window in relation to nominal timing of the ideal signal.

Δf is the RF frequency offset.

To minimize the error, the EVM window estimation is reused for the phase difference measurement for DMRS bundling.

In the following Δc represents the middle sample of the EVM window of length W or the last sample of the first window half if Wis even.

The test equipment shall

-	detect the start of each slot and estimate Δt and Δf,

-	determine Δc so that the EVM window of length W is centred

-	on the time interval determined by the measured cyclic prefix minus 16κ samples of the considered OFDM symbol for symbol l for subcarrier spacing configuration µ in a subframe, with l = 0 or l = 7*2^µ for normal CP, i.e. the first 16κ samples of the CP should not be taken into account for this step. In the determination of the number of excluded samples, a sampling rate of 1/Tc is assumed. If a different sampling rate is used, the number of excluded samples is scaled linearly.

-	on the measured cyclic prefix of the considered OFDM symbol symbol for all other symbols for normal CP and for symbol 0 to 11 for extended CP.

To determine the other parameters a sample timing offset equal to Δc is corrected from the signal under test. The test equipment shall then

-	correct the RF frequency offset Δffor each time slot within the bundled time slots, and

-	apply an FFT of appropriate size. The chosen FFT size shall ensure that in the case of an ideal signal under test, there is no measured inter-subcarrier interference.

The carrier leakage shall be removed from the evaluated signal; however, the removed relative carrier leakage power also has to satisfy the applicable requirement.

At this stage estimates of Δf, a(t,f), ϕ(t,f) and Δc are available. Δt is one of the extremities of the window W, i.e. Δtcan be Δc+α-W2 or Δc+W2, where α=0 if W is odd and α=1 if Wis even. The test equipment  shall then

-	calculate PhaseOffsetl with Δt set to Δc+α-W2,

-	calculate PhaseOffseth with Δt set to Δc+W2.

Annex F.9.2 Frequency response of the TX chain

Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal Z''(t,f) and the post-FFT ideal reference signal I(t,f), for each reference signal, over one measurement interval of one time slot. This process creates a set of complex ratios of the channel coefficient:

H(t,f)=α(t,f)⋅ejφ(t,f)=Z'(t,f)I(t,f)

Where the post-FFT ideal signal I(t,f), is DMRS reference signal within one time slot.

- Perform time averaging of the phase φt,f at each reference signal subcarrier of the channel coefficient, the time-averaging length is total number of N DMRS symbols over one time slot.

- φf=tφt,f N=ttan-1Im(Ht,f)Re (Ht,f) N

Re (H(t,f)) is the real part of the complex-valued H (t,f) and Im (H(t,f)) is the imaginary part of the the complex-valued H (t,f).

Annex F.9.3 phase offset measurement

For the reference time slot Tr, the phase of complex-valued channel coefficient is calculated as φf in Annex F.9.2 with channel coefficient Hφ(t,f).

For the time slot Ti, the phase of complex-valued channel coefficient is calculated as θf in Annex F.9.2 with channel coefficient Hθ(t,f) .

For the phase offset measurement between time slot Ti and the reference time slot Tr, the PhaseOffset (Ti) is defined as below:

PhaseOffset Ti= maxf(φf- θf)=maxf(ttan-1Re (Hθ(t,f)∙Hφt,f*)Im(Hθ(t,f)∙Hφt,f*)N)

P

The phase offset measurement of PhaseOffset (Ti) is measured as maximum absolute value over all allocated DMRS subcarriers and averaged over N DMRS symbols in one time slot. Hφt,f* is the complex conjugate of Hφt,f.

The measurement should be done within all bundled time slot except the reference time slot and repeated over [10] bundles, the rms value of phase offset should be calculated with below equation assuming the total number of measurement sample is L.

PhaseOffsetrms= 1LPhaseOffset Ti2L

Finally, the PhaseOffset measurement shall be tested against the maximum of the RMS average at the window W extremities of the measurements:

Thus PhaseOffsetl is calculated using in the expressions above and PhaseOffsethis calculated using .

Thus we get:

PhaseOffset=max(PhaseOffsetl ,PhaseOffseth)

# Conclusions

In this contribution, we present our view on the measurement  aspect of phase continuity with below proposal:

Proposal-1:Consider both DFT-s-OFDM and CP-OFDM in the RF test

Proposal-2:The option 2 should be used for correct test result.

# References

- R4-2202418,	WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei

- R4-114212, Relative Phase Discontinuity, Qualcomm Incorporated