3GPP TSG RAN WG4 Meeting #102-e	R4-2205633

Electronic Meeting, February 21 – 3 March, 2022

Agenda Item:	10.20.3.4

Source: 	Ericsson

Title:	Discussions on UE capabilities RRM requirements

Document for:	Discussion

# Introduction

RAN4 has received an LS from RAN2 with the action to provide feedback on the RAN2 agreements on capabilities and on whether there are any Rel-15 and/or Rel-16 UE features or capabilities which should not be applicable for RedCap UEs [1]. RAN4 has received an additional LS from RAN1 on UE capability containing the RAN1 agreements on the capability, see [3]. In this contribution we discuss the RRM aspects of this LS and provide our view, and a draft response LS is provided in the Annex.

The UE capability for RedCap in release 17 was also discussed and following agreements were reached [2].

RAN4 has agreed to define RRM requirements for following two Rel-16 features. o 2-step RA o NR measurements with autonomous gaps

In this contribution, we discuss and provide our view on support for the remaining release 16 features.

# Discussion

In table 1 we go through the RAN2 agreements on UE capabilities and provide view on their RRM impact.

| Capability | RedCap RRM support |
| --- | --- |
| Dual connectivity (including any and all features that require support for dual connectivity) | Not applicable |
| Carrier aggregation (including any and all features that require support for carrier aggregation) | Not applicable |
| DAPS and CAPC | Not applicable |
| CHO | FFS |
| Inter-RAT mobility | Inter-RAT to LTE |
| Measurement related capabilities | Applicable |
| URLLC except those affected by CA/DC | No RRM impact |
| IAB | Not applicable |

Table 1 List of RAN2 agreed capabilities and their impact on RRM

The RAN4 agreement has been to use release 15 NR RRM requirements as baseline and then to discuss which release 16 features to support for RedCap case by case based on justification. In table 2 we provide a list of other Rel-16 features that are not affected by CA/DC and not stated in [1] that are currently supported in TS 38.133 and provide our view on whether or not they should also be considered for Rel-17 RedCap UE.

| Rel-15/Rel-16 features in TS 38.133 | RedCap RRM requirements applicability in R17 | Comments |
| --- | --- | --- |
| Unlicensed operation (shared spectrum access) | Not applicable | The specifications have significant RRM impact. According to RAN plenary decision during RAN#93 unlicensed RedCap operation cannot be introduced if there is any specification impact. |
| Minimization of Drive Tests (MDT) | Not applicable | Not applicable in Rel-17 RedCap as current requirements cannot be reused. MDT provides tools to configure and log measurements e.g. related to particular events such as radio link quality etc. |
| Cross-Link Interference measurements | Not applicable | Not applicable for RedCap in Rel-17 since CLI is not in the scope of WI. CLI measurements are used to mitigate cross link interference and is an enhancement introduced in Rel-17 for legacy NR devices. |
| L1-SINR measurement reporting | Not applicable | Do not support L1-SINR measurement reporting for RedCap in Rel-17 as current requirements cannot be reused for Rel-17 RedCap and developing of new requirements is not possible from a workload perspective in Rel-17. |
| NR measurements for positioning | Not applicable | Do not support positioning measurements for RedCap in Rel-17 as it is not in the scope of the WID. The Rel-18 positioning WI covers includes also RedCap. |
| CSI-RS based L3 measurements | Not applicable | Not applicable in Rel-17 as current requirements cannot be reused directly for RedCap. In our view SSB based L3 measurements are most fundamental and can be used to provide the basic services to be provided by RedCap. |
| V2X requirements | Not applicable | It has specification impact but according to RAN plenary agreements during RAN#93 introduction of V2X requirements is allowed if there is no spec impact. |
| Measurement performance requirements for NR gNB | Not applicable | Not applicable for RedCap in Rel-17 as it is not in the scope of the WID. |

Table 2 List of release 16 features in TS 38.133 and their relation to RedCap

The above listed release 16 features were discussed at RAN4#101-bis-e meeting, however no consensus could be reached. In order to reach better progress in RAN4#102-e, our compromise proposal is to only list the features for which there is an agreement and to further clarify that RAN4 will not define any RRM requirements for the other release 16 features. In fact this approach was also proposed in [4].

Proposal #1: RAN4 to respond to RAN2 LS on UE capability as follows:

| Rel-15/Rel-16 features in TS 38.133 | RedCap RRM requirements applicability in R17 |
| --- | --- |
| Dual connectivity and carrier aggregation | Not applicable |
| 2-step RA | Applicable |
| NR measurements with autonomous gaps | Applicable |
| Rel-17 features in TS 38.133 |  |
| SDT | Applicable |
| Note: RAN4 will not define any RRM requirements for RedCap UE for other release 16 features which are not listed above in release 17. |  |

# Summary

In this contribution we have discussed the RAN2 LS on UE capabilities for RedCap and provided our view on Rel-15 and Rel-16 features and their relevance for RedCap UE in Rel-17. Based on the discussions, we have prepared a draft response LS to RAN2 in Annex.

# References

- R2-2109218, “LS on capability related RAN2 agreements for RedCap”, RAN2, 3GPP TSG RAN2#115-e, August 2021.

- R4-2120410, “WF on the use of NCD-SSB for RedCap UE”, ZTE Corporation

- R1-2112754, “Reply LS on capability related RAN2 agreements for RedCap”, RAN1

- R4-2201760, "On capability and NCD-SSB design for RedCap RRM", Apple

# Appendix

3GPP TSG RAN WG4 Meeting #102-e	R4-220xxxx

Electronic Meeting, February 21 – 3 March, 2022

Title:	Reply LS on UE capabilities for RedCap from RRM perspective

Response to:	R2-2109218

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

#### Name:	Santhan Thangarasa

#### E-mail Address:	santhan.thangarasa@ericsson.com

Attachments:

1. Overall Description:

RAN4 would like to thank RAN2 for the incoming LS in R2-2109218. RAN4 has discussed the RAN2 agreements on UE capabilities from a RRM perspective and further discussed the Rel-16 UE features and their applicability for RedCap in Rel-17. Based on the discussions, following conclusion is reached:

| Rel-15/Rel-16 features in TS 38.133 | RedCap RRM requirements applicability in R17 |
| --- | --- |
| Dual connectivity and carrier aggregation | Not applicable |
| 2-step RA | Applicable |
| NR measurements with autonomous gaps | Applicable |
| Rel-17 features in TS 38.133 |  |
| SDT | Applicable |
| Note: RAN4 will not define any RRM requirements for RedCap UE for other release 16 features which are not listed above in release 17. |  |

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take the above RAN4 RRM conclusions on UE capabilities in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #103-e								 	      	16 May – 27 May 2022, Electronic Meeting

TSG-RAN4 Meeting #104								 	      	22 Aug – 26 Aug 2022, Toulouse, France