# 3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205634

# Electronic Meeting, February 21 – 3 March, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | Draft | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Connected mode inter-frequency neighbour cell measurement before RLF for Rel-17 NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following the work split in R4-2202764, thic CR contains changes to introduce the CONNECTED mode inter-frequency neighbour cell measurement requirements applicable before RLF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: Inter-frequency cell detection and measurment requirements are introduced based on the agreements in R4-2202764. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-frequency requirements will be missing the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### 8.14.5	Neighbour cell measurement before RLF for UE Category NB1

#### 8.14.5.1	Introduction

This clause contains requirements for the neighbour cell measurement performed by the UE category NB1 in RRC_CONNECTED state. The requirements in this clause are applicable when the UE is in:

- normal coverage or in enhanced coverage on the serving cell and

- normal coverage on the target cell

#### 8.14.5.2	Requirements

The UE supporting connectedModeMeasurements-r17 [31] shall measure neighbour cell on anchor or non-carrier when criterion for triggering the neighbor cell measurement defined in [1] and UE is not in relaxed monitoring mode as defined in [1]. The UE may trigger the neighbour cell before or during the radio link failure depending on the fulfilment of the criterion.

The measurement quantities are defined in [4], the measurement model is defined in [22].

The requirements for intra-frequency neighbour cell measurement when the target carrier is same as serving carrier is defined in clause 8.14.5.1.

The requirements for inter-frequency neighbour cell measurement when the target carrier is different from serving carrier is defined in clause 8.14.5.2.

#### 8.14.5.3	Inter-frequency neighbour cell measurement when target cell in normal coverage

The UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_NB1-NC when the criteria for inter-frequency measurement is fulfilled [1]. An inter frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.x.y for a corresponding Band.

Tidentify _inter_NB1-NC = Tdetect_inter NB1-NC + Tmeasure _inter NB1-NC

Where

Tdetect_inter_NB1-NC=iNMin(5000, Ta,i) ms

- N = 70,

- Ta,i is the interval between available measurement samples in measurement occasions (MOdetect_inter_NB1-NC) for inter-frequency detection. The UE shall restart the cell detection when the following condition is not met:

40 ≤  Ta,i ≤ 5000 ms

- The UE is not required to monitor NPSS/NSSS more frequent than once per 40ms.

- MOdetect_inter_NB1-NC are time occasions containing at least subframes #0, #4, or #9 containing NPSS/NSSS and fulfil the following conditions:

  - Resources on which the UE is not scheduled for data transmission or reception,

  - Resources on which the UE is not required to do NPDCCH monitoring,

  - Resources occurring during the DRX inactive period

  - Length of MOdetect_inter_NB1-NC  is at least TBD ms.

Tmeasure_inter_NB1-NC is the physical layer measurement period of NRSRP on the detected inter-frequency cell as defined below:

Tmeasure_inter_NB1-NC=iMMin(5000, Tb,i) ms

- M = 60 for NRS-based RRM measurement and M = 40 for NSSS based RRM measurement,

- Tb,i is the interval between available measurement samples in measurement occasions (MOmeasure_inter_NB1-NC) for inter-frequency measurement. The UE shall restart the measurement when following conditions is not met:

20 ≤  Tb,i ≤ 5000 ms for NRS-based RRM measurement

40 ≤  Tb,i ≤ 5000 ms for NSSS-based RRM measurement

- The UE is not required to monitor NRS more frequent than once per 20ms for NRS-based measurement and NSSS more frequent than 40 ms for NSSS-based measurement.

- MOmeasure_inter_NB1-NC are time occasion containing at least NRS or NSSS that fulfil the following conditions:

  - Resources on which the UE is not scheduled for data transmission or reception,

  - Resources on which the UE is not required to do NPDCCH monitoring,

  - Resources occurring during the DRX inactive period,

  - Length of MOmeasure_inter_NB1-NC  is at least TBD ms.

When UE is monitoring multiple carriers, Tidentify_inter_NB1-NC = Tdetect_NB1-NC + Tmeasure_NB1-NC, where Tdetect_NB1-NC = Tdetect _intra_NB1-NC +Nfreq* Tdetect_inter_NB1-NC and Tmeasure_NB1-NC = Tmeasure _intra_NB1-NC +Nfreq* Tmeasure_inter_NB1-NC. Nfreq is number of inter-frequency carriers to be measured according to the measurement capability, and Tdetect_intra_NB1-NC  and Tmeasure_intra_NB1-NC are defined in clause 8.14.5.1

### <End of Change 1>

### <Start of Change 2>

### 8.14.6	 Requirements for monitoring multiple carriers

For RRC_CONNECTED state, the UE shall be capable of monitoring at least:

-	Depending on UE capability, an intra-frequency carrier.

-	Depending on UE capability, at least 2 inter-frequency carriers.

### <End of Change 2>