3GPP TSG-RAN WG4 Meeting #102-e    				     R4-2205673

Electronic Meeting, 21st of February - 3rd of March, 2022

Agenda item:	     10.13.1.2

Source:	THALES

Title:	Draft text proposal for Clause 4.3 Requirement reference points - TS 38.108

Document for:	Approval

# Introduction

Draft TS skeleton for Non-terrestrial networks (NTN) related to NR; Satellite Access Node radio transmission and reception (TS 38.108) has been approved in R4-2203087 [1] and the work split has been summarized in R4-2203080 Way Forward on NTN_solutions_Part1 [2] approved in RAN4 101-bis-e meeting. For further information, please also follow the discussions in R4-2203111 [3].

In this document, following changes have been proposed as a TP to update TS 38.108:

- Section 4.3: Requirement reference points

# Reference

[1] R4-2203087 Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, THALES.

[2] R4-2203080 Way Forward on NTN_solutions_Part1, THALES.

[3] R4-2203111 Email discussion summary for [101-bis-e][306] NTN_Solutions_Part1, THALES.

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>----------------------------------------------

## 4.3	Requirement reference points

### 4.3.1 	SAN type 1-C

The requirement is not applicable in Release-17.

### 4.3.2	SAN type 1-H

For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

Figure 4.3.2-1: Radiated and conducted reference points for SAN type 1-H

Radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

The transceiver unit array is part of the composite transceiver functionality receiving and transmitting modulated signals to ensure radio links with ground base stations and users.

The satellite payload is composed by a transceiver unit array and a composite antenna array. The transceiver unit array contains an implementation of specific number of transmitter units and an implementation of specific number of receiver units.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network which distributes the RF power generated by the transceiver unit array to the transmit antenna array.

How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement clause.

### 4.3.3	SAN type 1-O

For SAN type 1-O the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Figure 4.3.3-1: Radiated reference points for SAN type 1-O

The SAN type 1-O payload includes an active antenna with performances characterized in radiated mode (OTA).

------------------------------------------------<End of TP>-----------------------------------------------