3GPP TSG-RAN WG4 Meeting #102-e	R4-2205695

Electronic Meeting, 21 February – 03 March 2022

Source:	Ericsson, BT plc

Title:	TP for TR 37.717-11-11: DC_20_n67

Agenda item:			9.14.2

Document for:	Approval

# 1. Introduction

Rel-17 LTE-NR DC combinations LTE 1 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.717-11-11 to include DC_20_n67.

# 2. Text Proposal

# ---Start of changes---

### 6.1.x	DC_20A_n67A

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20_n67 | 20 | n67 | No |

#### 6.1.x.2	Configuration for DC

Table 6.x.2.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A_n67A | 20A | 20A | n67A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_20A_n67A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

Since n67 is SDL there are no dual UL and spurious emissions does not need to be considered.

#### 6.1.x.5	MSD analysis for DC

Since n67 is SDL there are therefore no dual UL and IMD does not need to be considered.

Table 6.x.1.5-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for DC_20_n67.

Table 6.x.1.5-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |
| n67 | N/A | N/A | 738 | 758 | N/A | N/A | N/A | N/A | N/A | N/A |

Based on above table, no harmonic for DC_20_n67

Table 6.x.1.5-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 20 | 832 | 862 | 791 | 821 | 1582 | 1456 | 2151 | 2184 | 2868 | 2912 |
| n67 | N/A | N/A | 738 | 758 | 1476 | 1516 | 2214 | 2274 | 2952 | 3032 |

Based on above table, no harmonic mixing for DC_20_n67.

### 6.1.x.6	∆TIB and ∆RIB values

For DC_20_n67, the TIB,c and RIB,c values are reused from CA_n20-n28 and CA_n20-n67 and are given in the tables below.

# Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n67 | 20 | 0.5 |
|  | n67 | N/A |

# Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_20_n67 | 20 | 0 |
|  | n67 | 0 |

### 6.1.x.7	MSD

Since n67 is SDL there are therefore no dual UL and MSD does not need to be considered.

---End of changes---

# Reference

[1]		RP-213060, Rel-17 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL), CHTTL