3GPP TSG-RAN WG4 Meeting #102-e	R4-2205720

Electronic Meeting, 21 February – 03 March 2022

Source:	Ericsson, Telstra

Title:	TP for TR 37.827 Addition of DC_7_n5-n78

Agenda item:			9.31.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.827 to add DC_7A_n5A-n78A, DC_7C_n5A-n78A.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_7_n5-n78

### 5.x.1	Transmitter Characteristics

#### 5.x.1.1	Maximum Output Power

Table 5.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n78A | 266 | +2/-3 |
| DC_7C_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.x.1.2	Configurations for EN-DC

Table 5.x.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | DC_7A_n78A DC_7C_n78A |

#### 5.x.1.2	Co-existence study

Co-existence studies shows that:

- 2nd and 4h order IMD products may fall into own Rx frequencies of band n78.

- 2nd and 5th order IMD products may fall into own Rx frequencies of band n5.

### 5.x.2	Receiver Characteristics

#### 5.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

Table 5.x.2.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n5A-n78A | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n5 | 836 | 5 | 25 | 881 | N/A | N/A |
|  | n78 | 3391 | 10 | 50 | 3391 | 33.5 | IMD2 x |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n5 | 836 | 5 | 25 | 881 | 34.7 | IMD2 y |
|  | n78 | 3436 | 10 | 50 | 3436 | N/A | N/A |
| NOTE x: This band is subject to IMD4 also which MSD is not specified. NOTE y: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

---End of changes---