3GPP TSG-RAN WG4 Meeting #102-e	R4-2205769

# Electronic Meeting, 21st Feb – 3rd Mar, 2022

Title: 	Discussion on UE NTN CSI

Source: 	Huawei, HiSilicon

Agenda item:	10.13.6.4

Document for:	Discussion

Background

During RAN#101bis-e meeting, WF [1] on general and NTN UE demodulation requirements was approved. In this contribution, we share our views about NTN UE CSI requirements.

Discussion

SCS/CBW set for PDSCH requirements

CSI reporting requirements - Agreements: - Do not consider PMI and RI reporting requirements, FFS for CQI reporting for LEO.

Here we derived the satellite-UE RTT delay for different tilt angle and different scenario as showing in Table 2.2-1.

Table 2.2-1 Satellite-UE RTT delay

| Tilt angle/° |  | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Delay/ms | LEO-600 | 12.9 | 9.29 | 7.17 | 5.89 | 5.08 | 4.56 | 4.24 | 4.06 | 4.00 |
|  | LEO-1200 | 20.9 | 16.4 | 13.3 | 11.3 | 9.93 | 9.01 | 8.43 | 8.11 | 8.01 |

For CSI reporting requirements, considering that there is large Satellite-UE RTT delay (order of tens of milliseconds delay for both LEO-600 and LEO-1200), we are not sure the reporting CQI is still valid. Also, we don’t see any algorithm changed for the CQI reporting between TN network and NTN network. Even we assume the CQI reporting value for some cases are valid with larger tilt angle and lower orbit altitude, the CQI reporting performance under these NTN scenarios still can be ensured by legacy TN CQI reporting cases. Therefore, we propose to not consider any CSI reporting requirements for NTN scenario.

Do not consider any CSI reporting requirements for NTN scenario.

Proposals

In this contribution, we discuss on UE CSI demodulation requirements for NTN. Our observations and proposals are:

- Do not consider any CSI reporting requirements for NTN scenario.

Reference

R4-2203042, WF on general and NTN UE demodulation requirements, RAN4#101bis-e, Qualcomm Incorporated