3GPP TSG-RAN WG4 Meeting #102-e	R4-2205773

# Electronic Meeting, 21st Feb – 3rd Mar, 2022

Title: 	Discussion on BS coverage enhancement demod PUCCH

Source: 	Huawei, HiSilicon

Agenda item:	10.18.3.2

Document for:	Discussion

Background

During RAN4#101bis-e meeting, WF [1] on PUCCH demodulation performance of Rel-17 NR coverage enhancement was approved. In this contribution, we share our views about the BS coverage enhancement PUCCH demodulation requirements.

Discussion

PUCCH demodulation with Joint Channel Estimation (JCE)

Whether to define BS PUCCH demodulation requirements with JCE

Whether to define BS PUCCH demodulation requirements with JCE Candidate options: - Option 1: Yes - Option 2: No - Option 3: FFS after the finalization of core requirements in RAN1 and RAN4

For PUCCH DMRS bundling (JCE), we don’t think it necessity to define requirements since PUCCH is not the weak point for the uplink transmission, good enough performance can be observed for legacy PUCCH repetition. Also, even we have PUCCH repetition features in Rel-16, the corresponding requirement is not considered in Rel-16. In addition, we have multi-slot requirements defined in Rel-15 to ensure PUCCH performance. So we propose to not define BS PUCCH demodulation requirements with JCE.

Do not define BS PUCCH demodulation requirements with JCE.

Proposals

In this contribution, we discuss on BS PUCCH demodulation requirements for coverage enhancement. Our observations and proposals are:

- Do not define BS PUCCH demodulation requirements with JCE.

Reference

R4-2203031, WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement, RAN4#101bis-e, Nokia, Nokia Shanghai Bell