3GPP TSG-RAN WG4 Meeting # 102 -e	R4-22xxxxx

# Electronic Meeting, Feb.21st-Mar.3rd,2022

Title: 	Discussion on PDCCH and PBCH requirements for RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	10.20.4.2.1

Document for:	Discussion

Background

As per [1], RAN 4 has agreed to introduce the PDCCH and PBCH requirements for RedCap UE. In this paper, we provide our views on remain open issues.

PDCCH requirements

The open issues for PDCCH requirements are listed as follows:

#### FDD 15kHz FR1 - Option 1: AL8 only - Option 2: AL4 and AL8. FFS for AL16 #### TDD 30kHz FR1 - Option 1: AL4 only - Option 2: AL4 and AL8. FFS for AL16 - TDD 120kHz FR2 - Option 1: AL16 only - Option 2: AL4 and AL8. FFS for AL16

Based on our understanding, RedCap UE is a low cost UE, so we should reduce test cost as much as possible. AL doesn’t affect demodulation processing, hence, we propose to only introduce one AL per SCS.

Proposal 1: Consider following assumptions for RedCap UE PDCCH test:

- FDD 15 kHz FR1: AL8 only

- TDD 30 kHz FR1: AL4 only

- TDD 120 kHz FR2: AL16 only

Same time, we propose to use assumptions in Table 1 for RedCap PDCCH test.

Table 1: Proposed assumptions for RedCap UE PDCCH test

| Frequency range | Duplex mode | UE capability | Bandwidth/SCS (MHz/kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | Interleaver size | REG bundle size | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | FDD | maxNumberMIMO-LayersPDSCH=2 (Note 1) | 20/15 | 48 | 2 | 8 | nonInterleaved | N/A | 6 | TDLC300-100 | 2x2 Low |
|  |  | maxNumberMIMO-LayersPDSCH=1 | 20/15 | 48 | 2 | 8 | nonInterleaved | N/A | 6 | TDLC300-100 | 2x1 Low |
|  | TDD | maxNumberMIMO-LayersPDSCH=2 | 20/30 | 48 | 2 | 4 | Interleaved | 3 | 6 | TDLC300-100 | 2x2 Low |
|  |  | maxNumberMIMO-LayersPDSCH=1 | 20/30 | 48 | 2 | 4 | Interleaved | 3 | 6 | TDLC300-100 | 2x1 Low |
| FR2 | TDD | maxNumberMIMO-LayersPDSCH=2(Note 2) | 100/120 | 60 | 2 | 16 | Interleaved | 3 | 2 | TDLA30-75 | 2x2 Low |
|  |  | maxNumberMIMO-LayersPDSCH=1 | 100/120 | 60 | 2 | 16 | Interleaved | 3 | 2 | TDLA30-75 | 2x1 Low |
| Note 1: the reused test cases |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Use assumptions in Table 1 for RedCap PDCCH test.

PBCH requirements

The open issues for PBCH requirements are listed as followings:

- RAN4 define PBCH demodulation requirements by reusing the existing PBCH test setup for 2Rx with SS/PBCH block index is unknown. - For the case with SS/PBCH block index is known, - Option 1: RAN4 define the case with SS/PBCH block index is known for 1Rx UE - Option 2: RAN4 don’t define the case with SS/PBCH block index is known for 1Rx UE

Based on our understanding, there is no need to reduce test number since PBCH is untestable, it would be better to keep the test setup align with existing 2RX test. I.e. SS/PBCH block index is known and unknown.

Proposal 3:  Define the case with SS/PBCH block index is known for 1Rx UE

Conclusion

In this paper, we provide our views on remain open issues for RedCap UE PDCCH and PBCH requirements. The proposals are:

Proposal 1: Consider following assumptions for RedCap UE PDCCH test:

- FDD 15 kHz FR1: AL8 only

- TDD 30 kHz FR1: AL4 only

- TDD 120 kHz FR2: AL16 only

Proposal 2: Use assumptions in Table 1 for RedCap PDCCH test.

Table 1: Proposed assumptions for RedCap UE PDCCH test

| Frequency range | Duplex mode | UE capability | Bandwidth/SCS (MHz/kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | Interleaver size | REG bundle size | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | FDD | maxNumberMIMO-LayersPDSCH=2 (Note 1) | 20/15 | 48 | 2 | 8 | nonInterleaved | N/A | 6 | TDLC300-100 | 2x2 Low |
|  |  | maxNumberMIMO-LayersPDSCH=1 | 20/15 | 48 | 2 | 8 | nonInterleaved | N/A | 6 | TDLC300-100 | 2x1 Low |
|  | TDD | maxNumberMIMO-LayersPDSCH=2 | 20/30 | 48 | 2 | 4 | Interleaved | 3 | 6 | TDLC300-100 | 2x2 Low |
|  |  | maxNumberMIMO-LayersPDSCH=1 | 20/30 | 48 | 2 | 4 | Interleaved | 3 | 6 | TDLC300-100 | 2x1 Low |
| FR2 | TDD | maxNumberMIMO-LayersPDSCH=2(Note 2) | 100/120 | 60 | 2 | 16 | Interleaved | 3 | 2 | TDLA30-75 | 2x2 Low |
|  |  | maxNumberMIMO-LayersPDSCH=1 | 100/120 | 60 | 2 | 16 | Interleaved | 3 | 2 | TDLA30-75 | 2x1 Low |
| Note 1: the reused test cases |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3:  Define the case with SS/PBCH block index is known for 1Rx UE

Reference

[1]   R4-2203032 WF on Redcap demodulation and CQI reporting requirements