3GPP TSG-RAN WG4 Meeting # 102 -e	R4-22xxxxx

# Electronic Meeting, Feb.21st-Mar.3rd,2022

Title: 	Discussion on CSI requirements for RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	10.20.4.3

Document for:	Discussion

Background

As per [1], RAN 4 has agreed to introduce the PDCCH and PBCH requirements for RedCap UE. In this paper, we provide our views on remain open issues.

CQI requirements

In last meeting, RAN 4 has agreed to introduce wideband CQI reporting requirements for RedCap UE. But it is FFS to discuss whether to define case with CQI table 2 for UE supporting 256QAM:

- Define CQI reporting tests for both static condition and fading condition for RedCap 1Rx/2Rx UE. - For the fading condition define wideband CQI reporting test only. - Use CQI table 1 for RedCap UE CQI reporting requirements. - For 2Rx tests for FR1 FDD, - Define new 2Rx tests for FR1 FDD with CQI table 1 - FFS whether to define test with CQI table 2 for Redcap UE supporting 256 QAM. If agreed, existing test requirements can be reused

We don’t see the need to define the CQI requirements with CQI table 2. Based on our understanding, in Rel-15 CQI reporting test, SNR is selected based on the principle that median CQI is reported up to 64QAM even CQI table 2 is used. RedCap should also follow this principle and there is no difference for CQI table 1 and CQI table 2 if CQI reporting is up to 64QAM.

Proposal 1: Not define CQI requirements for RedCap with CQI table 2.

PMI requirements

The options for PMI requirements are listed as follows:

- RAN4 discuss further whether to define PMI reporting tests for RedCap UE. - Option 1: Define for both 1Rx UE and 2Rx UE - Option 2: Not define for both 1Rx UE and 2Rx UE - Option 3 Define only for 2Rx UE - If PMI reporting test is defined, discuss further applied test cases

Based on our understanding, it is enough to define the requirements with CQI reporting for RedCap UE since BS not always rely on UE’s reporting to determine the precoding matrix. Considering RedCap UE is low cost UE, we should reduce the test cost as much as possible.

Proposal 2: Not define PMI requirements for RedCap UE

RI requirements

- RAN4 discuss further whether to define RI reporting tests for RedCap 2Rx UE. - Option 1: Define RI tests. It requires to rerun the simulation with CQI table 1 for FR1 FDD and TDD. - Option 2: Not define RI tests.

We don’t think it is necessary to define the RI test since RI requirements are only applicable for 2RX UE and we should reduce the test cost as much as possible.

Proposal 3: Not define RI test for RedCap UE

Conclusion

Proposal 1: Not define CQI requirements for RedCap with CQI table 2.

Proposal 2: Not define PMI requirements for RedCap UE

Proposal 3: Not define RI test for RedCap UE

Reference

[1]   R4-2203032 WF on Redcap demodulation and CQI reporting requirements