3GPP TSG-RAN WG4 Meeting # 102-e	R4-22xxxxx

# Electronic Meeting, Feb.21st -Mar.3rd ,2022

Title: 	Discussion on BS requirements for Rel-17 NB-IOT

Source: 	Huawei, HiSilicon

Agenda item:       12.9.6.2.2

Document for:	Discussion

Background

As per [1], RAN 4 has agreed to define the performance requirements for NPUSCH with 16QAM reception, but there are still some issues left. In this paper, we will provide our initial simulation results and our views on these open issues.

Discussions

The simulation assumptions and open issues are captured as follows:

Whether to define demodulation requirements for NPUSCH format 1 with 16QAM - Define NPUSCH format 1 requirements with 16QAM and single-TB scheduling. Number of antennas: 1T2R Frequency offset and time offset: No frequency offset and timing offset considered Propagation conditions: ETU 1Hz Low Repetition number: No repetition Number of allocated tones - Option 1: 3,6,12 tones - Option 2: 12 tones Number of scheduled RUs and TBS Interested companies are welcome to bring simulation results for next meeting and select one based on the evaluation results in the next meeting - Option 1: (IRU, ITBS)=(0,15), TBS=280 bits - Option 2: (IRU, ITBS)=(19,3), TBS=1736 bits - Option 3: (IRU, ITBS)=(0,14), TBS=256 bits - Other options are not precluded

The open issues left are number of allocated tones and number of scheduled RUs and TBS. Other parameters missing are RV and max HARQ transmission number. We propose to reuse the existing parameters in TS 36.104. I.e. RV is {0, 2, 0, 2} and max HARQ transmission number is 4.

Proposal 1: Use max HARQ transmission number equal to 4 and RV = {0, 2, 0, 2}

Based on the proposal 1, we provide simulation results with different number of allocated tones and different combinations of IRU and ITBS as in Figure 1

| Figure 1a: (IRU, ITBS)=(0,15), TBS=280 bits | Figure 1b: (IRU, ITBS)=(19,3), TBS=1736 bits |
| --- | --- |
| Figure 1c: Option 3: (IRU, ITBS)=(0,14), TBS=256 bits |  |

Figure 1: Simulation results for NPUSCH

We can observe that different number of allocated tones have same performance. To reduce test number, we propose to only consider 12 tones.

Observation 1: Different number of allocated tones have same performance

Proposal 2: Only consider 12 tones

As for IRU and ITBS, we can observe that option 2 ((IRU, ITBS)=(19,3), TBS=1736 bits) has worse performance compared to option 1 and option 2 but target SNRs of all options are at reasonable ranges. We prefer option 1.

Observation 2: Option 2 ((IRU, ITBS)=(19, 3), TBS=1736 bits) has worse performance compared to option 1 and option 2 but target SNRs of all options are at reasonable ranges.

Proposal 3: Consider option 1 .I.e. (IRU, ITBS) = (0, 15), TBS=280 bits

Conclusion

In this paper, we provide our initial simulation results and our views on remain open issues of NPUSCH requirements. The observations and proposals are:

Observation 1: Different number of allocated tones have same performance

Proposal 2: Only consider 12 tones

Observation 2: Option 2 ((IRU, ITBS)=(19, 3), TBS=1736 bits) has worse performance compared to option 1 and option 2 but target SNRs of all options are at reasonable ranges.

Proposal 3: Consider option 1 .I.e. (IRU, ITBS) = (0, 15), TBS=280 bits

Reference

[1]   R4-2203041 Way forward for performance requirements of Rel-17 NB-IOT and eMTC