3GPP TSG RAN WG4 Meeting #102-e	R4-2205835

# E-meeting, 21 Feb. 2022 – 03 Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UL gaps for TX power management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UL gaps are introduced in Rel-17 for UE Tx power management through P-MPR in Rel-17. UL gaps and their prioritisation rules needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UL gaps and their prioritisation to the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL gaps will be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.12 UL gaps for UE self-calibration and monitoring

The UL gap patterns listed in table 9.12.1 are introduced for UE transmit power management through P-MPR.

Table 9.12.1: UL gap patterns for UE TX power management

|  | UGL [ms] | UGRP [ms] |
| --- | --- | --- |
| UL MGP #0 | 1.0 | 20 |
| UL MGP #1 | 1.0 | 40 |
| UL MGP #2 | 0.5 | 160 |
| UL MGP #3 | 0.125 when SCS of active UL BWP =120kHz 0.25 when SCS of active UL BWP =60kHz | 5 |

#### 9.12.1 UL gaps applicability rules

UE may use a UL gap instance for TX power management if the UL gap instance is not fully or partially overlapping with any of the following signals.

- RACH occasion

- FFS: CSI-Report during SCell activation or SCell dormancy activation or SCell dormancy deactivation

- Ocassion of positing measurement report transmission

#### 9.12.2 UL gaps optionality

FFS

<End of Change 1>