3GPP TSG-RAN WG4 Meeting #102-e    				     R4-2205866

Electronic Meeting, 21st of February - 3rd of March, 2022

Agenda item:	     10.13.3.4

Source:	THALES

Title:	Draft text proposal for Clause 7.5 Out-of-band blocking - TS 38.108

Document for:	Approval

# Introduction

Draft TS skeleton for Non-terrestrial networks (NTN) related to NR; Satellite Access Node radio transmission and reception (TS 38.108) has been approved in R4-2203087 [1] and the work split has been summarized in R4-2203080 Way Forward on NTN_solutions_Part1 [2] approved in RAN4 101-bis-e meeting. For further information, please also follow the discussions in R4-2203111 [3].

In this document, following changes have been proposed as a TP to update TS 38.108:

- Section 7.5: Out-of-band blocking

- Section 7.5.1: General

- Section 7.5.2: Minimum requirements for Satellite Access Node

# Reference

[1] R4-2203087 Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, THALES.

[2] R4-2203080 Way Forward on NTN_solutions_Part1, THALES.

[3] R4-2203111 Email discussion summary for [101-bis-e][306] NTN_Solutions_Part1, THALES.

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>----------------------------------------------

## 7.5	Out-of-band blocking

### 7.5.1	General

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the TAB connector for SAN type 1-H in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

### 7.5.2	Minimum requirements for Satellite Access Node

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to SAN type 1-H TAB connector using the parameters in table 7.5.2-1.

The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each SAN channel bandwidth and further specified in annex A.1.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for SAN supporting FDD.

Minimum conducted requirement is defined at the antenna connector at the TAB connector for SAN type 1-H.

Table 7.5.2-1: Out-of-band blocking performance requirement for NR

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (Note) | -44 | CW carrier |
| NOTE 1: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the SAN channel bandwidth. |  |  |

------------------------------------------------<End of TP>-----------------------------------------------