3GPP TSG-RAN WG4 Meeting #102-e    				     R4-2205886

Electronic Meeting, 21st of February - 3rd of March, 2022

Agenda item:	     10.13.3.1

Source:	THALES

Title:	Draft text proposal for Clause 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) - TS 38.108

Document for:	Approval

# Introduction

Draft TS skeleton for Non-terrestrial networks (NTN) related to NR; Satellite Access Node radio transmission and reception (TS 38.108) has been approved in R4-2203087 [1] and the work split has been summarized in R4-2203080 Way Forward on NTN_solutions_Part1 [2] approved in RAN4 101-bis-e meeting. For further information, please also follow the discussions in R4-2203111 [3].

In this document, following changes have been proposed as a TP to update TS 38.108:

- Section 9.7.3: OTA Adjacent Channel Leakage Power Ratio (ACLR)

# Reference

[1] R4-2203087 Skeleton for TS 38.108 NR Satellite Access Node radio transmission and reception v0.0.1, THALES.

[2] R4-2203080 Way Forward on NTN_solutions_Part1, THALES.

[3] R4-2203111 Email discussion summary for [101-bis-e][306] NTN_Solutions_Part1, THALES.

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>----------------------------------------------

### 9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 9.7.3.1	General

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

#### 9.7.3.2	Minimum requirement for BS type 1-O

The OTA ACLR limit requirement for SAN type 1-O shall apply the ACLR in Table 6.6.3.2-1: Satellite Access Node ACLR limit.

The OTA SAN ACLR absolute basic limit does not apply to SAN type 1-O.

#### 9.7.3.3	Minimum requirement for BS type 2-O

The requirement is not applicable in Release-17.

------------------------------------------------<End of TP>-----------------------------------------------