3GPP TSG-RAN WG4 # 102-e	R4-2205906

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.6.5.4

Source:	MediaTek Inc.

Title:	Discussion on the CQI requirements for 1024QAM

Document for:	Discussion

- Introduction

In RAN4#101-bis-e, a way forward on DL1024QAM was agreed [1]. It was agreed to use existing CQI definition for 1024QAM. In this contribution, we provide our view for the remaining issues and present simulation results.

- Define new CQI definition requirements for CQI table with 1024QAM entries for 2Rx/4Rx UEs. - Interested companies are encouraged to evaluate the following test setup. - Test parameters: Reuse the existing CQI definition tests - Rank 1 - Tx EVM assumed for simulation: Aligned with PDSCH demodulation requirements. - Test metric: Reuse the existing CQI definition tests - Test points: One SNR test point corresponding to CQI index 14 - 2Rx UE: - Option 1: SNR = 28/29dB - Option 2: SNR = 29/30dB - Other options not precluded - 4Rx UE: - Option 1: SNR = 25/26dB - Option 2: SNR = 26/27dB - Other options not precluded - RAN4 will discuss whether to add extra margin on top of the SNR test point, after RAN4 derives the SNR test points for CQI tests.

- Discussion

We provide our simulation results as below.

Table 1.	Summary for simulation results

|  | 2Rx |  |  |  |  | 4Rx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR [dB] | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 | Median CQI | Percentage of {median CQI-1, median CQI, median CQI+1} | BLER at Median CQI -1 for | BLER at median CQI | BLER at Median CQI +1 |
| 25 | 13 | 100% | 0% | 0% | 100% | 14 | 100% | 0% | 0% | 100% |
| 26 | 13 | 100% | 0% | 0% | 66.27% | 14 | 100% | 0% | 0% | 100% |
| 27 | 14 | 100% | 0% | 0% | 100% | 14 | 100% | 0% | 0% | 36.56% |
| 28 | 14 | 100% | 0% | 0% | 100% | 15 | 100% | 0% | 0% | - |
| 29 | 14 | 100% | 0% | 0% | 100% | 15 | 100% | 0% | 0% | - |
| 30 | 14 | 100% | 0% | 0% | 17.41% | 15 | 100% | 0% | 0% | - |

According to the simulation results, we propose to select 28/29dB for 2Rx and 25/26dB for 4Rx for 1024 QAM CQI reporting test case.

Proposal 1: Propose to select 28/29dB for 2Rx and 25/26dB for 4Rx for 1024 QAM CQI reporting test case.

- Conclusion

In this contribution, we provide simulation results for 1024QAM with the agreed simulation assumption in [1]. The proposals are summarized as below:

Proposal 1: Propose to select 28/29dB for 2Rx and 25/26dB for 4Rx for 1024 QAM CQI reporting test case.

- Reference

- R4-2203044, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson