3GPP TSG-RAN WG4 Meeting #102-e		R4- 2206002

E-meeting, February 21 – March 03, 2022

Agenda item:	10.6.5.3

Source:	Intel Corporation

Title:	Discussion on SDR requirements for DL 1024QAM in FR1

Document for:	Discussion

# Introduction

During the RAN4#101-bis-e meeting the following was agreed for SDR requirements [1]:

- Add the MCS indexes for SDR test with 1024QAM based on Table 2 for 2Rx. - Rank 1 - Option 1: MCS25 - Option 2: MCS23 - Rank 2 - Option 1: MCS23 - Option 2: Not define SDR tests - Add the MCS indexes for SDR test with 1024QAM based on Table 3 for 4Rx. - Rank 1 - Option 1: MCS25 - Option 2: MCS23 - Rank 2 - Option 1: MCS23 - Option 2: FFS - Interested companies are encouraged to propose MCSpracticalj to decide MCSreqj, assuming Tx EVM 2.5% where - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj) - Simulation parameters: TS38.101-4 Table 5.5A-1 Table 2 MCS indexes for SDM test with 1024QAM for 2Rx vLayersj; Qmj; fj; MCSupperboundj; MCSpracticalj; MCSreqj. 1; 10; 1; 25; TBD; TBD. 1; 10; 0.8; 21; TBD. 1; 10; 0.75; 19; TBD. 1; 10; 0.4; 9; TBD. 2; 10; 1; 25; TBD; TBD. 2; 10; 0.8; 21; TBD. 2; 10; 0.75; 19; TBD. 2; 10; 0.4; 9; TBD. Table 3 MCS indexes for SDM test with 1024QAM for 4Rx vLayersj; Qmj; fj; MCSupperboundj; MCSpracticalj; MCSreqj. 1; 10; 1; 25; TBD; TBD. 1; 10; 0.8; 21; TBD. 1; 10; 0.75; 19; TBD. 1; 10; 0.4; 9; TBD. 2; 10; 1; 25; TBD; TBD. 2; 10; 0.8; 21; TBD. 2; 10; 0.75; 19; TBD. 2; 10; 0.4; 9; TBD.

In this paper we provide new simulation results for SDR requirements definition and discuss the values for MCSpracticalj and corresponding MCSreqj.

# Discussion

The MCS indexes for 1024QAM SDR tests can be defined as the minimum between MCSupperboundj and MCSpracticalj. MCSupperboundj is the higher MCS according to UE capability calculated by Sec. 2.1 of R4-1812165. Its values for 1024QAM were agreed during the RAN4#101-e meeting. MCSpracticalj is the highest practical MCS considering feasible SNR levels. To define its values the link level simulations need to be conducted.

In Figure 1 and Figure 2we illustrate link level results for scenarios with 1024QAM for nRx = 2 and nRx = 4, respectively. The corresponding simulation assumptions are shown in Table 1. In Table 2 we provide summary of simulation results at the 85% throughput point. For analysis we pick MCS equal to MCSupperboundj for scaling factor 1, which is MCS25 for both Rank 1 and Rank 2, and compare the results with MCS24 and MCS23 to check which MCS corresponds to more feasible SNR level.

Table 1. Test parameters for SDR test

| Parameters | Values |
| --- | --- |
| SCS/CBW | 15kHz and 10MHz |
| MCS | MCS23 MCS24 MCS25 |
| Rank | 1, 2 |
| Antenna configuration | 1x2, 2x2 with ULA Low |
| Propagation channel | AWGN |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |
| Tx EVM assumed for simulation | 2.5% |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 0 ms |
| Number of HARQ processes | 4 |
| Transform precoding | CP-OFDM |
| Allocated RBs | Full BWP |
| PRB bundling | 2 |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |
| Receiver type | MMSE-IRC |
| Test metric | 85% of maximum throughput |

| Figure 1. 1024QAM simulation results for nRx = 2 |  |
| --- | --- |

| Figure 1. 1024QAM simulation results for nRx = 4 |  |
| --- | --- |

Table 2. Summary of SDR simulation results (SNR @ 85% of Max T-put)

|  | nRx = 2 |  | nRx = 4 |  |
| --- | --- | --- | --- | --- |
|  | Rank 1 | Rank 2 | Rank 1 | Rank 2 |
| MCS23 | 25.5 | 29.4 | 23.4 | 27.4 |
| MCS24 | 27.5 | 31.4 | 25.4 | 29.2 |
| MCS25 | 29.6 | 33.5 | 28.3 | 31.5 |

Based on the results from Table 2 the following observation can be made:

Observation 1. For the scenario with Rank 1 MCS25 has feasible SNR operating points both for 2 Rx and 4 Rx antennas.

Observation 2: For the scenario with Rank 2 after considering impairments margin the SNR operating points for MCS25 and MCS24 become very high, while MCS23 still has feasible SNR operating points both for 2 Rx and 4 Rx antennas.

Based on Observations 1 and 2 we can define MCSpracticalj as MCS25 for Rank 1 and MCS23 for Rank 2

Proposal 1: MCS index for MCSpracticalj is 25 for Rank 1 and 23 for Rank 2. Therefore, the MCS indexes for 1024QAM SDR tests can be defined as it is shown in Table 3.

Table 3. MCS indexes for SDR test with 1024QAM

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 25 | 25 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 23 | 23 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

# Conclusion

Observation 1. For the scenario with Rank 1 MCS25 has feasible SNR operating points both for 2 Rx and 4 Rx antennas.

Observation 2: For the scenario with Rank 2 after considering impairments margin the SNR operating points for MCS25 and MCS24 become very high, while MCS23 still has feasible SNR operating points both for 2 Rx and 4 Rx antennas.

Proposal 1: MCS index for MCSpracticalj is 25 for Rank 1 and 23 for Rank 2. Therefore, the MCS indexes for 1024QAM SDR tests can be defined as it is shown in Table 3.

Table 3. MCS indexes for SDR test with 1024QAM

| vLayersj | Qmj | fj | MCSupperboundj | MCSpracticalj | MCSreqj |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 25 | 25 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 23 | 23 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

# References

- R4-2203044, Way forward for NR DL1024QAM demodulation and CQI reporting requirements, Ericsson, RAN4 #101-bis-e, Jan 17-25, 2022