3GPP TSG RAN WG4 Meeting #102-e	R4-2206067

E-meeting, 21 Feb. 2022 – 03 Mar. 2022

Agenda Item:	6.4.6.1.2

Source: 	Ericsson

Title:	Discussion on number of serving carriers to be supported for FR2

Document for:	Discussion

# Introduction

In this contribution, we discuss number of carriers supported by a UE in FR2.

# Discussion

## Number of serving carriers supported

As per existing specification (section 3.6.2.1 of TS 38.133), number of serving carriers supported for a UE in DL is 8 and number of UL carriers supported is 2.

3.6.2.1 Number of serving carriers for SA Requirements for standalone NR with NR PCell are applicable for the UE configured with the following number of serving NR CCs: - up to 8 NR DL CCs in total, with 1 UL (or 2 UL if SUL is configured) in PCell and up to 1 UL (or 2 UL if SUL is configured) in SCell. - SUL may be configured together with one of the UL

However, from the CA configuration defined in TS 38.101-3 (Table 5.5A.1-1), number DL carrier supported is 8 and number of UL carriers supported is 8 (CA_n257M for both DL and UL). Since band combinations are defined in RF for these CA configurations, our understanding is UE shall support at least these configurations.

Hence, applicability of number of carriers supported shall be updated to 8 DL CC and 8 UL CC.

Proposal 1: RAN4 to agree that, for FR2, number of carriers supported by UE shall be up to 8 NR DL CC and 8 NR UL CCs in total.

# Summary and Conclusion

In this contribution we have analysed the need for revision of number of carriers to be supported for FR2 CA and made the following proposals.

Proposal 1: RAN4 to agree that, for FR2, number of carriers supported by UE shall be up to 8 NR DL CC and 8 NR UL CCs in total.

# References

- TS 38.133-h40