3GPP TSG-RAN WG4 Meeting#102 -e	R4-2206090

Online, 21st Feb – 3rd Mar, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on FR1 HST PDSCH CA Tests

Agenda item:	10.8.3.2

Document for:	Discussion

- Introduction

In RAN4#99-e, simulation assumptions were agreed in [1] for defining HST requirements for CA scenarios. There were also some open issues mentioned in [2]. In this paper, we provide our views on FR1 HST PDSCH CA test cases.

- Requirements for FDD 35/45MHz BW

As these BWs are already introduced in Rel-17, we are ok to consider these BWs for HST CA requirements as well. At the same time, some companies may have concern on whether we have enough time to align the results or whether we should work on the same objective in two parallel WIs. Therefore, we have no strong preference on whether to introduce these requirements.

- HST-SFN Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below for FDD 35/45MHz BWs.

Table 1: HST-SFN Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 35 | 10.69 | 13.19 | 8.8 | 11.3 |
| 45 | 10.56 | 13.06 | 8.72 | 11.22 |

- HST-DPS Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below for FDD 35/45MHz BWs.

Table 2: HST-DPS Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 35 | 10.26 | 12.26 | 7.45 | 9.45 |
| 45 | 10.26 | 12.26 | 7.47 | 9.47 |

- Conclusions

This paper provides our views on FDD 35/45MHz BWs and simulation results for FR1 HST PDSCH CA requirements for those BWs.

# References

[1] R4-2108635, “WF on UE demodulation for FR1 HST”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19 – May 27, 2021.

[2] R4-2202985, “WF on UE demodulation for FR1 HST”, 3GPP TSG RAN WG4 Meeting #101-bis-e, Online, Jan 17 - 25, 2022.