3GPP TGS-RAN WG4 Meeting #102-e	R4-2206091
Electronic Meeting, February 21 – March 3, 2022

Agenda item:	11.1.2.1.4

Source:	Intel Corporation

Title:	On general aspects and UE testing methodology for FR2-2

Document for:	Discussion

# 1	Introduction

Over the last few meetings, RAN4 has been discussing how to address the test methodology of FR2-2 [1,2]. During RAN4 #101Bis-e, the permitted test methods captured in TR 38.810 were agreed to be extended to FR2-2 [3]. Additionally, the applicability of enhanced methods for polarization mismatch (Objective 2) and test time reduction (Objective 5) to FR2-2 was also approved [3].

Enhanced test methodology

Agreement: Approve the proposal and extend applicability of Objective 2 and Objective 5 solutions to FR2-2

Permitted test methods

Agreement: Unless otherwise stated, FR2-2 will follow the baseline UE RF methodology detailed in TR 38.810. MU assessment will be revised to reflect proper frequency-dependent parameters and worst-case array size.

Given these extensions, in this contribution we review the UE permitted test methods content that requires further discussion for FR2-2.

# 2	Discussion

Testing and calibration aspects

During the last RAN4 meeting, it was agreed that the current test methods captured in TR 38.810 will extend to FR2-2 [3]. Though somewhat implied, whether the calibration aspects detailed in Clause 5.2.1.3 can also be extended to FR2-2 has not been agreed. Thus, we should confirm if this is the case and capture it explicitly in TR 38.884.

Observation 1: Current test methods in TR 38.810 have been extended to FR2-2, but the general testing and calibration aspects have not.

Proposal 1: RAN4 should confirm if the testing and calibration aspects detailed in Clause 5.2.1.3 of TR 38.810 can be extended to FR2-2.

Permitted UE RF test methodology

Handheld UE – worst-case antenna array size

Agreement: The worst-case antenna assumption for testability and MU assessment of handheld UEs in FR2-2 is [8 x2]. Single UE antenna element pattern parameters, similar to Table 5.2.3.3-1, need to be finalized in RAN4#102-e.

For UE RF methods (Clause 5.2) and measurement uncertainty assessment, we need to determine what radiating antenna aperture (D) to use for this frequency range and the specific worst-case array size of the UE type (i.e., handheld UE). In RAN4 #101Bis-e, an 8-element array was agreed to be used as the assumption to derive the core requirements of PC3 [4].  Additionally, the worst-case array size to determine MU was approved to be 8x2 [3]. This means we can likely reuse 5cm for D in FR2-2 PC3 derivations (since a 16-element array in FR2-2 will not exceed a D of 5cm). In fact, considering the increase in path loss of FR2-2, we may be able to lower the value for D. If we reuse 5cm for D, Table 5.2.1.2-2 in TR 38.810 should be updated to include a column for 71 GHz.

Observation 2: RAN4 should discuss if a radiating aperture of 5cm can be reused for FR2-2 PC3 derivations. Given the increase in path loss, we may also consider lowering the value of D.

Proposal 2: If D = 5cm is reused for FR2-2, a column for 71 GHz will be added to the minimum range length of DFF table in TR 38.810 (Table 5.2.1.2-1).

Table 5.2.1.2-1: Minimum Range Length of DFF System for D = 5cm [5]

| f [GHz] QZ [cm] | 24.25 | 30 | 40 | 50 | 52.6 | 71 |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.45 | 0.55 | 0.72 | 0.88 | 0.93 | 1.23 |
| 30 | 0.53 | 0.63 | 0.79 | 0.96 | 1.00 | 1.31 |

# 3	Conclusions

In this contribution we discussed potential updates for the extension of permitted UE RF test methods to FR2-2. The following proposals and observations were made:

Observation 1: Current test methods in TR 38.810 have been extended to FR2-2, but the general testing and calibration aspects have not.

Proposal 1: RAN4 should confirm if the testing and calibration aspects detailed in Clause 5.2.1.3 of TR 38.810 can be extended to FR2-2.

Observation 2: RAN4 should discuss if a radiating aperture of 5cm can be reused for FR2-2 PC3 derivations. Given the increase in path loss, we may also consider lowering the value of D.

Proposal 2: If D = 5cm is reused for FR2-2, a column for 71 GHz will be added to the minimum range length of DFF table in TR 38.810 (Table 5.2.1.2-1).

Table 5.2.1.2-1: Minimum Range Length of DFF System for D = 5cm [5]

| f [GHz] QZ [cm] | 24.25 | 30 | 40 | 50 | 52.6 | 71 |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.45 | 0.55 | 0.72 | 0.88 | 0.93 | 1.23 |
| 30 | 0.53 | 0.63 | 0.79 | 0.96 | 1.00 | 1.31 |

# 4	References

- R4-2120746,	“Email discussion summary for [101-e][329] FR2_enhTestMethods_Part2_NWM,” Moderator (Intel), RAN4 #101e, November 2021

- R4-2203120, “Email discussion summary for [101-bis-e][327] FR2_enhTestMethods,” Moderator (Intel), RAN4 #101Bis-e, January 2022

- R4-2203079, “WF on OTA test methods for FR2-2,” Intel, RAN4 #101Bis-e, January 2022

- R4-2202366, “WF on 60 GHz UE RF,” Qualcomm, RAN4 #101Bis-e, January 2022

- 3GPP TR38.810, “Study on test methods,” version 16.6.1, November 2020